3GPP TSG-WG SA2 Meeting #156-e 	S2-2304602

# 17 - 21 April, 2023, Electronic Meeting	(revision of S2-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4357 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the Group-MBR parameter for a group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The realization and enforcement of Group-MBR for a group has been introduced to utilize the Slice-MBR mechinasim with some mapping. For 5G VN group, it’s workable as 5G VN group is identified by DNN and S-NSSAI. But for other group, e.g. created by O&M, if it’s not identified by DNN and S-NSSAI, it’s not possible to realize the control of Group-MBR reusing the Slice-MBR mechinasim. So it’s proposed the Group-MBR is only applicable for 5G VN group, for other groups created e.g. using O&M, if they are identified by DNN and S-NSSAI, the Group-MBR also can be applied. It's propsed to remove the following Editor’s note and add the following NOTE: Editor’s note: If Group-MBR should apply only to 5GVN groups or also to groups created e.g. using O&M is FFS. NOTE: The Group-MBR is applicable for 5G VN groups as well as other groups which are identified or linked to a DNN and S-NSSAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the Editor’s note for Group-MBR parameter and add another NOTE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support for group attribute management enhancements is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.20b.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.20b.4	Support Group-MBR for a group

The procedure as defined in clause 4.15.6.2 of TS 23.502 [3] are applicable for provisioning of Group-MBR for a group with the following clarifications and enhancements:

-	The AF request additionally contains the Group-MBR, and the Group-MBR is stored in UDR.

The realization of the Group-MBR for a group is specified in clause 6.1.5 and 6.2.1.11 of TS 23.503 [45].

NOTE: The Group-MBR is applicable for 5G VN groups as well as other groups which are identified or linked to a DNN and S-NSSAI.

# * * * * End of changes * * * *