3GPP TSG-SA WG2 Meeting #156-e	S2-2304603

# 17– 21, April, 2023, Electronic Meeting	(revision of S2-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0983 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the Group-MBR parameter for a group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The realization and enforcement of Group-MBR for a group has been introduced to utilize the Slice-MBR mechinasim with some mapping. For 5G VN group, it’s workable as 5G VN group is identified by DNN and S-NSSAI. But for other group, e.g. created by O&M, if it’s not identified by DNN and S-NSSAI, it’s not possible to realize the control of Group-MBR reusing the Slice-MBR mechinasim. So it’s proposed the Group-MBR is only applicable for 5G VN groups, for other groups, if they are identified or linked to DNN and S-NSSAI, the Group-MBR also can be applied. It's propsed to remove the following Editor’s note and add the following NOTE: Editor’s note: If Group-MBR should apply only to 5GVN groups or also to groups created e.g. using O&M is FFS. NOTE: The Group-MBR is applicable for 5G VN groups as well as other groups which are identified or linked to a DNN and S-NSSAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the Editor’s note for Group-MBR parameter and add another NOTE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support for group attribute management enhancements is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.501 CR3931, TS23.502 CR3744 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start change ***

6.1.5	Group related policy control

Group related policy control supports limitation of the data rate per group.

The Group MBR defines the maximum allowed aggregate data rate across all GBR and Non-GBR QoS Flows within the group.

A Group MBR can be configured for a group by the operator. The Group MBR has an UL and a DL value.

The PCF monitors the data rate of the group and ensures that it does not exceed the Group MBR for that group. The method of PCF to realize the above functions is same as limitation of the data rate per network slice as defined in clause 6.1.4.1 and 6.1.4.3 with the following mapping:

-	Instead of handling Maximum Slice Data Rate per S-NSSAI, the UDR and PCF handles the Maximum Group MBR per group identified by DNN and S-NSSAI.

-	Instead of deducting the value of the authorized Session-AMBR and the MBR of every GBR SDF for every PDU Session of a slice, the PCR deducts such value for every PDU Session accessing to the group.

NOTE: The Group-MBR is applicable for 5G VN groups as well as other groups which are identified or linked to a DNN and S-NSSAI.

*** End of change ***