SA WG2 Meeting #156E (e-meeting)	S2-2304607

# April 17 – 21, 2023											                                                  (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4359 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve the EN on ECN marking for L4S during UE mobility. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2022-11-xx |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At the S2#155 meeting, there was an EN left to be addressed: Editor's note: During UE mobility, e.g. NG-RAN handover or local PSA UPF relocation, whether there are other impacts for ECN marking for L4S is FFS. In the case of ECN marking for L4S by NG-RAN, SMF may also provide QoS Flow level ECN marking for L4S indicator to the target NG-RAN during the NG-RAN handover. In such case, local PSA UPF relocation has no impacts on ECN marking for L4S. In the case of ECN marking for L4S by PSA UPF, SMF may indicate the target NG-RAN to report the congestion information if NG-RAN handover occurs, and a QoS Flow level ECN marking for L4S indicator may be sent by SMF to the target PSA UPF if PSA UPF relocation occurs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the relevant EN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ECN marking for L4S may not be supported during UE mobility. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 5.37.3	Support of ECN marking for L4S to expose the congestion information

#### 5.37.3.1	General

L4S (Low Latency, Low Loss and Scalable Throughput) is described in IETF RFC 9330 [159], IETF RFC 9331 [160] and IETF RFC 9332 [161]. It exposes congestion information by marking ECN bits in the IP header of the user IP packets between the UE and the application server to trigger application layer rate adaptation.

In 5G System, ECN marking for L4S may be supported. ECN marking for L4S is enabled on a per QoS Flow basis in the uplink and/or downlink direction and may be used for GBR and non-GBR QoS Flows. ECN marking for the L4S in the IP header is supported in either the NG-RAN (see clause 5.37.3.2 and TS 38.300 [27]), or in the PSA UPF (see clause 5.37.3.3).

NOTE 1:	Whether NG-RAN or PSA UPF based ECN marking for L4S is used is decided by SMF based on operator's network configuration and policies.

In the case of ECN marking for L4S by UPF, the NG-RAN is instructed to perform congestion information monitoring.

NOTE 2:	As for any QoS flow, QoS rules in the UE and PDRs in the PSA UPF control which packets are bound to the L4S enabled QoS flow. The Packet Filter Set in the QoS rule or PDR can use packet filter(s) in clause 5.7.6.2 (e.g. ECT(1) and/or IP 5 tuple) to steer traffic to an L4S enabled QoS Flow.

NOTE 3:	A QoS flow may be enabled with ECN marking for L4S requirement e.g. statically when a PDU session is established based on configuration in SMF or PCF, or dynamically based on detection of the L4S traffic e.g. via ECT(1) and/or IP 5 tuple in the IP header whereby SMF or PCF triggers a setup of a QoS Flow enabled for L4S, or by requests by an AF.

NOTE 4:	To support this functionality, the UE needs to support L4S feedback as described in IETF RFC 9330 [159], which is not in the scope of 3GPP.

During UE mobility, SMF may provide an indication for ECN marking for L4S to the target NG-RAN or the target PSA UPF for a corresponding QoS Flow(s). In the case of ECN marking for L4S by UPF, SMF also indicates the target NG-RAN to report the congestion information if NG-RAN handover occurs.

#### 5.37.3.2	Support of ECN marking for L4S in NG-RAN

ECN marking for L4S may be supported in NG-RAN as specified in TS 38.300 [27].

To enable support of ECN marking for L4S in NG-RAN, dedicated QoS Flow(s) are used for carrying L4S enabled IP traffic. The SMF may be configured to, based on PCC Rule provide an indication for ECN marking for L4S to NG-RAN for a corresponding QoS Flow(s), but in the absence of such configuration the use of L4S on a QoS flow is controlled by a coordinated configuration in NG-RAN and 5GC.

The criteria based on which NG-RAN decides to mark ECN bits for L4S is NG-RAN implementation specific.

#### 5.37.3.3	Support of ECN marking for L4S in PSA UPF

To enable ECN marking for L4S by a PSA UPF, a QoS Flow level ECN marking for L4S indicator may be sent by SMF to PSA UPF over N4. SMF also indicates to NG-RAN to report the congestion information (i.e. a percentage of packets that UPF uses for ECN marking for L4S) of the QoS Flow on UL and/or DL directions via GTP-U header extension to PSA UPF. If there is no UL packet when report for DL and/or UL needs to be provided, NG-RAN may generate an UL Dummy GTP-U Packet for such a reporting.

Upon successful activation of congestion information reporting for UL and/or DL, PSA UPF uses information sent by NG-RAN in GTP-U header extension (see TS 38.415 [116] and TS 38.300 [27]) to perform ECN bits marking for L4S for the corresponding direction.

* * * End of Changes * * *