3GPP TSG-SA WG2 Meeting #156E	S2-2304614

# Elbonia, April 17 – 21, 2023,

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4039 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIMLsys: KI#7 – Adding maximum number of UEs as a member selection filtering criterion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As the text agreed below in Clase 5.46.2 of TS 23.501, the number of UEs is included as one of the examples of filtering criteria. - Receiving a request from the AF that shall include a list of target member UE(s), optionally (a) time window(s), and one or more filtering criteria as specified in Table 4.15.13.2-1 of TS 23.502 [3] (e.g., number of UEs, UE location, trajectory and direction, QoS requirements, Area of Interest, DNN, S-NSSAI, expected application operation time duration, preferred access/RAT type, Desired UE transmission latency performance). In addition, step 1 in the clause 4.15.13.3.2 of TS 23.502 describes the use of the maximum number of UEs provided by the AF. When providing a target area for FL, the AF may provide sub-areas, and provide a maximum number of UEs that should take part in FL from each sub-area. This CR proposes to add the number of UEs, specifically the "maximum" number of UEs, as one of the filtering criteria. The AIML service for FL may require a maximum number of UEs for FL operation and/or the AF may not need all the UEs that satisfies the filtering criteria. Although the AF can filter by itself, if the AF provides a maximum number of UEs to perform the application operation, this can save the resources for the NEF and other NFs to collect UE information. The maximum number of UEs provided by the AF may be used as an optional input parameter (Maximum number of SUPIs (SUPImax) in Clause 6.1.3 of TS 23.288) when the NEF requests analytics from the NWDAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | AF may provide maximum number of UEs for application operation as a filtering criterion. If the AF provides maximum number of UEs, NEF can provide one or more lists of candidate UEs less than the maximum number of UEs, not all candidate UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to provide a 5GC mechanism to support the member selection functionality to assist the Application AI/ML operation with the number of UEs criteria as described in TS 23.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.15.13.2	UE Member Filtering Criteria for 5GS assistance to UE member selection

Table 4.15.13.2-1 provides a summary of the UE member filtering criteria that the AF may request.

Table 4.15.13.2-1: Description of UE Member filtering criteria

| UE Member filtering criteria | Description of filtering criterion | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| QoS | The Quality of Service of the selected UEs match or exceed the QoS of the filtering criteria |  | 4.15.13.3 |
| Access Type and/or RAT Type of the PDU Session | Indicate the Access Type and/or RAT Type of the selected UE for the PDU Session used by the application (e.g. 3GPP/NR, Non-3GPP/WLAN, additional Access Type and RAT Type for MA PDU session) | Service operation: Nsmf_EventExposure Filter: a list of GPSI(s) or SUPI(s), DNN/S-NSSAI Event ID: Change of Access Type and/or Change of RAT Type | This can be provided by existing service operations, events, and procedures. |
| End-to-end data volume transfer time | Indicate the end-to-end data volume transfer time that refers to a time for completing the transmission of a specific data volume between UE to AF | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Analytics ID = End-to-end data volume transfer time | Clause 6.18 of TS 23.288 [50]. |
| UE current location | Indicate the certain area that the selected UE currently located in. | Service operation: Namf_EventExposure Filter: a list of GPSI(s) or SUPI(s) | 4.15.13.3 |
| UE historical location | Indicate the certain area that the selected UE appeared in a historical period of time | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period | 4.15.13.3 |
| UE direction | Indicate the selected UEs should include different moving direction | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID= UE Mobility, Direction | 4.15.13.3 |
| Maximum number of UEs | Indicate the maximum number of candidate UEs that need to be fed back. Can be optionally used with other criteria | Number of UEs | N/A |

Editor's note:	The list of UE Member filtering criteria and their corresponding actions are FFS.

Editor's note:	Each Filter Criteria may add a new clause where the specific procedures for such filter criteria are described. For example:"4.15.13.3	Specific procedure for criterial foo".

* * * * End of Changes * * * *