3GPP TSG-SA WG2 Meeting #156E	S2-2304637

17 - 21 April 2023, E-meeting	(revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.032 | CR | 0022 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New GAD Shapes for Ranging and Sidelink Positioning Location Results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New GAD shapes are needed in TS 23.032 to define location results applicable to ranging and sidelink positioning of one or more target UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definitions are added for the following new types of location result: - Range and Direction - Relative Location - Relative Velocity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to request and obtain well defined location results for ranging and sidelink positioning of one or more target UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 5, 5.x1 (new), 5.x2 (new), 5.x3 (new), 8.4a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# 1	Scope

The present document defines an intermediate universal Geographical Area Description which subscriber applications, GSM, UMTS, EPS or 5GS services can use and the network can convert into an equivalent radio coverage map.

For GSM, UMTS, EPS or 5GS services which involve the use of an "area", it can be assumed that in the majority of cases the Service Requester will be forbidden access to data on the radio coverage map of a particular PLMN and that the Service Requester will not have direct access to network entities (e.g. BSC/BTS, RNC/Node B, eNB or gNB).

The interpretation by the PLMN operator of the geographical area in terms of cells actually used, cells that are partly within the given area and all other technical and quality of service aspects are out of the scope of the present document.

This specification also provides a description of velocity that may be associated with a universal Geographical Area Description when both are applied to a common entity at a common time.

The specification further provides a description of range and direction, relative location and relative velocity for a pair of devices such as 2 UEs.

**** Next Change ****

# 5	Shapes

The intention is to incorporate a number of different shapes, that can be chosen according to need.

-	Ellipsoid Point;

-	Ellipsoid point with uncertainty circle;

-	Ellipsoid point with uncertainty ellipse;

-	Polygon;

-	Ellipsoid point with altitude;

-	Ellipsoid point with altitude and uncertainty ellipsoid;

-	Ellipsoid Arc;

-	High Accuracy Ellipsoid point with uncertainty ellipse;

-	High Accuracy Ellipsoid point with scalable uncertainty ellipse;

-	High Accuracy Ellipsoid point with altitude and uncertainty ellipsoid;

-	High Accuracy Ellipsoid point with altitude and scalable uncertainty ellipsoid.

-Shapes relevant to Local Co-ordinates:

-	Local 2D point with uncertainty ellipse (only in 5GS);

-	Local 3D point with uncertainty ellipsoid (only in 5GS).

Shapes relevant to a pair of devices:

-	Range and Direction (only in 5GS);

-	Relative Location (only in 5GS).

Each shape is discussed individually.

**** Next Change ****

## 5.x1	Range and Direction

The "range and direction" from a point A to a point B is characterised by three components comprising a range from point A to point B, an azimuth direction from point A to point B and an elevation direction from point A to point B as shown in Figure 3d. The range provides a straightline distance from point A to point B. The azimuth provides a direction to point B from point A in a horizontal plane through point A and as measured clockwise from North. The elevation provides a direction to point B from point A in a vertical plane through the points A and B and as measured upwards or downwards from a horizontal plane through point A. The range, azimuth and elevation can be each independently included or excluded in a “range and direction” and each has an uncertainty and a confidence.

Figure 3d: Description of a Range and Direction

## 5.x2	Relative 2D Location with uncertainty ellipse

The "relative 2D location with uncertainty ellipse" is characterised by a point described in 2D local co-ordinates with origin corresponding to another known point, distances r1 and r2 and an angle of orientation A. It describes formally a set of points which fall within or on the boundary of an ellipse centered on a 2D point with semi-major axis of length r1 oriented at angle A (0 to 180o) measured clockwise from North and semi-minor axis of length r2. The confidence level with which the position of a target entity is included within this set of points is also included with this shape.

This shape is only applicable to 5GS.

Editor's note:	The structure of a relative 2D location with uncertainty ellipse will be defined by CT WGs.

## 5.x3	Relative 3D Location with uncertainty ellipsoid

The "relative 3D location with uncertainty ellipsoid" is characterised by a point described in 3D local co-ordinates with origin corresponding to another known point, distances r1 (the "semi-major uncertainty"), r2 (the "semi-minor uncertainty") and r3 (the "vertical uncertainty") and an angle of orientation A (the "angle of the major axis"). It describes formally the set of points which fall within or on the surface of a general (three dimensional) ellipsoid centred on a 3D point whose semi-major, semi-minor and semi-mean axis are, respectively,  r1, r2, r3 with r1  r2. The r3 axis is aligned vertically, while the r1 axis, which is the semi-major axis of the ellipse in a horizontal plane, is oriented at an angle A (0 to 180 degrees) measured clockwise from North. The confidence level with which the position of a target entity is included within the shape is also included.

This shape is only applicable to 5GS.

Editor's note:	The structure of a relative 3D location with uncertainty ellipsoid will be defined by CT WGs.

**** Next Change ****

## 8.4a	Relative Velocity with Uncertainty

The relative velocity with uncertainty of a device B relative to a device A is characterised by a radial velocity component and a perpendicular transverse velocity component. The radial velocity component is characterized by a rate of change of a range between the device A and device B.  The transverse velocity component is characterized by a rate of change of a direction to the device B from the device A, where the direction includes an angle of azimuth measured clockwise from North in a horizontal plane through the device A and an angle of elevation measured upwards or downwards in a vertical plane through the devices A and B from a horizontal plane through the device A. The rates of change of the range and the angles of azimuth and elevation can be each independently included or excluded in the relative velocity and each has an uncertainty and a confidence,

Figure 12a: Description of a Relative Velocity with Uncertainty

**** End Of Changes ****