3GPP TSG-SA WG2 Meeting #156E	S2-2304674

# Elbonia, April 17 – 21, 2023	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4374 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on “LADN per DNN and S-NSSAI” |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Supporting LADN per DNN and S-NSSAI has been specified in the current specification, but there are still some texts showing unclear AMF/SMF behaviour. Therefore, further clarifications are needed. In principle, this CR proposes that AMF and SMF follow the same way as described in clause 5.6.5, as many as possible. First, to resolve the following editor’s note, Editor's note: If there is no overlapping between the LADN service area configured in AMF and service area received from UDM, how to handle this case is FFS. The determination of LADN Service Area and sending it to UE are up to AMF decision based on operator policy. Second, to resolve the following editor’s note, Editor's note: The SMF does not need to know whether there is a LADN Service Area for a DNN/S-NSSAI, SMF does not need to know the content of the LADN Service Area. Whether this should be reflected in the data provided by UDM is FFS. The AMF can inform the SMF of the UE presence in the LADN service area per DNN and S-NSSAI, based the group information of the subscription information provided by UDM. It is in the same way as described in clause 5.6.5 with the difference that the LADN service area is defined per DNN and S-NSSAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarifying the optional AMF behaviour providing the LADN Service Area Information per LADN DNN and S-NSSAI to the UE, by adding “may”. - Clarifying the AMF behaviour determining the LADN Service Area based on the local configuration and/or the subscribed LADN service area received from UDM according to operator policy, with removing the corresponding editor’s note. - Just removing the corresponding editor’s note because the AMF behaviour on UE presence notification is in the same way as described in clause 5.6.5 with the difference that the LADN service area is defined per DNN and S-NSSAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.5a, 5.20b.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.6.5a	Supporting LADN per DNN and S-NSSAI

The 5GS may support LADN per DNN and S-NSSAI, and the functions specified in clause 5.6.5 are used (with the extension to apply per DNN and S-NSSAI whenever applicable) with the following enhancements:

-	The UE indicates the support of LADN per DNN and S-NSSAI in the UE MM Core Network Capability of the Registration Request message.

-	The LADN Service Area can be provisioned for a group (e.g. 5G VN group) or an individual subscriber using UDM/NEF parameter provisioning service triggered by AF request as described in clause 4.15.6 of TS 23.502 [3].

-	The LADN Service Area is determined by AMF, based on the local configuration and/or the subscribed LADN service area received from UDM.

-	If UE indicates the support of LADN per DNN and S-NSSAI, the AMF may provide the LADN Service Area Information per LADN DNN and S-NSSAI to the UE.

-	The AMF enforces the LADN Service Area per LADN DNN and S-NSSAI in the same way as described in clause 5.6.5 with the difference that the LADN service area is defined per DNN and S-NSSAI.

-	The UE enforces the LADN Service Area per LADN DNN and S-NSSAI, if received from the AMF, in the same way as described in clause 5.6.5 with the difference that the LADN service area is defined per DNN and S-NSSAI.

* * * * Start of 2nd Change * * * *

### 5.20b.2	Support service area for a group

The procedure as defined in clause 4.15.6.2 of TS 23.502 [3] is applicable for provisioning of service area for a group with the following clarifications and enhancements:

-	The AF request additionally contains the LADN service area as part of DNN and S-NSSAI specific Group Parameters, and the service area is stored in UDR as subscription data and delivered to AMF as well as SMF.

-	If the AF provides the LADN service area in the form of geographical information, the NEF maps the geographical information to a list of TAs before sending the service area to the UDM.

LADN per DNN and S-NSSAI as defined in clause 5.6.5a is applicable for enforcement of LADN service area for a group with the following clarifications and enhancements:

-	If the group is provisioned with the LADN service area, the AMF configures the DNN of the group as LADN DNN and determines the LADN service area based on local configured LADN service area corresponding to the DNN (if any) and/or the subscribed service area received from UDM according to operator policy.

-	If the group is provisioned with the LADN service area, the SMF configures the DNN of the group as LADN DNN.

* * * * End of Changes * * * *