SA WG2 Meeting #156E (e-meeting)	S2-2304680

April 17 – 21, 2023	(revision of S2-2300xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4375 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling UE policy association when UE registered over both 3GPP and Non-3GPP access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As indicated in the section 5.3.2.4 of TS 23.501, that the UE can be registered over both 3GPP and Non-3GPP access. When served by the same PLMN for 3GPP and non-3GPP accesses, an UE is served by the same AMF. And the AMF will select the one PCF to provide the UE policy to UE on both 3GPP and Non-3GPP access. And as indicated in the section 4.11.1.2.1 of TS 23.502, in step 18, during the handover from 5GS to EPS, only the deregistration of the old AMF for 3GPP access will happen, and any registration associated with the non-3GPP access in the old AMF is not removed. It means the 5GS can still provide UE policy by the UE policy association via non-3GPP access. So for this scenario, when UE is registered over both 3GPP and Non-3GPP access, and when UE does handover from 5GS to EPS and the UE is not deregistered the non-3GPP access, there is still one UE policy association in non-3GPP access in 5GS. The UE still can receive the UE policy via this association. In order to avoid to have two UE policy association from the same PCF for UE, it is not needed to estabulish the UE policy association for this UE in EPS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When UE is registered over both 3GPP and Non-3GPP access, and when UE does handover from 5GS to EPS and the UE is not deregistered the non-3GPP access, it is not needed to estabulish the UE policy association for this UE in EPS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be two UE policy association, one is on EPS and another one is on non-3GPP access in 5GS, and it is redundant and duplicated to have two UE policy association to provide UE policy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

5.17.8	URSP Provisioning in EPS

When the UE first registers in 5GS, the UE includes the Indication of URSP Provisioning Support in EPS in the UE Policy Container carried in Registration Request. Only when receiving this indication in the UE Policy Container, the PCF will provision the URSP to UE in EPS.

When the UE does Initial Attach to EPC, the UE includes the UE Policy Container ePCO in PDN Connectivity Request carrying default APN encapsulated in Attach Request; otherwise, the UE will include the UE Policy Container ePCO in the PDN Connectivity Request during the first request for PDN connectivity. The PDN Connection used by UE and SMF/PGW-C to convey UE Policy Container ePCO shall be kept when the UE is in the CONNECTED mode. When the UE is attached to EPS, the PCF for the PDU Session retrieves the PCRTs for UE Policy from PCF for the UE and subscribes to the applicable PCRTs to SMF+PGW-C. The PCF for the PDU Session sends the UE Policy Container in the same PDN connection/PDU session in which the UE Policy Container was received. Then the PCF generates the corresponding UE Policy Container in a similar way as it is done in 5GC and then includes it into a new IE for sending it to the SMF over N7, then further to UE as described in clause 4.11.0a.5 of TS 23.502 [3].

During EPS to 5GS mobility with N26, the UE Policy Association is terminated by PCF for PDU Session when it receives the indication of RAT type change from the SMF+PGW-C.

Editor's note:	It's FFS on how to discover the same PCF for UE serving the UE in both EPS and 5GS.

During 5GS to EPS mobility with N26, the AMF doesn't initiate the UE Policy Association termination procedure, instead the AMF will request the PCF for the UE to delay the termination of UE Policy Association for a configured period. The PCF for the UE initiates the UE Policy Association Termination procedure either after it receives the UE Policy Association Establishment request from PCF for the PDU Session or after the configured period. The PCF for the PDU Session determines whether the UE supports URSP delivery in EPS by checking UE context policy control subscription information in UDR. The PCF for the PDU Session discovers the address of PCF for the UE serving the UE by querying BSF. The PCF for the UE recovers the information about the PSI list in the UE and the subscribed PCRTs in 5GS from former UE Policy Association for the UE after receiving the UE Policy Association Establishment request including a UE Policy Container only including an indication about the trigger for the UE Policy Association Establishment ("5GS to EPS handover").

For the scenario of UE is served by the same PLMN for 3GPP and non-3GPP accesses, during 5GS to EPS mobility with N26, if any registration of UE associated with the non-3GPP access is not removed, it is not needed to PCF for the PDU Session to establish UE policy association in EPS to PCF for UE. The PCF for the PDU Session doesn’t need to discover the address of PCF for the UE serving the UE by querying BSF, and the PCF for the PDU Session doesn’t need to trigger to establish the UE policy association to PCF for the UE.

* * * End of Change * * *