SA WG2 Meeting #156E (e-meeting)	S2-2304685

April 17 – 21, 2023	(revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0117 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ECS Address Configuration Information retrieving from UDR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_ph2 |  |  |  |  |  | Date: |  |  | 2023-04-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is FFS whether a new API is used for V-AF to provide ECAI information for roamers or existing NEF services can be extended to do so. Reusing the Parameter Provision procedure is the potential way forward to not introduce the new API. The AF may provide ECAI that associated with PLMN ID during the Parameter Provision procedure, before the HR session created. And such information will be stored in the UDR. And one possbile way is, when the HR session created, the V-SMF may retrieve the ECAI from V-UDR via V-NEF to check whether there exists the ECAI that associated with this PLMN ID or DNN, S-NSSAI, and then sends the ECAI to HPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The AF in the VPLMN may provide the VPLMN EACI via External Parameter Provisioning procedure ahead of HR session establishedto the V-NEF that stores this information in V-UDR. And during HR session established procedure, the V-SMF retrieves the information from V-NEF, the retrieval is not related with an individual UE but with a DNN, S-NSSAI and PLMN ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The VPLMN can’t provide the ECAI to HPLMN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

6.5.2.6	ECS Address Provisioning in Roaming

For both LBO and HR case, the subscription data of the ECS Address Configuration Information in UDM or UDR is stored per PLMN ID.

For the LBO case, an AF in the visited PLMN may provide the EACI via External Parameter Provisioning procedure as described in clause 4.15.6.3d in TS 23.502 [3] to UDM via H-NEF. This ECS Address Configuration Information is further provided to SMF as part of subscription information.

For the HR case when access to EHE in VPLMN is allowed:

-	The HPLMN has the knowledge of EACI in the VPLMN: For this scenario, the AF is able to interact with NEF in HPLMN. The AF may provide the VPLMN EACI to H-NEF via External Parameter Provisioning procedure as described in clause 4.15.6.3d in TS 23.502 [3], and further to the UDM. During the HR PDU session establishment procedure, the H-SMF sends the VPLMN EACI to V-SMF and then to UE. The V-SMF does not modify, but just delivers the EACI provided by the H-SMF.

-	HPLMN does not have the knowledge of EACI in VPLMN: For this scenario, the AF can't interact with NEF in HPLMN. The AF in the VPLMN may provide the VPLMN EACI via External Parameter Provisioning procedure to the V-NEF that stores this information in V-UDR.  During the HR PDU Session establishment, the V-SMF retrieves the VPLMN EACI information from V-NEF, the V-SMF sends the VPLMN EACI obtained from V-NEF to the H-SMF, and H-SMF decides the VPLMN ECS Address Configuration Information sent to V-SMF and then to UE according to the PLMN ID additionally.

In this case the V-SMF retrieval of the VPLMN EACI information is not related with an individual UE but with a DNN, S-NSSAI.

NOTE 1:	It depends on the PLMN operation that the HPLMN can decide the ECAI if both VPLMN and UDM provides the ECAI.

NOTE 2: the AF in the VPLMN provides the VPLMN EACI ahead of HR PDU sessions

\

* * * End of Change * * *