3GPP TSG-WG SA2 Meeting #156E	S2-2304759

# Elbonia, Apr 17 – Apr 21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4065 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Multi-member AF session with required QoS for a list of UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | First, the following Editor’s Notes in clauses 4.15.6.13.2 and 4.15.6.13.3 need to be addressed, respectively. Editor’s note: Whether and how to support Consolidate_MBR monitoring concurrently when requesting the resource for the new UE is FFS. Editor's note: Proper reference to the UPF Event Exposure needs further clarification. Second, it is not clear why AF transaction ID is required in step 12 of clause 4.15.6.13.3 in that it is sufficient to only have Transaction Reference ID assigned by NEF without AF transaction ID asigned by AF. Third, it is not clear how to distinguish between each of three kinds of update operations (i.e. update the UE(s) in the list, update the resource allocation, and update the type of QoS monitoring) in steps 4-5 of clause 4.15.6.13.3 and clause 5.2.6.31.5. Last, clarification for procedures other editorial corrections are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Step 12 of Clause 4.15.6.13.2 is updated to address the following Editor’s Note. Editor's note: Proper reference to the UPF Event Exposure needs further clarification. Steps 5-6 of Clause 4.15.6.13.3 is updated to address the following Editor’s Note. Editor’s note: Whether and how to support Consolidate_MBR monitoring concurrently when requesting the resource for the new UE is FFS. Step 12 of Clause 4.15.6.13.3 is updated to remove AF Transaction (Reference) ID as an unnecessary parameter. Clause 5.2.6.31.5 is updated to add a Note (i.e. Note 5) for clarification of how to distinguish between each of three kinds of update operations. All the affected clauses are partly updated for editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification for the feature of Multi-member AF session with required QoS for a list of UEs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.13.1, 4.15.6.13.2, 4.15.6.13.3, 5.2.6.31.1, 5.2.6.31.2, 5.2.6.31.3, 5.2.6.31.4, 5.2.6.31.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

4.15.6.13	Multi-member AF sessions with required QoS for a list of UEs

4.15.6.13.1	General Descriptions

This clause describes the procedure to request QoS and to perform QoS monitoring for the communication between a list of UEs, identified by the UE IP address(es) and the AF.

Editor's note:	It is FFS whether to converge with GMEC to leverage common service interface to also support Multi-member AF sessions with required QoS for a list of UEs.

The NEF receives the Multi-member AF session request with required QoS for a list of UEs identified by their IP addresses from the AF. The NEF then converts the Multi-member AF session request to the per-UE individual AF session request with required QoS to each of the UE's serving PCF which then applies AF session with required QoS as described in clause 4.15.6.6 "Setting up an AF session with required QoS procedure" to each UE, except for the TSCTSF related steps.

The NEF receives the outcome (i.e. whether the requested QoS is allowed) of the individual AF session with required QoS corresponding to each UE's IP address, and consolidates the individual outcomes into single response before forwarding it to the AF.

When the AF subscribes to QoS Monitoring for the list of UEs identified by their respective IP addresses, the AF also provide a Consolidated MBR threshold that is stored in the NEF. The NEF subscribes to QoS Monitoring for each AF session with required QoS parameter(s) for each UE identified by its IP address. The list of the QoS Monitoring for Data Rate reports will be sent to the AF only if the aggregated bit rate exceeds the Consolidated-MBR threshold. Otherwise, the list of QoS Monitoring for Data Rate report will be sent to the AF according to the reporting frequency.

The following describes the mapping between the 4 types of NEF service requests of the multi-member AF session with required QoS and the N5 service requests.

Table 4.15.6.13.1-1:

| Nnef_MultiMemberAFSessionWithQoS Request | N5 request |
| --- | --- |
| Create | Requesting to allocate network resources to support a list of UEs' communication with the AF and the UEs are identified by the UE IP address(es). In addition, the AF can subscribe to be notified of QoS Monitoring reports. |
| Update | Updating (a) the list of UEs, (b) the requested QoS (c) the type of QoS monitoring for the list of QoS flows. More than one of the above operations can be requested at the same time. |
| Revoke | Deleting the establishment of the Multi-member AF sessions for a list of UEs. |
| Notify | Notifying AF with the individual or aggregated QoS Monitoring report(s) for a specific list of AF sessions |

The Consolidated Maximum Bit Rate (Consolidated-MBR) threshold is described as follows:

-	The Consolidated-MBR threshold provides the upper bound of the aggregate bit rate across all GBR QoS Flows corresponding to the list of PDU Sessions of the UEs who participate in concurrent transmissions (e.g. Federated Learning (FL) operation) with active user planes. The NEF aggregates bit rate across the set of selected QoS Flows and compares the aggregated bit rate against the Consolidated-MBR threshold.

In the case of the QoS monitoring for a list of QoS flows, Consolidated-MBR monitoring or concurrent reporting for the list of QoS flows' data rate or both could be supported at the same time by the NEF according to the AF request.

NOTE 1:	If Consolidated-MBR threshold is provided, the QoS parameter(s) to be measured indicates the bit rate for the requested AF sessions shall be provided.

NOTE 2:	When the event report is for Consolidated-MBR monitoring, the QoS Flow data rate reporting for the list of UEs is provided to the AF by the NEF only when the Consolidated-MBR threshold is exceeded.

NEXT CHANGE

4.15.6.13.2	Procedures for Creating Multi-member AF sessions with required QoS for a list of UEs

Figure 4.15.6.13.2-1: Procedures for creating Multi-member AF sessions with required QoS for a list of UEs

1.	The AF sends a request to reserve resources for AF sessions for a list of UEs using Nnef_MultiMemberAFsessionWithQoS_Create request message (a list of UE address(es), AF Identifier, Flow description information or External Application Identifier, QoS Reference or individual QoS parameters, Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20]), QoS parameter(s) to be measured, Reporting frequency, Target of reporting, an indication of local event notification, Consolidated-MBR threshold, DNN, S-NSSAI) to the NEF. The AF may, instead of a QoS Reference, provide one or more of the following individual QoS parameters: Requested 5GS Delay (optional), Requested Priority (optional), Requested Guaranteed Bitrate, Requested Maximum Bitrate, Maximum Burst Size and Requested Packet Error Rate. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as described in clause 6.1.3.22 of TS 23.503 [20]. The AF may provide QoS parameter(s) to be measured as defined in clause 5.46 of TS 23.501 [2], Reporting frequency, Target of reporting, an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20]. To activate the Consolidated-MBR monitoring, the AF shall include the UL bitrate or DL bitrate as defined in clause 5.46 of TS 23.501 [2] in the QoS parameter(s) to be measured, Consolidated-MBR threshold, and the Target of reporting set to the NEF.

Editor's note:	Proper reference to the UPF Event Exposure needs further clarification.

Editor's note:	Whether and how to leverage direct or indirect UPF event notification is FFS.

Editor's note:	Whether and how to activate the Consolidated-MBR monitoring during the create procedure is FFS.

Editor's note:	The name of the service is FFS.

2.	The NEF authorizes the AF request and may apply policies to control the overall amount of QoS authorized for the AF. If the authorisation is not granted, all steps (except step 8) are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed. The NEF generates the Transaction Reference ID and associates the list of UE address(es) received from the AF to the Transaction Reference ID.

3-4.	The NEF finds BSF serving the UE IP address(es) using NRF and then the NEF discovers each PCF selected for the UE identified by UE IP address via Nbsf_Management_Discovery service operation.

Steps 5-10 applies for each UE address in the list of UE addresses.

5.	The NEF provides the UE address and the received parameters in step 1 to the PCF in the Npcf_PolicyAuthorization_Create request.

6.	Step 4 in Figure 4.15.6.6-1 applies except the PCF sending the response message to the NEF. The PCF generates authorized QoS Monitoring policy according to the information provided from the NEF in step 5 and provides PCC rules with the policy to the SMF as described in clause 6.1.3.21 of TS 23.503 [20]. The SMF configures the UPF to perform QoS Monitoring as described in clause 5.46 of TS 23.501 [2].

Editor's note:	Proper reference to the UPF Event Exposure needs further clarification.

7.	The PCF sends the Npcf_PolicyAuthorization_Create response message to the NEF.

8.	If the NEF receives the Npcf_PolicyAuthorization_Create response, the NEF sends a Nnef_MultiMemberAFsessionWithQoS_Create response message (Transaction Reference ID, Result for the list of UE address(es)) to the AF. Result for the list of UE address(es) includes whether the request is granted or not for every UE address in the list of UE address(es).

9.	Step 6 in Figure 4.15.6.6-1 applies.

10.	Step 7 in Figure 4.15.6.6-1 applies.

Editor's note:	Whether and how to leverage direct or indirect UPF event notification is FFS.

11.	The NEF sends Nnef_MultiMemberAFsessionWithQoS_Notify message with the aggregated resource allocation status events to the AF.

12.	Based on the parameters received in step 5, the QoS Monitoring events are reported to the NEF via Nupf_EventExposure service operation as described in clause 5.2.26.2.

13.	The NEF sends Nnef_MultiMemberAFsessionWithQoS_Notify message with the aggregated QoS Monitoring events to the AF. If the NEF received Consolidated-MBR threshold in step 1, the NEF aggregates the bitrate received from step 10 and if the aggregated bit rate exceeds the Consolidated-MBR threshold, the NEF sends the event report for Consolidated-MBR Monitoring which includes the aggregated QoS Monitoring event for bitrates and the indication whether the aggregated bitrate is exceeded or not. If the Reporting Frequency received from the AF is periodic, the NEF periodically sends the event report for Consolidated-MBR Monitoring to the AF.

The AF may send Nnef_MultiMemberAFsessionWithQoS_Revoke request containing the Transaction Reference ID to the NEF in order to revoke all the AF sessions related to the Transaction Reference ID. The NEF authorizes the revoke request and triggers the Npcf_PolicyAuthorization_Delete and the Npcf_PolicyAuthorization_Unsubscribe operations for every UE in the list of UE address(es) associated with the Transaction Reference ID.

NEXT CHANGE

4.15.6.13.3	Procedures for Updating Multi-member AF sessions with required QoS for a list of UEs

Figure 4.15.6.13.3-1: Multi-member AF sessions with required QoS for a list of UEs

1.	For established sessions with required QoS for a list of UEs, the AF which controls the operation for a set of multi-member AF sessions invokes the Nnef_MultiMemberAFSessionWithQoS_ Update request via the support of NEF to initiate the update of the UE(s) in the list and/or update of the resource allocations and/or to update of the required type of QoS monitoring for a selected list of UEs to support a specific application (e.g. FL operation).

When the AF requests to update the UE(s) in the list and/or to update either the resource allocation and/or to update the type of QoS monitoring, the AF initiates the Nnef_MultiMemberAFSessionWithQoS_Update request which includes the list of UE addresses, the AF Identifier, Transaction Reference ID, [Flow description information], [QoS Reference or individual QoS parameters], [Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20])]) to NEF for updating the UE(s) in the list and/or the reserved resources and/or the type of QoS monitoring. The Transaction Reference ID provided in the AF session with required QoS update request message is set to the Transaction Reference ID that was assigned by the NEF. The AF may, instead of a QoS Reference, provide one or more of the following individual QoS parameters: Requested 5GS Delay (optional), Requested Priority (optional), Requested Guaranteed Bitrate, Requested Maximum Bitrate, Maximum Burst Size and Requested Packet Error Rate. The Consolidated-MBR threshold for Consolidated-MBR monitoring (optional); in addition, the list of UE addresses subjected for Consolidated-MBR monitoring may be provided. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as specified in clause 6.1.3.22 of TS 23.503 [20].

NOTE 1:	If Consolidated-MBR threshold is provided, the QoS parameter(s) to be measured indicates the Guaranteed Bitrate for the requested AF sessions shall be provided.

NOTE 2:	When the AF request is for Consolidated-MBR monitoring, the QoS Flow data rate reporting for the list of UEs is provided to the AF by the NEF only when the Consolidated-MBR threshold is exceeded.

NOTE 3:	When the Consolidated-MBR threshold is provided, it applies to the list of UE addresses by default. However, if the list of UE addresses subjected for Consolidated-MBR monitoring is also provided together with the Consolidate-MBR threshold, then such list has to be the subset of the list of UE addresses.

NOTE 4:	If the AF wants to terminate the Consolidated-MBR monitoring for the list of AF sessions, the AF does not include the Consolidate-MBR threshold in the AF request.

Editor's note:	Whether and how to leverage direct or indirect UPF event notification is FFS.

2.	The NEF authorizes the multi-member AF session request, and may apply policies to control the overall amount of QoS authorized for the AF. If the authorisation is not granted, all following steps are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed.

3-4.	The NEF finds BSF serving each UE IP address using NRF and then the NEF discovers each PCF selected for the UE identified by UE IP address via Nbsf_Management_Discovery service operation.

5-6. Once the UE(s)' serving PCF(s) are identified, the NEF converts the multi-member AF session request into per-UE individual AF session request. The UE's serving PCF authorizes the NEF request and responds to NEF with Npcf_PolicyAuthorization response according to the success or failure of the authorization.

When the AF requests for Nnef_MultiMemberAFSessionWithQoS_Update in order to update the UE(s) in the list and/or update the resource allocation and/or to update the type of QoS monitoring, the NEF refers to the local context corresponding to the list of UEs and determines whether the new UE is to be added to the list, whether the existing UE is to be removed from the list, and/or whether the resource allocation or the QoS Monitoring type for the existing UE is to be updated by invoking one or more of the following Npcf_PolicyAuthorization procedures:

a)	If the Multi-member AF session update request involves a new UE is to be added to the list, after inquiring BSF via Nbsf_Management Discovery service operation to find out the address of the UE's serving PCF, the NEF initiates the Npcf_PolicyAuthorization_Create as described in clause 5.2.5.3.2.

NOTE 5:	If the Multi-member AF session update request includes both a new UE to be added to the list and the Consolidated-MBR threshold, Consolidate_MBR monitoring can be triggered concurrently when requesting the resource for the new UE.

Editor's note:	Proper reference to the UPF Event Exposure needs further clarification.

Editor's note:	Whether and how to activate the Consolidated-MBR monitoring during the create procedure is FFS.

b)	If the Multi-member AF session update request involves an existing UE is to be removed from the list, the NEF initiates the Npcf_PolicyAuthorization_Delete as described in clause 5.2.5.3.4.

c)	If the Multi-member AF session update request involves either the update the resource allocation or QoS monitoring type or both for an existing UE in the list, the NEF initiates the Npcf_PolicyAuthorization_Update to the UE’s serving PCF as described in clause  5.2.5.3.3.

If the list of UE addresses subjected for the Consolidated-MBR monitoring is received from the AF, NEF updates the list of UEs subjected for Consolidated-MBR monitoring.

7.	The NEF aggregates the authorization responses from the UE(s)’ serving PCF(s) as described in step 6 of clause 4.15.6.6a and sends the Nnef_MultiMemberAFSessionWithQoS Update response corresponding to the earlier AF session request to the AF. The consolidated response for NEF provides specific success or failure response corresponding to each UE. The NEF disassociates the UE IP address(es) to be removed and/or failed to be authorized by the PCF with the existing Transaction Reference ID.

8.	If the NEF requested Npcf_PolicyAuthorization_ Update is authorized by the PCF as described in step 5 above, the PCF processes the Npcf_PolicyAuthorization_Update request according to the actions described in step 4 of clause 4.15.6.6 for SM policy Association Modification procedure and/or QoS Monitoring as described in clause 6.1.3.21 of TS 23.503 [20].

9.	The NEF shall send a Npcf_PolicyAuthorization_Subscribe message to the PCF to subscribe to notifications of Resource allocation status and may subscribe to other events described in clause 6.1.3.18 of TS 23.503 [20].

Editor’s note:	Whether and how to leverage direct or indirect UPF event notification is FFS.

10.	When the event condition is met, e.g. that the establishment of the transmission resources corresponding to the QoS update succeeded or failed, the UE’s serving PCF sends Npcf_PolicyAuthorization_Notify message to the NEF notifying about the event.

If the AF is considered to be trusted by the operator, the UE’s serving PCF sends Npcf_PolicyAuthorization_Notify message directly to the AF.

11.	The NEF consolidates the notification messages from the respective UEs’ serving PCFs before notifying the AF about the events provided by the PCFs via Nnef_MultiMemberAFSessionWithQoS_Notify. The NEF associates the UE IP address(es) which have been newly allocated with resources with the existing Transaction Reference ID.

12.	If the QoS Monitoring event for Data Rate Monitoring is indicated by the SMF, the UPF reports the data rate for the target QoS flow periodically to the NEF via Nupf_EventExposure_Nofity; otherwise, if the QoS Monitoring event for Data Rate Monitoring is revoked by the SMF, the UPF terminate the UPF reporting for data rate.

13.	If the NEF receives the Data Rate Monitoring report of the QoS Monitoring event for specific QoS flow from UPFs, and the Consolidated-MBR threshold was provided from the AF by the prior Nnef_MultiMemberAFSessionWithQoS request, the NEF sums up the data rates of all the active QoS flows according to the list of Ues which are subjected to Consolidate-MBR monitoring during the reporting period. If the given Application AI/ML traffic is asymmetric, the bit rate aggregation is done per uplink and downlink separately; otherwise, the bit rate aggregation is done per either uplink or downlink. The NEF compares the aggregated bit rate of the uplink or downlink against the Consolidated-MBR threshold for the uplink or downlink, accordingly.

14.	In the case of event reporting for Consolidated-MBR monitoring, only if the aggregate bit rate exceeds the Consolidated-MBR threshold, the NEF will notify the Application AI/ML AF via Nnef_MultiMemberAFSessionWithQoS_Notify for the collected UPF QoS monitoring reports together with the status of the Consolidated-MBR monitoring; otherwise, the NEF notifies the AF periodically of the collected UPF QoS monitoring reports together with the status of the of the Consolidate-MBR monitoring, if requested.

The UPF QoS Monitoring event for Data Rate Monitoring continues until the AF terminates the Consolidated-MBR monitoring. The UPF continues to perform the QoS Monitoring event for Data Rate Monitoring for the given QoS flow to the NEF periodically corresponding to the reporting period as described in step 12 above.

NEXT CHANGE

5.2.6.31	Nnef_MultiMemberAFSessionWithQoS service

5.2.6.31.1	General

See clause 4.15.12.

In addition to support the resource allocation to provide specific QoS for a list of Ues, this service is also used to support subscription and notification of QoS Monitoring event for Data Rate Monitoring for a list of QoS flows, as described in clause 5.2.26.

5.2.6.31.2	Nnef_MultiMemberAFSessionWithQoS_Create service operation

Service operation name: Nnef_MultiMemberAFSessionWithQoS Create

Description: The consumer requests the network to allocate resources for a specific list of AF sessions and potentially to perform QoS monitoring (e.g. delay, jitter, bit rate or Consolidated-MBR).

Inputs, Required: AF Identifier, a list of UE addresses (i.e. IP addresses), Flow description information as described in clause 6.1.3.6 of TS 23.503 [20] or External Application Identifier, QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

Inputs, Optional: Time Period, Traffic Volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured, Consolidated-MBR threshold, a list of UE addresses subjected to Consolidated-MBR monitoring, Reporting frequency, Target of reporting and an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], DNN if available, S-NSSAI if available, flow direction.

NOTE 1:	If Consolidated-MBR threshold is provided, the QoS parameter(s) to be measured indicates the Guaranteed Bitrate for the requested AF sessions shall be provided.

NOTE 2:	When the Multi-member AF request is for Consolidated-MBR monitoring, the QoS Flow data rate reporting for the group of Ues is provided to the AF by the NEF only when the Consolidated-MBR threshold is exceeded.

NOTE-3: When the Consolidated-MBR threshold is provided, it applies to the list of UE addresses by default. However, if the list of UE addresses subjected for Consolidated-MBR monitoring is also provided, then such list has to be the subset of the list of UE addresses.

Outputs, Required: Transaction Reference ID, result for the success or failure of the request(s) corresponding to individual UE(s) in the list.

Output (optional): None.

5.2.6.31.3	Nnef_MultiMemberAFSessionWithQoS_Notify service operation

Service operation name: Nnef_MultiMemberAFSessionWithQoS_Notify

Description: NEF reports the QoS Flow level event(s) to the consumer.

Inputs, Required: Transaction Reference ID, Reports of the events as defined in clause 6.1.3.18 of TS 23.503 [20].

NOTE:	When the event report is for Consolidated-MBR monitoring, the QoS Flow data rate reporting for the list of Ues is provided to the AF by the NEF only when the Consolidated-MBR threshold is exceeded.

Inputs, Optional: When the event report is for Consolidated-MBR Monitoring, includes Consolidated-MBR Monitoring report.

Outputs, Required: None.

Output (optional): None.

5.2.6.31.4	Nnef_MultiMemberAFSessionWithQoS_Revoke service operation

Service operation name: Nnef_MultiMemberAFSessionWithQoS_Revoke

Description: The consumer requests the network to revoke the list of AF sessions with required QoS.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: None.

Outputs, Required: Transaction Reference ID, result for the success or failure of the request corresponding to individual UE.

Output (optional): None.

5.2.6.31.5	Nnef_MultiMemberAFSessionWithQoS_Update service operation

Service operation name: Nnef_MultiMemberAFSessionWithQoS_Update

Description: The consumer requests the network to update list of Ues (identified by IP addresses) of the list of AF sessions, the Service Requirement(s) and/or additional Alternative Service Requirement(s) for the list of AF sessions and/or the type of QoS monitoring.

Inputs, Required: Transaction Reference ID, a list of UE addresses (i.e. IP addresses).

Inputs, Optional: Flow description information as described in clause 6.1.3.6 of TS 23.503 [20], QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], time period, traffic volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured, Consolidated-MBR threshold, a list of UE addresses subjected to Consolidate-MBR monitoring, Reporting frequency, Target of reporting and an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], flow direction.

NOTE 1:	If Consolidated-MBR threshold is provided, the QoS parameter(s) to be measured indicates the Guaranteed Bitrate for the requested AF sessions shall be provided.

NOTE 2:	When the AF request is for Consolidated-MBR monitoring, the QoS Flow data rate reporting for the group of Ues is provided to the AF by the NEF only when the Group-MBR threshold is exceeded.

NOTE 3:	When the Consolidated-MBR threshold is provided, it applies to the list of UE addresses by default. However, if the list of UE addresses subjected for Consolidated-MBR monitoring is also provided, then such list has to be the subset of the list of UE addresses.

NOTE 4:	If the AF wants to terminate the Consolidated-MBR monitoring, the AF does not include the Consolidate-MBR threshold in the AF request.

NOTE 5:	What to be updated is determined based on the combination of inputs. If the AF request includes a new UE in the required inputs, the new UE is considered to be a new UE to the list of UE(s). If the AF request includes an existing UE in the required inputs without optinal inputs, the existing UE is considered to be removed from the list of UE(s). If the AF request includes an existing UE in the required inputs with optinal inputs for the resource allocation or QoS Monitoring type, the resource allocation or QoS Monitoring type of the existing UE is considered to be updated.

Outputs, Required: Transaction Reference ID, result for the success or failure of the operation execution result.

Output (optional): None.

END CHANGE