3GPP TSG-WG SA2 Meeting #156-e	S2-2304767

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2096 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of NSWO for 3GPP UE behind 5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to support 5G NSWO (as defined in clause 5.42 of TS 23.501 [2]) for 3GPP UE, 5G-RG shall support the SWa' interface to NSWOF, but the support in 5G-RG is in the scope of BBF and/or Cable Labs specifications. From 3GPP point of view the impact in normative is to describe the architecture, and the support in 5G-RG and W-AGF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause is added to define the reference architceture and the impact in the involved network function. In clause 5.1.1 is added the support of DIAMETER proxy functionality for SWa’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of support of 5G NWSO for 3GPP UE conencted to a 5G-RG |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x (new), 5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change all text is new* * * *

## 4.X	Support for device behind 5G-RG

This clause defines the support of devices behind a 5G-RG

### 4.X.1	Support of NSWO for 3GPP UE behind a 5G-RG

In order to support NSWO defined in clause 5.42 of TS 23.501 [2] for 3GPP UE, 5G-RG shall support the SWa' interface to NSWOF playing the role of WLAN Access, as shown in figure 4.x.1-1 and figure 4.X.1-2. The W-AGF  may act as a 3GPP AAA proxy between the 5G-RG and the NSWOF

The SWa’ reference point corresponds to SWa reference point as defined in TS 23.402 where the EAP user ID is a SUCI and not an IMSI.

It is assumed that SWa’ support in Wireline access network either by 5G-RG or by W-AGF has no impact on 3GPP specification. The specification of support of functionalities in 5G-RG or W-AGF related to Swa’ and the transport of SWa’ over Y4 is under BBF and Cablelabs reposnability.

NOTE: Typically the UE is assumed to be connected via WLAN access to the 5G-RG.

Figure 4.X.1-1: Reference architecture to support authentication for Non-seamless WLAN offload in 5GS

Figure 4.X.1-2: Service based reference architecture to support authentication for Non-seamless WLAN offload in 5GS

Editor’s Note: The specification in this clause may be further revised based on feedabckfrom BBF and Cable Labs

# * * * * Second change * * * *

## 5.1	Network Function Functional description

### 5.1.1	W-AGF

The functionality of W-AGF in the case of Wireline 5G Access network includes the following:

-	Termination of N2 and N3 interfaces to 5G Core Network for control - plane and user-plane respectively.

-	Handling of N2 signalling from SMF (relayed by AMF) related to PDU Sessions and QoS.

-	Relaying uplink and downlink user-plane packets between the 5G-RG and UPF and between FN-RG and UPF. This involves:

-	Enforcing QoS corresponding to N3 packet marking, taking into account QoS requirements associated to such marking received over N2.

-	N3 user-plane packet marking in the uplink.

-	Supporting AMF discovery and selection defined in TS 23.501 [2] clause 6.3.5 where the 5G-S-TMSI is not used for AMF selection since the wireline AS layer can only carry the GUAMI.

-	Termination of wireline access protocol on Y4 and Y5.

-	In the case of FN-RG the W-AGF acts as end point of N1 on behalf of the FN-RG.

- 	Support of SWa’ reference point toward the NSWOF and toward 5G-RG and Diameter proxy functionalities. The specification of transport of SWa’ DIAMETR over Y4 is under BBF and Cablelabs responsibility.

Editor’s Note: The above specification may be further revised based on feedabckfrom BBF and Cable Labs

In the case of Wireline 5G Broadband Access network the definition of W-AGF functionalities is specified in BBF TR-456 [9] and WT-457 [10].

NOTE:	The W-AGF is specified as AGF (Access Gateway Function) in BBF TR-456 [9] for supporting 5G-RG and FN-RG and as FMIF (Fixed Mobile Interworking Function) for supporting FN-RG only in the case of presence of BNG in WT-457 [10]. Both cases for FN-RG support, i.e. AGF and FMIF, have identical interfaces towards 5GC, i.e. it is transparent to 5GC whether AGF or FMIF is used and no difference between AGF or FMIF cases is defined in this specification.

In the case of Wireline 5G Cable Access network the definition of W-AGF functionalities is specified by Cablelabs WR-TR-5WWC-ARCH [27].

# * * * * Third change * * * *

# * * * * Fourth change * * * *

# * * * * Fifth change * * * *

# * * * * End of changes * * * *