3GPP TSG-SA WG2 Meeting #156e 	S2-2304811

April 17 – 21, 2023, e-meeting	()

Source:	vivo

Title:	TS 23.586: SL positioning without NAS and 5GC NF involvement

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper tries to implement the procedures for Ranging/Sidelink Positioning device discovery based on the conclusions of KI#3 in TR23.700-86.

1. Introduction/Discussion

The following conclusions in TR23.700-86 for the KI#5 should be captured:

-	If Target UE cannot establish NAS connection with AMF, for MO-LR or pending MT-LR (e.g. deferred MT-LR), the following procedures can be used:

Option a):

-	The Target UE performs the Located UE's discovery and selection.

-	The Target UE may request for the Location of the Located UE(s).

-	If the Located UE(s) needs network assistance to retrieve its location, the Located UE(s) may trigger a MO-LR. This assumes that the Located UE(s) has a NAS connection to an AMF.

-	The Target UE uses the location returned by Located UE(s) together with the ranging/SL positioning measurement data or result to estimate its own location. The Ranging/Sidelink positioning and the positioning of the Located UE(s) can be scheduled with the same time (according to TS 23.273 [11]) to improve the Target UE positioning accuracy.

2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

### 5.5.4	SL positioning for target UE without NAS connection when 5GC NF is not involved

When Target UE cannot establish the NAS connection with AMF, the Target UE can use the Location of the Located UE(s) and the ranging/SL positioning measurement data or result with Located UE(s) to estimate its own location without the 5GC NF involvement in the SL positioning. The basic principles are listed as follows:

-	The Target UE performs the Located UE's discovery and selection.

-	The Target UE requests for the Location of the Located UE(s).

-	If the Located UE(s) needs network assistance to retrieve its location, the Located UE(s) may trigger a MO-LR procedure as defined in TS23.273[8].

-	The Target UE uses the location returned by Located UE(s) together with the ranging/SL positioning measurement data or result to estimate its own location. The Ranging/Sidelink positioning and the positioning of the Located UE(s) can be scheduled with the same time (according to TS 23.273 [8]) to improve the Target UE positioning accuracy.

Editor’s Note: Security aspects require confirmation from SA WG3.

# * * * * End of changes * * * *