3GPP TSG-SA WG2 Meeting #156e 	S2-2304816

April 17 – 21, 2023, e-meeting	(was)

Source:	vivo

Title:	TS 23.586: SL Positioning Server UE Discovery & Selection

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper tries to adds the High level functionality and features for SL Positioning Server UE Discovery & Selection.

1. Introduction/Discussion

This paper adds the High level functionality and features for SL Positioning Server UE Discovery & Selection

2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

5.2.3	SL Positioning Server UE Discovery & Selection

The discovery of SL Positioning Server UEs follows the same principles as specified in clause 5.2.1. The UE can indicate its role “SL Positioning Server UE” in its list of supported roles during discovery, if it is authorized to be a SL Positioning Server UE in a given PLMN as per the Authorization and Provisioning for Ranging/SL positioning service as specified in clause 5.1.

A Target UE or Reference UE discovers and selects the SL Positioning Server UE when it doesn’t support the functionalities of the SL Positioning Server UE in case of being out-of-coverage or for UE-only Operation indicated by its serving network of not supporting Ranging/SL Positioning.

# * * * * End of changes * * * *