3GPP TSG-SA WG2 Meeting #156e 	S2-2304817

April 17 – 21, 2023, e-meeting	(was)

Source:	vivo

Title:	TS 23.586: Subscription updates

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper tries to implement the procedures for Ranging/Sidelink Positioning device discovery based on the conclusions of KI#3 in TR23.700-86.

1. Introduction/Discussion

This paper provides some updates to the subscriptions for Ranging/SL Positioning.

2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

## 5.8	Subscription to Ranging/SL Positioning

The subscription information in the UDM contains information to give the user permission to use 5G Ranging/SL Positioning.

At any time, the operator can amend or remove the UE subscription rights from subscription information in the UDM, or to revoke the user's permission to use Ranging/SL Positioning.

The following subscription information is defined for Ranging/SL Positioning in addtion to the part defined in TS23.287[6] and TS23.304[7]:

-	subscription for UE acting as Target UE.

-	subscription for UE acting as SL Reference UE.

-	subscription for UE acting as Located UE.

-	subscription for UE acting as Assistant UE.

-	subscription for UE acting as SL Positioning Client UE.

-	subscription for UE acting as SL Positioning Server UE.

Editor's note:	FFS if subscription for acting as Assistant UE is needed.

-	Ranging/SL positioning QoS parameters.

-	the list of the PLMNs authorized for Ranging/SL Positioning services, including:

-	the list of the PLMNs where the UE is authorised for perform Ranging/SL positioning service over PC5 reference point.

-	the list of the PLMNs where the UE is authorised for Ranging/SL positioning service with Direct Discovery Model A, i.e. to announce or monitor or both.

-	the list of the PLMNs where the UE is authorised for Ranging/SL positioning service with Direct Discovery Model B, i.e. to perform Discoverer operation or Discoveree operation or both.

-	the list of the PLMNs where the UE is authorised to act as a Target UE.

-	the list of the PLMNs where the UE is authorised to act as a SL Reference UE.

-	the list of the PLMNs where the UE is authorised to act as a Located UE.

-	the list of the PLMNs where the UE is authorised to act as a SL Positioning Client UE.

-	the list of the PLMNs where the UE is authorised to act as a SL Positioning Server UE.

# * * * * End of changes * * * *