3GPP TSG-WG SA2 Meeting #156e	S2-2304824

# E-meeting, April 17th – 21st, 2023						                              revision of S2-23xxxx

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0131 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Usage monitoring for HR-SBO PDU Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per usage monitoring control requirement in clause 4.3.4 of TS 23.503, the usage monitoring capability shall be also possible for all traffic of a HR-SBO PDU Session. To address this, it is required to enable V-SMF to report H-SMF with the N4 information corresponding to traffic usage reporting for V-PSA UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing a general description on usage monitoring for HR-SBO PDU Session |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Usage monitoring control requirements cannot be satisfied for HR-SBO PDU Session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st Changes * * * *

6.7	Support of the local traffic routing in VPLMN for Home Routed PDU Session for roaming (HR-SBO)

6.7.1	General

When roaming, the UE establishes a Home Routed Session that is capable of supporting session breakout in V-PLMN based on the subscription. In this scenario, the Home PLMN and Visited PLMN have an agreement on the support of the local traffic routing (i.e. session breakout performed by V-SMF also called HR-SBO) in VPLMN for the home routed session.

After establishing the HR-SBO PDU Session, the UE can access EAS deployed in EHE in VPLMN while the UE can also access the data network in the Home PLMN.

If the HR-SBO PDU Session is established, it is required to enable the traffic usage monitoring in HPLMN for the total traffic on the HR-SBO PDU Session (i.e., traffic locally routed from/to the local part of DN in VPLMN and home-routed traffic from/to the H-UPF). For the purpose of this usage monitoring on HR-SBO PDU Session, the V-SMF managing the HR-SBO PDU Session sends to the H-SMF the N4 information corresponding to the traffic usage reporting, which includes the usage information for the traffic from and to the local part of DN in VPLMN as a part of HR-SBO. To enable this, the H-SMF provides the V-SMF with the corresponding Usage Monitoring Control related information to be applied in VPLMN during the HR PDU Session Establishment procedure if the HR-SBO is authorized for the PDU Session.

The reference architecture supporting this scenario is depicted in Figure 4.2-5 in clause 4.2.

# * * * * End Changes * * * *