3GPP TSG-SA WG2 Meeting #156E	S2-2304850

# E-meeting, April 17 - 21, 2023													 (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4417 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on PDU Session for PIN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following ENs in clause 5.44.3.1 need to be resolved. [EN#1] Editor's note: How and whether to handle the case where PINs share a PDU session and local switching is FFS. [EN#2] Editor's note: One PIN served by more than one PDU sessions in PEGC is FFS. [EN#3] Editor's note: The handling of the PEMC in 5GC in relation with PIN is FFS. For EN#1, it is proposed that a PDU Session is not shared by more than one PINs. For EN#2, a PEGC may establish more than one PDU Session for a PIN as specified in clause 5.8.2.2.1 as below: In the case that the initial PDU Session was established with a PDU Session Type and the UE needs another single IP version PDU Session Type, the UE may initiate another PDU Session Establishment procedure to this (DNN, S-NNSAI) in order to activate a second PDU session with that PDU Session Type. For EN#3, it is proposed to just remove this EN because the handling of PEMC related to PDU Session Establishment for PIN is not needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §5.44.3.1 - Remove all ENs. - Describe that a PDU Session is not shared by more than one PINs with NOTE saying that the URSP rules for PIN are properly set to ensure that more than one PINs do not share same PDU Session. - Add clause 5.8.2.2.1 regarding that a PEGC may establish more than one PDU Sessions for a PIN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved ENs exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.44.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.44.3.1	PDU Session Establishment for PIN

When a PDU Session associated with a PIN is established by UE with PEGC, an SMF is selected according to clause 4.3.2.2.3 of TS 23.502 [3]. The UE with PEGC may use IP address allocation methods as specified in clause 5.8.2.2 (e.g. IPv6 Prefix Delegation feature).

One PEGC may serve more than one PINs and in this case the PEGC shall have at least one PDU Session for each PIN if the PIN traffic is via PEGC/5GC. The PEGC may establish more than one PDU Sessions for a PIN as specified in clause 5.8.2.2.1. A PDU Session is not shared by more than one PINs.

NOTE:	It is assumed that the URSP rules for PIN are properly set to ensure that more than one PINs do not share same PDU Session.

* * * * End of Changes * * * *