3GPP TSG-WG SA2 Meeting #156-e	S2-2304856

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4422 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of local switch via UPF on GEO satellites |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSATB |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some editorial/clarificaion changes are needed for the local switch via UPF on GEO satellites, e.g., SMF can reuse a UPF (UL CL/BP/local PSA) that is already inserted for a UE’s PDU session in the case of UL CL/BP and local PSA UPF deployed on satellite, thus when SMF perform the UL CL/BP/local PSA selection and insertion, a ‘may’ is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify SMF ‘may’ select and insert the UPF deployed on GEO satellite according to the DNAI as UL CL/BP and L-PSA - Some other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL CL/BP/Local PSA may not be reused. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.43.3.1, 5.43.3.2, 5.43.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.43.3	Local switch for communication between UEs via UPF deployed on GEO satellite

5.43.3.1	General

The traffic between UEs may be locally routed by UPF(s) deployed on satellite (i.e., through local switch) to the target UE without traversing back to the satellite gateway on the ground.

Local switching via UPF(s) deployed on satellite in this clause only applies on GEO satellite backhaul case and considers only DNNs and slices for 5G VN.

N19 tunnel may be established between two UPFs deployed on different satellites for traffic between UEs. Also, N6 may be used for carrying traffic between UPFs deployed on different satellites.

Only a single SMF is supported for local switching and N19 forwarding, i.e. both UEs are served by the same SMF.

NOTE:	The latency optimisation that can be gained by inter-satellite link between UPFs on different GEO satellites depends on the distance between the satellites that can be substantial, depending on the number of deployed satellites.

Clause 5.43.3.2 describes the case of PSA UPF deployed on satellite, clause 5.43.3.3 describes the case of UL CL/BP and local PSA deployed on satellite (PSA UPF is on the ground). Selection of PSA UPF or UL CL/BP/local PSA on satellite is described in clause 6.3.3 and determination of DNAI to select the UPF deployed on the corresponding GEO satellite reuses the mechanism described in clasuse 5.43.2.

A combination of DNN/S-NSSAI is assigned by the operator for the communications between UEs where backhaul with UPF is deployed on GEO satellite, the URSP is described in TS 23.503 [45] and its configuration to enable the selection PSA UPF on the GEO satellite reuses the mechanism described in clasuse 5.43.2.

# * * * * Second change * * * *

5.43.3.2	 Local switching with PSA UPF deployed on satellite

If SMF selects the UPF deployed on satellite as PSA of UE’s PDU Session, the SMF configures the UE’s N4 session to forward/detect packet to/from the internal interface as specified for the configuration for the 5GVN group member's N4 Session in clause 5.8.2.13.1 (Support for unicast traffic forwarding of a 5G VN).

SMF may reuse the mechanism described in clause 5.8.2.13.1 to configure group-level N4 session rules for each N19 tunnel.

For establishing N19 tunnel between the PSA UPFs onboard the satellite, the PSA UPFs are controlled by the same SMF.

-	To process packets between UE and servers residing in DN, SMF configures rules to route traffic via N6 as described in clause 5.8.2.13.1.

The group-level N4 session is per DNN and S-NSSAI. The SMF can create, update or delete the group-level N4 Session, i.e., add or delete N4 rules, allocate or release the N19 tunnel resources based on operator deployment, e.g., based on GEO satellite’s planned obsolescence or new GEO satellite setup.

N6 may be used for carrying traffic between PSA UPFs deployed on different satellites. If N6 is used, SMF configures corresponding N4 rules for processing traffic to/from N6.

# * * * * Third change * * * *

5.43.3.3	Local switching with UL CL/BP and local PSA UPF deployed on satellite

For UPF deployed on satellite acting as UL CL/BP/local PSA case, the PSA UPF anchored by UE PDU sessions is on the ground, if the UEs are served by the same SMF, the SMF may determine to activate local switching and N19 forwarding for the UEs using the GEO satellite backhaul, based on:

1)	AF request including UE identifiers which require communication between UEs as described in clause 5.29.2;

2)	Destination IP address(es) reported by on-ground PSA UPF as current reporting mechanism in clause 5.8.2.11.7. To enable the destination IP address(es) reporting, SMF configures the on-ground PSA UPF to detect UL packets with destination IP addresses which belong to the current UPF address pool.

If SMF determines that the UEs (i.e., corresponding to AF request in bullet 1) and Destination IP address(es) reported in bullet 2)) are under same GEO satellite (or multiple connectable GEO satellites) based on GEO Satellite ID(s) reported by AMF, and the UEs are allowed to access the DNAIs corresponding to the GEO satellite IDs, for each UE communicating with target UE(s) in the communication group, the SMF may select and insert the UPF deployed on GEO satellite according to the DNAI as UL CL/BP and L-PSA, and configures UL CL/BP with the following rule:

-	Route the data traffic received from the UE and destined to IP address(es) of the target UE(s) to the L-PSA.

-	Route other data traffic received from the UE to the PSA UPF of the UE's PDU Session.

The SMF configures the Local PSA with local forwarding rules to forward the data traffic to the target UEs directly. If the selected L-PSAs are different for the UEs in the communication group, N19 tunnel is established between the L-PSAs. For establishing N19 tunnel between the UPFs onboard the satellite, the UPFs are controlled by the same SMF. If UEs are members of the same 5G VN group, the SMF may configure the local data forwarding rules on L-PSA(s) using 5GVN user plane handing mechanism in clause 5.8.2.13.1 (Support for unicast traffic forwarding of a 5G VN).

NOTE:	The selected UPF deployed on satellite can be inserted as UL CL/BP/L-PSA reusing existing UL CL/BP insertion procedures defined in TS 23.502 [3] or TS 23.548 [130].

N6 may be used for carrying traffic between L-PSA UPFs deployed on different satellites. If N6 is used, SMF configures corresponding N4 rules for processing traffic to/from N6.

# * * * * End of changes * * * *