3GPP TSG-WG SA2 Meeting #156e	S2-2304928

Elbonia, April 17 – 21, 2023

Source:	Huawei, HiSilicon

Title:	Discussion on Support of local switching

Document for:	Decision

Agenda Item:	9.3.2

Work Item / Release:	PIN / Rel-18

Abstract: This paper discusses the EN in the TR regarding whether PINs can share a PDU session and how to support local switching, and proposes way forward.

1.	Overall Description

There is an Editor’s Note in clause 5.x.c.1 of TS 23.501 as follows.

Editor's note: How and whether to handle the case where PINs share a PDU session and local switching is FFS.

On the PINs sharing a single PDU session, this paper describes some issues with it and proposes that a single PDU Session shall only serve a single PIN.

Regarding the local switching, one solution is proposed in this paper how to support it, considering the benefit of local switching to have optimized routing without introducing loopback traffic over N6, compared with pure N6 routing, with which the UPF simply throws traffic to N6 and then come back to it in case the same UPF serves the different PDU Sessions for the same PIN.

2.	Discussion

2.1 Support the local switching based on PIN ID

There is an indirect PINE to PINE communication via PEGC and UPF for a PIN. In order to support the traffic routing between the PINEs via the different PEGC served by the same UPF, the N6 routing or the local switching can be applied. Obviously local switching solution have optimized routing than the N6 based solution. So in this part it discusses how to support the local switching for one PIN.

Each PEGC establishes the PDU session related to the PIN to route the traffic between PINEs connected via different PEGCs of the same PIN. These two PDU sessions may connect to the same or the different UPFs. Based on the current specification, the 5G VN group feature supports the local switching between PDU sessions from member of the 5G VN group in one UPF and between the two different UPFs via N19 reference point. However the 5G VN group feature is not suitable for the PIN case for the following reasons, though we can take PIN as a 5G VN group and the PEGCs of the same PIN are members of the 5G VN group:

Case 1) Two PDU sessions from different PEGCs for the same PIN connect to the same UPF.

In order to support the local switching for one PIN in the UPF, firstly, the UPF shall identify that this PDU session is related to the PIN feature and the packets in this PDU session will be transported to the internal interface instead of the N6 interface. Similar as 5G VN group solution, this can be achieved by reusing the "5G VN Internal" IE set to the Destination Interface. Secondly, the UPF shall identify which PIN the PDU session belongs to during the packet detection and forwarding process. As one UPF may serve more than one PIN, an IE representing the PIN group is needed in addition to the internal interface indication. There are two options to set the IE representing the PIN group:

Option 1: The identification PDU session for PIN is based on combination of DNN and S-NSSAI.

Option 2: The identification PDU session for PIN is based on combination of PIN ID, DNN and S-NSSAI.

In the Option 1, similar as 5G VN group mechanism, the combination of DNN and S-NSSAI is applied to identify the PDU sessions belonging to a specific 5G VN group. While it should be noted that the number of PINs may be orders of magnitude greater than the number of 5G VN groups, so millions of different DNNs and slices are required, which obvious does not work. Therefore, in additional to the DNN and S-NSSAI which can be shared by all the PIN service, other information is needed to identify the specific PIN. This additional information may be PEGC SUPI, PEGC SUPI list for the PIN, or a new defined PIN ID. These will be analysed one by one as below.

If the PIN is identified by PEGC SUPI and DNN and S-NSSAI, it cannot support the case when a PIN includes multiple PEGCs. For example, there exists a situation where the DNN+S-NSSAI+PEGC_A SUPI may identify the same PIN as DNN+S-NSSAI+PEGC_B SUPI if both PEGC_A and PEGC_B belong to the same PIN. The consequence is that even the extended identifier DNN+S-NSAAI+PEGC SUPI allows to differentiate PDU sessions from different PEGC but not if two PDU sessions belong to the same PIN via different PEGC.
Further extending the identifier to include a list of all the PEGCs that serve a PIN would address the issue, but it will become quite laborious. The PEGCs’ SUPI and the PEGC SUPI list for the PIN are all sent by the SMF to the UPF. The UPF will firstly identify a PIN PDU session based on the DNN+S-NSSAI, and then check if different PDU sessions from different PEGCs belonging to the same PIN based on PEGCs’ SUPI list. These processing impact the forwarding efficiency seriously.

Based on the above analysis, it is proposed to use the PIN ID sent from SMF to UPF to identify the PDU sessions for a PIN. As proposed in Option 2, the UPF can identify the specific PIN based on the specific PIN ID directly. The SMF can obtain the PIN ID for the PDU session based on the subscription data from UDM or in the PDU session establishment request message from the UE. The PIN ID is stored in the UDM which uniquely identifies the PIN information added to the subscription data. The SMF could identify the PIN ID based on the DNN+S-NSSAI received in the PDU session establishment procedure and the correlation between DNN+S-NSSAI and PIN ID in the received subscription data of this PIN from the UDM. For example, PEGC_A and PEGC_B belongs to the same PIN ID_1, PEGC_C belongs to the PIN ID_2and PIN ID_3. PEGC_A subscription data includes DNN_PIN_game/ S-NSSAI and PIN ID_1, PEGC_B subscription data includes DNN_PIN_game/ S-NSSAI and PIN ID_1, PEGC_C subscription data includes DNN_PIN_game/ S-NSSAI with PIN ID_2, and DNN_PIN_children / S-NSSAI with PIN ID_3. Based on the DNN_PIN_game / S-NSSAI received in the PDU session establishment procedure, the SMF identify the PEGC_A and PEGC_B’s PDU session of PIN ID_1, and PEGC_C’s PDU session of PIN ID_2. If the DNN_PIN_children / S-NSSAI is received for PEGC_C, the PIN_ID_3 is identified by the SMF.

To sum up, different from 5G VN Group, the PIN ID, in addition to DNN and S-NSSAI, uniquely associates the PDU session or PDU sessions to each PIN, which also solves the problem of having multiple PEGCs in the same PIN.

Observation 1: 5G VN group mechanism to support local switching in one UPF, based on associating the PDU Sessions per 1:1 mapping between the DNN and S-NSSAI and VN Group is not applicable to PIN.

Proposal 1: To define the PIN ID in addition to the pair DNN/S-NNSAI to uniquely identify the PDU session(s) associated to a PIN, so that the UPF can support the local switching based on the identification of the PIN for the PDU session.

Case 2) Two PDU sessions from different PEGCs for the same PIN connect to the different UPFs.

As defined in 5G VN group, N19 tunnel is established between the two UPFs for traffic routing between the PDU sessions from members of the 5G VN group. However, this N19 tunnel solution is not applicable for the PIN case. Firstly, the N19 tunnel is established per 5G VN group granularity, so in the PIN case it will be per PIN granularity. As the PIN is defined for Personal IoT network, which results in much more number of N19 tunnels established for PIN than for 5G VN group. Secondly, the N19 tunnel is shared by the group members. But for the PIN, there is not so many PIN members than 5G VN group members to benefit from this N19 tunnel, then it is foreseen the N19 tunnel for PIN will be established and released more frequently than for 5G VN group. And when one PEGC moves, e.g. the PEGC for wearable equpiments moves, and the other PEGC for smart home is static, this may trigger the upate on the N19 tunnels. So huge number of N19 tunnels may need to be frequently established, released and udpated, which would lead to too much resource consumption and make the N19 solution not workable for the PIN.

Observation 2: N19 tunnel based solution for 5G VN group is not applicable to PIN, then no solution is available and further investigation is needed on how to support local switching in different UPFs scenario for PIN.

Proposal 2: Local switching between UPFs serving the different PDU Sessions for the traffic routing between two PINEs connected to the different PEGCs for the same PIN is not supported in this release.

2.2 	Multiple PINs sharing a single PDU session.

The local switching can be supported in one UPF based on the identification of PIN ID for the PDU session. Additionally, the UPF can also support the broadcast in PIN group based on the identification of PIN ID. However, if the PDU session is shared by more than one PIN, the broadcast feature cannot be supported. Figure 1 below shows an example of multiple PINs sharing a single PDU session established by a PEGC, with all the PINs sharing the same DNN and S-NSSAI denoted as DNN-1 and S-NSSAI-1 in the figure.

Figure 1: Multiple PINs Share One PDU Session and Local Switching

The table below shows the relationship among PINEs, PEGCs and PDU sessions.

| PIN 1 | PINE 1A | PEGC 1 | PDU Session 1 | DNN-1, S-NSSAI-1 |
| --- | --- | --- | --- | --- |
|  | PINE 1B | PEGC 2 | PDU Session 2 | DNN-1, S-NSSAI-1 |
| PIN 2 | PINE 2A | PEGC 1 | PDU Session 1 | DNN-1, S-NSSAI-1 |
|  | PINE 2B | PEGC 2 | PDU Session 2 | DNN-1, S-NSSAI-1 |
| PIN 1 | PINE 3 | PEGC 3 | PDU Session 3 | DNN-1, S-NSSAI-1 |

In this example, following assumptions are applied:

- PINE 1A and PINE 1B belong to PIN 1, while PINE 2A and PINE 2B belong to PIN 2.

- For PIN 1, PINE 1A and PINE 1B connects with PEGC 1 and PEGC 2 respectively.

- For PIN 2, PINE 2A and PINE 2B connects with PEGC 1 and PEGC 2 respectively.

- For PIN1, PINE 3 connects with PEGC 3.

- Both PEGC 1 and PEGC 2 establish a single PDU session to serve PIN 1 and PIN 2.

- The PDU sessions established by PEGC 1, PEGC 2 and PEGC 3 are using the same DNN and S-NSSAI.

- The PIN traffic can use local switching

Since the PDU session 1 is shared by PIN 1 and PIN 2, then it is not possible for the UPF to differentiate the broadcast traffic for PIN 1 or PIN 2. Therefore, this may cause the issue when PEGC deliver broadcast traffic for a specific PIN.

Figure 2 shows the example of broadcast traffic transmission for PIN 1 within one PIN by using local switching defined by subclause 2.1.

Figure 2: Broadcast Traffic Transmission by Using Local Switching

Assume that PINE 1A generates a broadcast traffic for PIN 1. PEGC 1 will transmit the broadcast traffic by using broadcast address (e.g. broadcast IP address or MAC address). When UPF PSA receive the broadcast traffic, it will replicate and send to all PEGCs belong to the PIN 1(i.e. PEGC 2 and PEGC 3 in the example). However, firstly, the UPF cannot know this broadcast packet is for PIN 1 or PIN 2, so it cannot decide whether to duplicate the packet to PEGC3 or not. Additionally, if the UPF replicate the packet to PEGC 2, the PEGC 2 will send to all other PINEs they are connected with (i.e. PINE 1B, PINE 2B in the figure). This way, PINE 2B will also receive the broadcast traffic although it was only intended for members of PIN 1.

In addition, when multiple PINs share a single PDU session, there is no resource isolation among those multiple PINs. Therefore, transmitting broadcast traffic will lead to privacy issue, which is not aligned with PIN service.

Observation 3: To support broadcast service, the PDU sessions established by PEGCs for a PIN cannot be shared with other PIN(s).

Proposal 3: One PEGC, when serving more than one PIN, shall have at least one distinct PDU Session for each PIN.

3. Conclusion and proposal(s)

Based on the analysis in this paper we propose the following way forward:

Proposal 1: To define the PIN ID in addition to the pair DNN/S-NNSAI to uniquely identify the PDU session(s) associated to a PIN, so that the UPF can support the local switching based on the identification of the PIN for the PDU session.

Proposal 2: Local switching between UPFs serving the different PDU Sessions for the traffic routing between two PINEs connected to the different PEGCs for the same PIN is not supported in this release.

Proposal 3: One PEGC, when serving more than one PIN, shall have at least one distinct PDU Session for each PIN.