3GPP TSG-SA WG2 Meeting #156-e	S2-2304961

# Elbonia, April 17 – 21, 2023	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4443 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network control of the slice usage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For the slice usage control there is one open issue on whether the slice usage control can be applied to roaming case. One typical example of the slice usage control is for NSAC usage. In Rel-17 the NSAC support the roaming control if there are SLA agreement between operator. Hence similar consideration can be applied here, i.e. if there are SLA agreement between operators, the slice usage control can also be applied to VPLMN. For the UE slice usage control, two informaiton is sent to the UE, - Indication on whether the slice is on demand slice. This is associated with the configured S-NSSAI, i.e. it also apply to initial registration. - Deregistration inactivity timer value, it is associated with the Allowed S-NSSAI as it is related to how long the registered slice is to be deregistered. - For the slice usage control of the PDU session, it seems the existing PDU session inactivity timer is reused. However this timer is only used for PDU session UP deactivation. It is benefit to have two independent timers, one for UP deactivation, the other for PDU session release. It is propsed to distinguish these two timers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the slice usage control for the roaming case and also how it is associated with the configured S-NSSAI and Allowed S-NSSAI. - Update the PDU session inactivity timer to PDU session slice usage inactivity timer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Unclear whether the slice usage control can be applied to roaming case. - Mixed usage of the UP deactivation timer for PDU session release. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.15.1, 5.15.15.2, 5.15.15.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.15.15	Support of Network Slice usage control

5.15.15.1	General

Network Slice usage control is achieved as follows:

1)	Configuring network-controlled Slice Usage Policy to supporting UEs (see clause 5.15.15.2).

2)	Configuring PDU Sessions inactivity timers, and Network Slice deregistration inactivity timers (see clause 5.15.15.3).

5.15.15.2	UE Configuration of network-controlled Slice Usage Policy

The UE during the Registration procedure may indicate in UE MM Core Network Capability that it supports UE configuration of network-controlled Slice Usage Policy. If so, the AMF determines Slice Usage Policy for a Network Slice for the UE and may configure the UE with this information together with Configured NSSAI to control the usage of this Network Slice. The AMF may be locally configured with network Slice Usage Policy, or receive the policy from the (AM-)PCF, or per the information received from UDM for AF managed timer values (see clause 5.15.15.3 for more details).

The network-controlled Slice Usage Policy is provided to the UE in the Registration Accept or the UE Configuration Update Command and may include:

-	An indication, for one or more of S-NSSAI(s) of the HPLMN in the Configured NSSAI, whether the UE only registers with the Network Slice with the network when applications in the UE require data transmission in the Network Slice (i.e. the UE can only register the Network Slice only on demand and consider the Network Slice as on demand S-NSSAI). In the non roaming case, this indication is associated with the Configured NSSAI for HPLMN. In the roaming case, this indication is associated with the Configured NSSAI for serving PLMN.

NOTE:	All Other Network Slices in the Configured NSSAI are handled by the UE using UE specific policies (e.g. they may be registered irrespective of applications need).

-	For each on demand S-NSSAI in the Configured NSSAI, there is always a deregistration inactivity timer that causes the UE to deregister the Network Slice after the last PDU Session associated with the S-NSSAI is released. The AMF receives deregistration inactivity timer values as described in clause 5.15.15.3. When the UE registers on demand S-NSSAI(s) to the network, it receives the deregistration inactivity timer value(s) associated with related Allowed S-NSSAI(s). This deregistration inactivity timer is started at the UE and AMF per access type when the last PDU Session associated with the S-NSSAI is released, or the Network Slice is included in the Allowed NSSAI and no PDU session is established. The deregistration inactivity timer is stopped and reset when the first PDU session is established or the S-NSSAI is removed from the Allowed NSSAI. The AMF and UE may locally remove the S-NSSAI from the Allowed NSSAI when the timer expires. The AMF may also send a UE Configuration Update Command to remove the slice from the Allowed NSSAI.

If the UE and network state became misaligned, the UE may, for example, request connectivity in a Network Slice which is no longer allowed. In this case, the AMF shall provide the updated Allowed NSSAI in a UE Configuration Update Command after rejecting the PDU Session establishment. The UE may then re-register with the Network Slice if needed.

The AMF receives deregistration inactivity timer values as described in clause 5.15.15.3.

5.15.15.3	Network-based per UE Network Slice usage behaviour control

The 5GC performs Network Slice usage monitoring to be able to enforce the release of inactive PDU Sessions, and deregistering of UEs from Network Slices with no PDU Sessions on them according to its own policies. In order to support usage monitoring for a Network Slice:

-	the AMF runs a slice deregistration inactivity timer per S-NSSAI and access type to deregister the Network Slice which is started when the Network Slice is not used by any PDU Session over the corresponding access type. The slice deregistration inactivity timer is stopped and reset when at least a PDU Session associated with the Network Slice is successfully established or the Network Slice is removed form the Allowed NSSAI. When the slice deregistration inactivity timer for a Network Slice over an access type expires, the AMF removes the Network Slice from the Allowed NSSAI over the access type by sending the UE Configuration Update Command to impacted UE(s).

-	the SMFs provide to UPFs that handle the PDU sessions in the Network Slice a PDU Session slice inactivity timer. The PDU Session slice inactivity timer is started after no data packet is transmitted or received and runs until the next data packet is transmitted or received which restarts the timer again. If the PDU Session slice inactivity timer expires before any packet is received or transmitted, the UPF reports this PDU Session slice inactivity event to the SMF to cause the SMF to release the PDU Session. While releasing the PDU session the SMF may indicate the release cause because of slice inactivity. When the AMF receives the notification of PDU Session release and it includes the release cause of slice inactivity and if the Network Slice of the released PDU Session is not used by other PDU Sessions (i.e. the last PDU Session using the Network Slice is released) over the corresponding access type, the AMF may trigger the UE Configuration Update procedure to remove the Network Slice from the Allowed NSSAI over that corresponding access type or start slice deregistration inactivity timer for the Network Slice.

If an S-NSSAI is dedicated for a single AF, and if authorized by operator policy to provide deregistration inactivity/PDU Session slice inactivity timer values for the S-NSSAI, the AF uses external parameter provisioning procedure to provide deregistration inactivity and PDU session slice inactivity timer values as described in clause 4.15.6.2 of TS 23.502 [3]. In this case, the AF provided timer values are stored in the UDM and provided to the AMFor SMF as part of subscription data for the corresponding S-NSSAI. If no AF is authorized to provide deregistration inactivity timer values or PDU Session slice inactivity timer values for the S-NSSAI, the slice deregistration inactivity timer value and PDU Session slice inactivity timer value are either pre-configured in the AMF/SMF or received by the AMF/SMF during the AM Policy Association / SM Policy Association procedure respectively. In the roaming case, the AMF receives the deregistration inactivity timer values from UDM. For the LBO PDU session the SMF receives the PDU Session slice inactivity timer valus from UDM.

If AMF receives a deregistration inactivity timer value associated with an S-NSSAI, besides using this timer value for slice usage control as described above, the AMF notifies UE, which supports UE configuration of network-controlled Slice Usage Policy, that the associated configured S-NSSAI for serving PLMN is a on demand Slice and the deregistration inactivity timer value for the associated Allowed S-NSAI.

If the SMF receives a PDU Session slice inactivity timer valu associated with an S-NSSAI, the SMF use this timer value for slice usage control.

In roaming case, the usage of the deregistration inactivity value or PDU Session slice inactivity timer values at the AMF or SMF (for LBO PDU session) is per SLA agreement.

To enable a serving network to direct UEs to a preferred Network Slice, the AMF may request the UE to transfer a PDU Session from one S-NSSAI to another S-NSSAI as described in clause 5.15.19.

* * * * End of Changes * * * *