3GPP TSG-WG SA2 Meeting #156e	S2-2305011

# Elbonia, April 17 – 21, 2023		(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0135 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AF triggered EAS Re-discovery Procedure in HR-SBO Roaming scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Inter PLMN HO and inter V-SMF change in HR-SBO case :The call flow in 6.7.3.2 needs a complete reshuffling to align with the procedure described in clause 4.23.7.3 (N2 based handover) of TS 23.502 EN on AF triggered EAS Re-discovery Editor's note: AF triggered EAS Re-discovery is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align with the procedure in clause 6.7.3.2 with clause 4.23.7.3 (N2 based handover) of TS 23.502 Ensure AF triggered EAS discovery can work in case of inter PLMN mobility involving HR SBO: the AF needs to know the candidate DNAI(s) in the target serving PLMN, thus needs to know the target serving PLMN; Likewise in case IP replacement is requested , the SMF in the target PLMN needs to know the source and the target EAS IP addresses |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter PLMN HO does not work in HR-SBO case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.1, 6.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.7.3	EAS Re-discovery Procedure

#### 6.7.3.1	General

The EAS re-discovery in VPLMN can be triggered due to UE mobility, AF interacting with HPLMN, or AF interacting with VPLMN.

#### 6.7.3.2	EAS Re-discovery when HR-SBO is possible

Figure 6.7.3.2-1 shows the procedure of EAS re-discovery at inter PLMN mobility when HR-SBO is supported and allowed in the target serving PLMN.

Editor’s Note: the case where HR-SBO is not possible for the target serving PLMN is FFS

Figure 6.7.3.2-1: EAS rediscovery procedure when HR-SBO is possible

0.	the AF subscribes onto H-SMF for the event related with a mobility of a PDU Session towards a serving PLMN where local traffic offload is possible (either towards HPLMN or towards a VPLMN where HR-SBO is possible i.e. supported and allowed)

1.	The procedure described in clause 4.23.7.3 (N2 based handover) of TS 23.502 [3] is performed by replacing I-SMF with V-SMF and SMF with H-SMF.

Editor’s Note: Intra PLMN inter V-SMF change with Xn based HO is FFS

In the case of V-SMF insertion or V-SMF change, (as in step 3 of clause 4.23.7.3.2 of TS 23.502 [3]) the AMF sends Nsmf_PDUSession_CreateSMContext Request to the target V-SMF. In the requestthe AMF includes HR-SBO allowed indication if received from the UDM.

1a.	(V-SMF insertion case) The (target) V-SMF retrieves SM context from H-SMF using Nsmf_PDUSession_Context Request/Response indicating HR-SBO support in the Request.

1b.	(V-SMF change case) The target V-SMF retrieves SM context from the source V-SMF using Nsmf_PDUSession_Context Request/Response indicating HR-SBO support in the Request.

In step 1b, the SM context includes Authorization Result for HR-SBO and VPLMN Specific Offloading Policy if source and target V-SMFs belong to same VPLMN.

The SM context also includes DNAI information to be refreshed for EAS re-discovery. The EAS information corresponds to the old target DNAI if it has been inserted by the H-SMF (V-SMF insertion case) or the source V-SMF (V-SMF change case).

Step 1 to 7 are skipped if the UE moves without V-SMF change.

Steps 1-6 are skipped if the UE moves with V-SMF removal.

The (target) V-SMF may perform UL CL/BP and local PSA insertion/change/removal as described in step 2 of Figure 6.7.2.2-1.

NOTE:	When there are other V-UPF(s) between UL CL/BP and H-UPF, the (target) V-SMF sets up user plane between this ULCL/BP and the existing V-UPF.

2.	The (target) V-SMF may (re-)select a new V-EASDF, e.g. based on the target UE location. The (target) V-SMF may invoke Neasdf_DNSContext_Create service including the DNN, S-NSSAI, HPLMN ID to obtain the new V-EASDF address.

3.	As defined in

step 6 of clause 4.23.7.3.3 of

TS 23.502 [3], The (target) V-SMF invokes Nsmf_PDUSession_Update Request to the H-SMF. The V-SMF indicates HR-SBO support as well as (if an EASDF has been selected in step 2) the selected V-EASDF in the request

In case of intra V-PLMN V-SMF change,

For the case of local PSA removal, the request message is sent to the H-SMF without including any DNAI.

The DNAI information to be refreshed for EAS re-discovery may correspond to:

-	the new target DNAI in case of local PSA insertion.

-	the new target DNAI and old target DNAI in case of local PSA change.

-	the old target DNAI in case of local PSA removal or local PSA is unchanged.

4.	If the UE has changed of serving VPLMN or has moved from HPLMN to a VPLMN the H-SMF retrieves the optional VPLMN Specific Offloading Policy from H-PCF

5.	As defined in step 8 of clause 4.23.7.3.3 of TS 23.502 [3], the H-SMF invokes Nsmf_PDUSession_Update Response, providing the same H-SBO related information than sent by the H-SMF in step 2 of Figure 6.7.2.2-1 and providing the V-EASDF address (optional) in PCO; in case the V-EASDF address has been received from H-SMF the V-SMF sends it to the UE

6.	The hand-over continues as defined in clause 4.23.7.3.3 of TS 23.502 [3],

If the serving PLMN has changed, the DNS context in the old (V-)EASDF is removed by the (source) V-SMF/H-SMF using Neasdf_DNSContext_Delete service.

In the case of V-SMF insertion, the H-SMF removes the DNS context in the EASDF in HPLMN. In case of V-SMF change, the source V-SMF removes the DNS context in the old V-EASDF after the source V-SMF has received Nsmf_PDUSession_UpdateSMContext Request (Handover Complete indication,etc…) as in step 2 of TS 23.502 [3] Figure 4.23.7.3.3-1 .

If the serving PLMN has not changed the new V-SMF is responsible of determining whether and when to remove the context in the old V-EASDF.

7	(in case of change of serving PLMN and step 0 applied) the H-SMF notifies the AF of a change of PDU Session serving PLMN towards a PLMN where local traffic offload is possible providing the new PLMN serving Id (VPLMN ID if mobility to or between VPLMN(s), HPLMN ID if mobility to HPLMN) .

If the AF has an SLA with the target Serving PLMN, the AF subscribes to the list of candidate DNAI(s) in that PLMN. Once the AF has received the candidate DNAI(s), the AF can determine a change of EAS

8.	(V-SMF removal case) The (H-)SMF selects an EASDF in HPLMN and creates a context for the PDU session in that EASDF and then initiates PDU Session Modification command including PCO to the UE.

9.	For AF triggered EAS rediscovery , the AF may indicate the EAS rediscovery for the impacted applications, which are identified by Application Identifier(s), to the NEF of the new serving PLMN using the AF influence on traffic routing procedure as described in clause 4.3.6 of TS 23.502 [3].When IP address replacement is desired the AF provides both the source and the target EAS IP address.

NOTE: in case of serving PLMN change, the new SMF does not know the old EAS IP address and thus needs to receive it

The AF triggered EAS rediscovery may take following form:

9a When the new serving PLMN is the H-PLMN the H-SMF is triggered via the H-PCF

9b	When the new serving PLMN is a V-PLMN, the V-SMF is triggered by the V-NEF.

10a	(When H-SMF has ben invoked in step 9) The H-SMF initiates Nsmf_PDUSession_Update Request towards the (target) V-SMF including the PCO.

The PCO includes EAS rediscovery indication (optional), impact field (optional).

10	(V-SMF insertion or change case) The V-SMF initiates PDU Session Modification command to the UE including the PCO (received in step 10a or determined in step 9b).

.	(V-SMF removal case) The (H-)SMF selects an EASDF in HPLMN and creates a context for the PDU session in that EASDF and then initiates PDU Session Modification command including PCO to the UE.

(in steps 10a and 11) The PCO includes EAS rediscovery indication (optional) and the impact field (optional).

# * * * * End of changes * * * *