SA WG2 Meeting #S2-156-e	S2-2305027

# Elbonia, Apr 17 – 21, 2023, Electronic

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4463 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to 5GS Packet Delay Variation monitoring and reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Due to UE mobility, when NG-RAN handover occurs, an instantaneous packet delay measurement value may appear very large. Due to such a very large packet delay value that occurs momentarily, it may affect the statistical result value of the overall packet delay variation. Therefore, when an event such as handover occurs, corresponding delay packet information should be excluded when calculating the statistical packet delay variation value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a note that large delay values ​​temporarily occurring due to events such as handover may be excluded when calculating packet delay variation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When calculating the packet delay variation value, the mobility of the terminal cannot be considered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of first change >>>

5.37.7	5GS Packet Delay Variation monitoring and reporting

5.37.7.1	General

The 5GS Packet Delay Variation is the variation of packet delay measured between UE and PSA UPF. The AF may send the requirement for Packet Delay Variation monitoring to 5GS together with the requirement for packet delay measurement, as described in clause 6.1.3.21 of TS 23.503 [12]. Based on the QoS monitoring mechanism described in clause 5.33.3, the 5GS Packet Delay Variation is derived by the PCF and then reported to the AF.

Upon AF request for Packet Delay Variation monitoring, the PCF triggers the QoS monitoring procedure, and gets the UL, DL or RT QoS Monitoring result from the SMF. The PCF derives the 5GS Packet Delay Variation based on the QoS Monitoring result and then reports to the AF/NEF both packet delay measurements and Packet Delay Variation.

NOTE:	The derivation of 5GS Packet Delay Variation by PCF, based on QoS Monitoring, is implementation dependent. The Packet Delay Variation calculation method needs to be the same within the PLMN, based on operator policy.  The Packet Delay Variation calculation can be calculated excluding large packet delay values ​​that are temporarily measured due to UE mobility.

Editor's note:	Whether all QoS monitoring needs can be addressed with the limitation of one QoS Monitoring control information per PCC rule is FFS.

<<< End of changes >>>