3GPP TSG-WG SA2 Meeting #156-e	S2-2305035

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4468 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for the description of requested coverage area for time synchronization service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the last meeting, we fully discussed how to ensure the AF provided coverage area for the time synchronization service coinciding with the UE registration area. The reached consensus is to include an ‘Adjust AoI based on RA’ indicator to indicate that the Area of Interest may be adjusted depending on UE’s RA. The indicator is useful when the Time Synchronization Coverage Area requested by the AF includes at least one TA that is a part of UE’s Registration Area (RA). In the case, the 5GS may provide the AF-requested time synchronization service to the UE. This means that all TAs of the RA shall be treated as a Time Synchronization Coverage Area even if some of the TAs were not requested by the AF. This addition of feature ensures that there is no impact on the RA of UE and the UE can continue receiving the time synchronization service when it moves within the RA in the RRC_IDLE. However, if the UE is in the CM-Connected state, the AMF can determine the TA the targeted UE within and provide the AF-requested time synchronization service to the UE. Therefore, the ‘Adjust AoI based on RA’ indicator is applied to the 5G Clock synchronization and it is not necessary in the (g)PTP domain synchronization. Because the targeted UE involved in (g)PTP-based Time Distribution is always in the connected state. Furthermore, in order to save radio resources and ensure the time synchronization service in the AF-requested coverage area, the AMF can configure the access stratum time to the targeted RRC-IDLE UE within the subset TAs of the RA and the requested AoI and CM-Connected UE within the target TA. Also, for determining the presence of the UE in an AoI, the TSCTSF discovers the AMF(s) serving the list of TA(s) that comprise the spatial validity using the NRF and subscribes to the discovered AMF(s) to receive notifications of UE presence in an AoI. Here comes the problem if the AMF(s) do not have the targeted UE context and the subscription fails. The TSCTSF shall discover the UE serving AMF and determine whether the serving AMF is included in the AMF(s) serving the list of TA(s). If the serving AMF is included, the TSCTSF subscribes to the serving AMF to receive notifications of UE presence in an AoI. Based on the discussion above, the CR makes the following changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR implements 5GS providing the AF-requested time synchronization service to the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can’t support for the requested coverage area for time synchronization service . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

5.27.1.10	Support for coverage area filters for time synchronization service

This feature enables the AF to request time synchronization service for a UE or a group of UEs in a specific geographical area (so called coverage area). The requested coverage area contains a list of Tracking Area (TA) or a geographical area (e.g. a civic address or shapes that the NEF transforms to list of TAs based on pre-configuration).

TSCTSF checks with the UDM if the UE is allowed to receive the time synchronization service requested by AF.

The coverage area defines a spatial validity condition for the UE that is resolved at the TSCTSF. In order to do that, the TSCTSF discovers the AMF(s) serving the list of TA(s) that comprise the spatial validity using the NRF and the serving AMF for the UE. If the serving AMF is included in the AMF(s) serving the list of TA(s), the TSCTSF subscribes to the serving AMF to receive notifications about presence of the UE in an Area of Interest events (as described in clause 5.3.4.4 of TS 23.501 [2]). The TSCTSF may include an "Adjust AoI based on RA" indicator in the AoI for the UE involved the 5G Clock synchronization. An Area of Interest (AoI) for each AMF is represented by a list of TA(s), wherein the Area of Interest is identical to the requested coverage area, or the Area of Interest is a TA.

Based on the outcome provided by the AMF about the UE's presence in the AoI, the TSCTSF determines if the time synchronization service is activated or deactivated.

For access stratum distribution activation/deactivation, the TSCTSF will enable/disable access stratum time distribution to the UE at the serving NG-RAN node reusing the procedures in clause 4.15.9.4 of TS 23.502 [3]. For (g)PTP distribution activation/deactivation, the TSCTSF will modify the PTP instance configuration by means of sending a PMIC to the impacted UE/DS-TTs and UMIC to the impacted UPF/NW-TT, as described in clause K.2.2 of TS 23.501 [2].

If the time synchronization service is modified based on the reports of UE presence in the Area of Interest, the TSCTSF informs the AF for the impacted UE(s) by indicating the PTP port state for the related DS-TT PTP port (in the case of (g)PTP based time distribution) or notifying the AF with the indication of 5G access status time distribution status (enabled or disabled, for ASTI based time distribution).

If the Time Synchronization Coverage Area requested by the AF includes at least one TA that is a part of UE’s Registration Area (RA), the 5GS may provide the AF-requested time synchronization service to the targeted UE involved in the 5G Clock synchronization within its RA, i.e. all TAs of the RA shall be treated as a Time Synchronization Coverage Area even if some of the TAs were not requested by the AF, or the subset TAs of the RA and the requested AoI.

NOTE 1:	This ensures that (1) there is no impact on the Registration Area (RA) of UE if/when the AMF receives a time synchronization service request with a spatial validity condition (i.e., for specific geographical area); and (2) the UE can continue receiving the time synchronization service when it moves within the RA in the RRC_IDLE state.

NOTE 2: 	Since the RA can be specific to the UE, the result can be different Time Synchronization Coverage Area for different DS-TT ports of different UEs within a PTP instance.

# * * * * End of the change * * * *