3GPP TSG-WG SA2 Meeting #156-e	S2-2305041

# Elbonia, April 17 – 21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4099 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Providing user location information of MBSR to the AMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is proposed to update TS 23.502 based on the conclusion for KI#6 of TR 23.700-05. The current user location information on NGAP is extended to support the additional user location information of IAB-UE for UEs connected to MBSR. Given that there are quite a few procedures impacted in TS 23.502, it is proposed to use a dedicated clause to document the additional ULI feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Providing user location information of IAB-UE to the AMF serving the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Additional ULI is not supported. Not aligned with RAN WGs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (All text new) * * * *

4.X	Procedures for Mobile Base Station Relay (MBSR)

4.X.1	General

This clause specifies the procedures that enable the support of Mobile Base Station Relay (MBSR), as defined in clause 5.35a of TS 23.501 [2].

4.X.2	Additional User Location Information

As described in clause 5.35A of TS 23.501[2], when an UE is being served by an MBSR, for any N2 messages sent by NG-RAN to AMF, if the User Location Information is included, the N2 parameters shall also include the User Location Information of this MBSR.

# * * * * End of changes * * * *