3GPP TSG-SA2 Meeting #156E(e-meeting)	S2-2305067

# April 17 - 21, 2023, Electronic Meeting										 (revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0354 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new LMF service operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following new service operations are introduced in clause 6 in TS 23.273, - Clause 6.17.4: Nlmf_Location_Measurement Data Request, Nlmf_Location_MeasurementData Response - Clause 6.18.1: Nlmf_Location_UPNotify - Clause 6.18.2 and clause 6.18.3: Nlmf_Location_UPConfig Request, Nlmf_Location_UPConfig Response It is proposed to update LMF services clause 8.3 to add the service operations above. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new LMF service operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of LMF service operations are missing in clause 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1, 8.3.2.x (new), 8.3.2.y (new), 8.3.2.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

8.3.1	General

The following table shows the LMF Services and LMF Service Operations.

Table 8.3.1-1: List of LMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nlmf_Location | DetermineLocation | Request/Response | AMF |
|  | EventNotify | Notify | GMLC |
|  | CancelLocation | Request/Response | AMF |
|  | LocationContextTransfer | Request/Response | LMF |
|  | MeasurementData | Request/Response | LMF |
|  | UPConfig | Request/Response | AMF |
|  | UPNotify | Request/Response | AMF |
| Nlmf_Broadcast | CipheringKeyData | Notify | AMF |

* * * Second Change * * *

8.3.2.x	Nlmf_Location_MeasurementData service operation

Service operation name: Nlmf_Location_MeasurementData

Description: Provides PRU location measurements to the consumer NF.

Input, Required: Target UE cell ID, indication of location measurements requested from PRU.

Input, Optional: PRU identifier(s).

Output, Required: PRU location measurements

Output, Optional: None.

See clause 6.17 for an example of usage of this service operation.

* * * Third Change * * *

8.3.2.y	Nlmf_Location_UPConfig service operation

Service operation name: Nlmf_Location_UPConfig

Description: The consumer NF requests to set up, modify or terminate an LCS-UP connection.

Input, Required: UE identifier (GPSI, SUPI).

Input, Optional: AMF reallocation indication, user plane connection termination indication.

Output, Required: Success/Failure indication

Output, Optional: Failure Cause (in the case of failure indication provided).

See clause 6.18.2 and clause 6.18.3 for an example of usage of this service operation.

* * * Fourth Change * * *

8.3.2.z	Nlmf_Location_UPNotify service operation

Service operation name: Nlmf_Location_UPNotify

Description: Notify the consumer NF that the user plane connection between UE and LMF has been established.

Input, Required: LCS Correlation Identifier, indication that the user plane connection has been established.

Input, Optional: None.

Output, Required: None.

Output, Optional: None.

See clause 6.18.1 for an example of usage of this service operation.

* * * End of Changes * * *