3GPP TSG-SA2 Meeting #156E(e-meeting)	S2-2305068

# April 17 - 21, 2023, Electronic Meeting										 (revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0355 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the Architecture of Location Service for interconnection between 5GC and EPC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The location service continuity has been discussed during Rel-16. In Rel-16, it was agreed: - The architecture of location service for interconnection between 5GC and EPC in clause 4.2a.2. - Some principles in clause 4.2a.1: - An EPC/5GC common interface is used between the LCS Client and the 5GC GMLC to enable the location service request being handled based on whether the target UE is served by EPC or 5GC. The AF initiates the service request to the 5GC GMLC via NEF. - EPC GMLC and 5GC GMLC can be collocated in implementation, in such case, Lr' is not needed. - Lr' is not standardized. Based on the background above and the progress on location service continuity in Rel-18, it is proposed that: - Update the architecture in clause 4.2a.2 to add N26 interface between AMF and MME. Because the location service continuity procedures in clause 6.19.1 and in cluse 6.19.2 added in Rel-18 require the architecture supports the N26 interface. - To simplify the interaction between EPC-GMLC and 5GC-GMLC, it is proposed to reuse the principles in clause 4.2a.1, i.e. the operator network exposes one interface (5GC-GMLC) to LCS Client/AF and the Lr’ interface is not standardized. Based on this proposal, the procedure in clause 6.19.2 is updated to align with the principle. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the architecture of location service for interconnection between 5GC and EPC based on progress in Rel-18 and update the procedure in clause 6.19.2 to re-use the Rel-16 principles. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2a.2, 6.19.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

4.2a.2	Non-roaming architecture

Figure 4.2a.2-1 represents the non-roaming architecture of Location Services for interconnection between 5GC and EPC.

Figure 4.2a.2-1: Non-roaming architecture of interconnection between 5GC and EPC

NOTE 1:	EPC GMLC and 5GC GMLC can be collocated in implementation, in such case, Lr' is not needed.

NOTE 2:	For this release, Lr' is not standardized.

NOTE x:	Support of N26 interface in the network is optional for interworking.

* * * Next Change * * *

6.19.2	Location Service Continuity from EPS to 5GS for Immediate Location Request

Figure 6.19.2-1: Location Service Continuity from EPS to 5GS

0a.	Steps 1-3 in 5GC-MT-LR procedure for the commercial location services in Figure 6.1.2-1 are perforemed.

0b.	Based on the network addresses of the current serving AMF and additionally the address of EPC-GMLC received from UDM, the 5GC-GMLC sends the location request to the EPC-GMLC.

1.	Target UE positioning is started in EPS, e.g. Steps 1-7 of EPC-MT-LR procedure in Figure 9.18 or steps 1-4 of EPC-MO-LR procedure in Figure 9.8d of TS 23.271 [4].

2.	During the EPC-MT-LR or EPC-MO-LR, EPS to 5GS handover happens successfully, as described in steps 1-14 in figure 4.11.1.2.2.2-1 of TS 23.502 [19] - MME receives Forward Relocation Complete Notification from the AMF and stored the AMF ID.

3.	MME sends Subscriber Location Report to GMLC with Handover event type, UE identifies and AMF ID as target node information, EPC-GMLC identifies the handover by event type and target AMF.

4.	the LCS session is cancelled as described in clause 9.4.3a of TS 23.271 [4].

5.	The EPC-GMLC sends the location response to the 5GC-GMLC including handover event type and the target AMF ID.

6.	Steps 4-24 of 5GC-MT-LR procedure in clause 6.1.2 or Steps 1-13 of 5GC-MO-LR procedure in clause 6.2 are performed.

* * * End of Changes * * *