3GPP TSG-SA2 Meeting #156E(e-meeting)	S2-2305076

# April 17 - 21, 2023, Electronic Meeting										 (revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0363 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the positioning procedure of a target UE involving MBSR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Three changes are proposed in this CR For the first change: Based on the procedure in clause 6.1.4, when the positioning procedure is for MBSR which is not acting as the target UE, the MBSR location is obtained via the 5GC-MO-LR procedure or LMF triggered UE positioning procedure which is proposed by the third change in this CR. Because the privacy check does not apply to the 5GC-MO-LR procedure and LMF triggered UE positioning procedure, the privacy check will not performed in this case. Thus there is no need to enhance the privacy check mechanism. The clause 5.9.3 is removed. For the second and the third changes: The positioning procedure involving MBSR can be applied to 5GC-MO-LR, 5GC-MT-LR, 5GC-NI-LR and Deferred 5GC-MT-LR procedures, but the current description and the procedure can only apply to 5GC-MT-LR procedure. The procedure in clause 6.1.4 is removed and a new clause (6.x) is added. The procedure in clause 6.x is based on the procedure in clause 6.1.4, the main difference is that it is more general and can be applied to the proceudres above, but not just to 5GC-MT-LR. Furthermore, based on the conclusion in Rel-18 5G_eLCS_Ph3 WID, the LMF supports the functionality of correlation ID allocation, which is already introduced in step 5a in clause 6.17.1 in TS 23.273. So the LMF can initiate the UE positioning procedure directly without requesting GMLC. The pre-condition is that the LMF knows the UE ID and serving cell ID. So the procedure for option#2 is simplified to re-use this functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the positioning procedure of target UE involving MBSR and remove the privacy check enhancements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The usage of MBSR is limited. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.3, 6.1.4, 6.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

* * * Second Change * * *

* * * Third Change * * *

6.x	Positioning of a Target UE involving Mobile Base Station Relay

Figure 6.x-1 illustrates the network positioning for the target UE when Mobile Base Station Relay(s) is involved.

In this scenario, it is assumed that the target UE may be identified using an SUPI or GPSI. The procedure follows the functionality in clause 5.9. It is further assumed that:

-	The NG-RAN in figure 6.x-1 is a donor-CU to the MBSR.

-	When the MBSR is integrated or fully migrated to a new gNB, the OAM triggers the LMF to perform TRP Information Exchange procedure. The LMF learns that a new integrated TRP at a gNB is mobile and its MBSR IAB UE ID (GPSI) via a TRP information exchange towards the gNB with the Cell ID of the TRP. The LMF may also performs NRPPa TRP information exchange procedure before the positioning procedure for a target UE, e.g. to determine the position capability of NG-RAN.

-	The LMF that performs the location estimation of the MBSR can be different than the LMF that performs the location estimation of the target UE (not shown in figure 6.1.4-1).

Figure 6.x-1: Positioning of a Target UE involving Mobile Base Station Relay

1.	The serving LMF for the target UE receives a location request from the serving AMF for the target UE. The location request may be included in an Nlmf_Location_DetermineLocation Request service operation for a 5GC-MO-LR, 5GC-MT-LR or 5GC-NI-LR for the target UE. Alternatively, the location request may be implied by receipt of an Namf_Communication_N1MessageNotify service operation carrying a supplementary services event report from the target UE for a periodic or triggered 5GC-MT-LR. If the AMF is aware that the target UE is served by a MBSR, it would select the LMF that can support the MBSR handling. If the AMF is aware that the target UE is served by a MBSR, it would select the LMF that can support the MBSR handling.

2.	LMF derives if any MBSR(s) is involved in the positioning of the target UE based on the cell-ID used for positioning measurements in the step 1. The AMF serving the target UE may indicate that the serving cell is an MBSR (if applicable). When the MBSR was integrated as a TRP (IAB-DU) with a gNB, the LMF may determine from in a TRP information exchange procedure that the cell-ID belongs to the MBSR and/or the UE-ID (GPSI) associated with MBSR. As the MBSR can be mobile the LMF may need to determine an updated location of the MBSR by either performing step 3-5 (option 1) or performing step 6 (option 2) if option 1 is not feasible. If several MBSRs were derived, then step 3-5 or step 6 are be performed for each MBSR.

Editor's note:	The trigger of LMF to initiate the TRP information exchange needs further discussion for the case of MBSR's inter-CU mobility.

Editor's note:	Allowing the LMF to obtain the UE ID of the MBSR via NRPPa from the donor gNB needs to be coordinated with RAN WGs.

3.	[Conditional] The LMF initiates a NRPPa TRP Information Exchange procedure to request the updated location information of the TRP(s) associated with MBSR(s) by directing the TRP information exchange messages to the gNB associated with the Cell ID(s), for example, by setting the NRPPa TRP Information Type Item IE to "geo-coordinates". Donor-CU further send a F1 TRP INFORMATION REQUEST message to any MBSR IAB-DU.

4.	[Conditional] The MBSR IAB-UE initiates a 5GC-MO-LR procedure (as defined in clause 6.2) to get its location information. The MBSR IAB-UE's location info is used by the co-located MBSR IAB-DU to determine the updated location for its TRP.

5.	[Conditional] The MBSR IAB-DU report its updated TRP location, e.g. TRP's geo-coordinate, velocity and the time for obtaining them, to the Donor-CU, which is further forwarded to LMF.

6.	[Conditional] The LMF uses the procedure defined in clause 6.11 to obtain location information of the MBSR. The LMF received the GPSI of the MBSR in a TRP information exchange when the MBSR was integrated as a TRP.

7.	The LMF performs one of the positioning procedures with the target UE described in clauses 6.11.1 and 6.11.2. To reduce the timing offset of the positioning measurements, the UE positioning procedure may be scheduled with the same scheduled location time as the MBSR positioning in step 4 or 6. If Network Assisted procedure is used, as in 6.11.2, the NG-RAN may provide the MBSR updated location and velocity information to the LMF as part of the NRPPa procedure.

8.	The LMF uses the received location and velocity of the MBSR(s) when estimating the location of the target UE together with the measurements reports in step 7.

9a.	If an Nlmf_Location_DetermineLocation Request service operation for a 5GC-MO-LR, 5GC-MT-LR or 5GC-NI-LR was received at step 1, the serving LMF returns the location estimate of the target UE to the serving AMF.

9b.	If an Namf_Communication_N1MessageNotify service operation carrying a supplementary services event report from the target UE for a periodic or triggered 5GC-MT-LR was received at step 1, the serving LMF sends an event report for the target UE to a GMLC with the location estimate obtained at step 8 as described in clause 6.3.1.

* * * End of Changes * * *