3GPP TSG-WG SA2 Meeting #156-e	S2-2305082

# Albonia, April 17 – 21, 2023		(revision of S2-2302925)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3930 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Hierarchical NSACF architecture procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, NEC, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | eNS-Ph3(TR23.700-41) concluded the option of hierarchical NSACF architecture to support multiple service area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add the service operation of NSAC for hierarchical NSACF architecture option. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC cannot support the service operation of NSAC for hierarchical NSACF architecture option. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.11.1, 4.2.11.2, 4.2.11.2a (new), 4.2.11.4, 4.2.11.4a (new), 4.15.3.2.10, 4.15.3.2.10a (new),5.2.21.1, 5.2.21.2.2, 5.2.21.2.4, 5.2.21.2.w (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS23.501 CR3959 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.2.11	Network Slice Admission Control Function (NSACF) procedures

#### 4.2.11.1	General

The Network Slice Admission Control Function procedures are performed for an S-NSSAI which is subject to Network Slice Admission Control (NSAC) as described in TS 23.501 [2]. If charging needs to be enabled, the NSACF may act as a NF (CTF) and interact with the CHF to support the Event based charging as defined in TS 32.290 [42].

Depending on the NSAC architecture deployed in the network, three options of NSAC procedures are defined:

- Option 1: The NSAC procedure for number of UEs or PDU sessions for an S-NSSAI is based on non-Hierarchal NSAC architecture. The corresponding procedures are described in clause 4.2.11.2 and clause 4.2.11.4 respectively.

- Option 2: The NSAC procedure for number of UEs or PDU sessions for an S-NSSAI is based on centralized NSAC architecture. The corresponding procedures are described in clause 4.2.11.x and clause 4.2.11.y respectively.

- Option 3: The NSAC procedure for number of UEs or PDU sessions for an S-NSSAI is based on hierarchical NSAC architecture. The corresponding procedures are described in clause 4.2.11.2a and clause 4.2.11.4a respectively.

#### 4.2.11.2	Number of UEs per network slice availability check and update procedure

This clause describes the case when the non-Hierarchal NSACF architecture is deployed in the network.

The number of UEs per network slice availability check and update procedure is to update (i.e. increase or decrease) the number of UEs registered with an S-NSSAI which is subject to NSAC. The AMF is configured with the information indicating which network slice is subject to NSAC.

Figure 4.2.11.2-1: Number of UEs per network slice availability check and update procedure

1.	If the AMF is not aware of which NSACF to communicate, the AMF performs NSACF discovery as described in clause 6.3.22 of TS 23.501 [2] and in clause 5.2.7.3.2. The AMF triggers the Number of UEs per network slice availability check and update procedure to update the number of UEs registered with a network slice when a network slice subject to NSAC is included in the Allowed NSSAI (i.e. the AMF requests to register the UE with the S-NSSAI) or removed from the Allowed NSSAI (i.e. the AMF requests to de-register the UE from the S-NSSAI) for a UE. The trigger event at the AMF also includes the change of Allowed NSSAI in case of inter-AMF mobility. The procedure is triggered in the following cases:

-	At UE Registration procedure, according to clause 4.2.2.2.2 (including Registration types of Initial Registration or Mobility Registration Update in inter-AMF mobility in CM-CONNECTED or CM-IDLE state):

-	before the Registration Accept in step 21 if the EAC mode is active; or

-	after the Registration Accept message if the EAC mode is not active;

-	At UE Deregistration procedure, as per clause 4.2.2.3, after the Deregistration procedure is completed;

-	At UE Configuration Update procedure (which may result from NSSAA procedure or subscribed S-NSSAI change):

-	before the UE Configuration Update message if the EAC mode is active and the update flag is to increase; or

-	after the UE Configuration Update message if the EAC mode is active and the update flag is to decrease; or

-	after the UE Configuration Update message if the EAC mode is not active.

NOTE 1:	Depending on the deployment, there may be different NSACF for different S-NSSAI subject to NSAC and hence, during the registration, AMF triggers the Number of UEs per network slice availability check and update procedure to multiple NSACFs.

2.	The AMF sends Nnsacf_NSAC_NumOfUEsUpdate_Request message to the NSACF. The AMF includes in the message the UE ID, Access Type to which the Allowed NSSAI is applied, the S-NSSAI(s), the NF ID and the update flag which indicates whether the number of UEs registered with the S-NSSAI(s) is to be increased when the UE has gained registration to network slice(s) subject to NSAC or the number of UEs registered with the S-NSSAI(s) is to be decreased when the UE has deregistered from S-NSSAI(s) or could not renew its registration to an S-NSSAI subject to NSAC.

If this is the first time to perform NSAC procedure for the S-NSSAI towards the NSACF, the AMF includes notification endpoint for EAC Notification to implicitly subscribe the EAC notification for the S-NSSAI from the NSACF.

3.	The NSACF determines whether the Access Type provided by the AMF is configured for the NSAC based on its configuration. If the Access Type is not configured for the NSAC, the NSACF always accepts the request from the AMF without increasing or decreasing the number of UEs. If the Access Type is configured for the NSAC, the NSACF updates the current number of UEs registered for the S-NSSAI, i.e. increases or decrease the number of UEs registered per network slice based on the information provided by the AMF in the update flag parameter.

If the update flag parameter from the AMF indicates increase, the following applies:

-	If the UE ID is already in the list of UEs registered with the network slice, the current number of UEs is not increased as the UE has already been counted as registered with the network slice. The NSACF creates a new entry associated with this new update and shall also maintain the old entry associated with previous update. The multiple entries for the same UE ID in the NSACF are differentiated based on the NF ID of the NF sending the update request. The NSACF removes the entry associated with the NF ID upon reception of a request having update flag indicating decrease.

NOTE 2:	The use case of having two or more entries in the NSACF for the same UE can happen during (a) inter-AMF mobility when the new AMF request update to the NSACF before the old AMF sends request to deregister the UE; or (b) PDN connections establishment in the EPC when multiple SMF +PGW-Cs (i.e. used for different PDN Connections associated with the same S-NSSAI) send update requests for maximum number of UEs to the NSACF.

NOTE 3:	To handle AMF graceful removal, the NSACF can subscribe for unavailability notifications with the AMF (directly or via NRF) as described in clause 5.21.2.2 and act accordingly, e.g. update the NF ID with the target AMF ID.

-	If the UE ID is not in the list of UE IDs registered with the network slice and the maximum number of UEs registered with the network slice has not been reached yet, the NSACF adds the UE ID in the list of UEs registered with the network slice as a new entry associated with this new update and increases the current number of the UEs registered with the network slice. If the UE ID is not in the list of UEs registered with that S-NSSAI and the maximum number of UEs for that S-NSSAI has already been reached, then the NSACF returns a result parameter indicating that the maximum number of UEs registered with the network slice has been reached.

If the update flag parameter from the AMF indicates decrease and if there is only one entry associated with the UE ID, the NSACF removes the UE ID from the list of UEs registered with the network slice for each of the S-NSSAI(s) indicated in the request from the AMF and also the NSACF decreases the number of UEs per network slice that is maintained by the NSACF for each of these network slices. If there are multiple entries associated with the UE ID, the NSACF removes the entry associated with the NF ID but the UE ID is kept in the list of UEs registered with the S-NSSAI.

The NSACF takes access type into account for increasing and decreasing the number of UEs per network slice as described in clause 5.15.11.1 of TS 23.501 [2].

The NSACF stores the notification endpoint for EAC Notification associated with the S-NSSAI if it is received from the AMF. The NSACF can use this AMF notification endpoint to update the EAC mode as described in clause 4.2.11.3.

NOTE 4:	This enables the NSACF to maintain up-to-date information about the AMFs serving the S-NSSAIs.

4.	The NSACF returns the Nnsacf_NSAC_NumOfUEsUpdate_Response message including Result indication per S-NSSAI. The Result indication includes either 'maximum number of UEs registered with the network slice reached' or 'maximum number of UEs registered with the network slice not reached'.

At UE Registration procedure, if only some of the S-NSSAIs reached the maximum number of UEs per S-NSSAI, the AMF sends a Registration Accept message to the UE in which the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI list for which the NSACF has indicated that the maximum number of UEs per network slice has been reached and for each rejected S-NSSAI the AMF includes a reject cause set to 'maximum number of UEs per network slice reached' and optionally a back-off timer.

When for all the Requested S-NSSAI(s) provided in step 2 the NSACF returned the maximum number of UEs per network slice has been reached and if one or more subscribed S-NSSAIs are marked as default in the subscription data and not subject to NSAC, the AMF can decide to include these Default Subscribed S-NSSAIs in the Allowed NSSAI. Otherwise, the AMF rejects the UE request for registration. In the Registration Reject message, the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI parameter and for each rejected S-NSSAI the AMF includes a reject cause to indicate that the maximum number of UEs per network slice has been reached and optionally a back-off timer.

NOTE 5:	If the use case requires the UE to remain reachable at all times with at least one slice, it is recommended that at least one of the Subscribed S-NSSAIs is marked as the default S-NSSAI which is not subject to NSAC. This will ensure the UE is able to access to services even when maximum number of UEs per network slice has been reached.

#### 4.2.11.2a	Hierarchical NSACF-based Number of UEs per network slice availability check and update procedure

Figure 4.2.11.2a-1: Hierarchical NSACF-based Number of UEs per network slice availability check and update procedure

For an S-NSSAIs subject to counting of the number of registered UEs, if hierarchical NSACF architecture is deployed in the network the enforcement of maximum number of UEs registered for an S-NSSAI is performed as follows:

1.	Same as the steps 1 defined in clause 4.2.11.2.

2.	In addition to the information included in the Nnsacf_NSAC_NumOfUEsUpdate_Request as described in the step 2 of clause 4.2.11.2, the AMF  may provide UE already registered indication  to the NSACF.

3.	The NSACF performs NSAC for the indicated S-NSSAI.

If the update flag parameter from the AMF indicates increase, the following applies:

- For NSACF which support UE admission quota based control:

-	If the local maximum number of UEs is not reached yet, the NSACF executes the same action as the step 3 defined in clause 4.2.11.2. Steps 4-8 are skipped.

-	If the local maximum number of UEs is reached, the NSACF sends a delegation request of NSAC to the Primary NSACF. Steps 4-8 are executed.

- For NSACF which support UE admission threshold based control:

-	If the UE admission is below the threshold level, the NSACF executes the same action as the step 3 defined in clause 4.2.11.2. Steps 4-8 are skipped.

-	If the UE admission is at or above the threshold level and the local maximum number has not been reached, the NSACF checks whether the UE already registered indication is present.

- If the UE already registered indication is not present then  the NSACF immediately rejects the NSAC request. Steps 4-8 are skipped.

- If the UE already registered indication is present the NSACF executes the same action as the step 3 defined in clause 4.2.11.2 in order to allow for service continuity. Steps 4-8 are skipped.

-	If the local maximum number has been reached and the UE already registered indication is present then the NSACF sends a delegation request of NSAC to the Primary NSACF requesting for quota increase in order to allow for service continuity. Steps 4-8 are executed.

If the update flag parameter from the AMF indicates decrease, the following applies:

- If the UE entry to be deleted is stored at the NSACF, the NSACF executes the same action as the step 3 defined in clause 4.2.11.2. Steps 4-8 are skipped.

- If the UE entry to be deleted is not stored at the NSACF, the NSACF sends a delegation request of NSAC to the Primary NSACF. Steps 4-8 are executed.

4.	If the Primary NSACF has not been discovered before, the NSACF discovers and selects the Primary NSACF which manages the global service area per clause 6.3.22 of TS23.501 [2].

5.	The NSACF invokes Nnsacf_NSAC_NumOfUEsUpdate_Request to the Primary NSACF.

The request includes the NSAC request information received from AMF. In addition it may include the current local maximum number of UEs or the current UE admission threshold respectively, which is the last value received from the Primary NSACF.

6.	The Primary NSACF performs NSAC for the indicated S-NSSAI.

If the update flag parameter from the NSACF indicates increase, the following applies:

- The Primary NSACF creates a new entry associated with the UE ID within the received NSAC request due to the remaining global maximum number is only managed by Primary NSACF and not distributed to the NSACF(s) and executes the same action as step 3 in clause 4.2.11.2 is executed with the replacement of NSACF with Primary NSACF. Alternatively, the updated local maximum number of the UEs can be send to the NSACF by the Primary NSACF, for the admission control locally by the NSACF.

NOTE:	To support the service continuity across different service area, the Primary NSACF always keep part of the global maximum number, i.e. the whole global maximum number is not distributed to all contacted NSACF(s).

If the update flag parameter from the NSACF indicates decrease and the UE entry is managed by the Primary NSACF, the same action as step 3 in clause 4.2.11.2 is executed with the replacement of NSACF with Primary NSACF.

7.	The Primary NSACF returns the Nnsacf_NSAC_NumOfUEsUpdate_Response message to the NSACF. The response may include the indication (e.g., rejection) of NSAC delegation request for the S-NSSAI.

If the Primary NSACF determines to adjust the configured value stored at the NSACF, the updated local maximum number of UEs or UE admission threshold is also included in the response respectively.

8.	The NSACF checks the response from Primary NSACF.

If the response includes the updated configured value,

- The NSACF, which supports UE admission quota based control, replaces the existing local maximum number of UEs with the received updated value. The same action is executed per step 3 in clause 4.2.11.2 based on the updated configured value.

- The NSACF, which supports UE admission threshold based control, replaces the existing UE admission threshold and/or the maximum number of Registered UEs with the received updated value. The same action is executed per step 3 in clause 4.2.11.2 based on the updated configured value.

If the response does not include the updated configured value, the NSACF returns the response to AMF based on the received NSAC response from Primary NSACF.

9.	Same as the step 4 defined in clause 4.2.11.2.

# * * * * Next change * * * *

#### 4.2.11.4	Number of PDU Sessions per network slice availability check and update procedure

This clause describes the case when the non-Hierarchal NSACF architecture is deployed in the network.

The number of PDU Sessions per network slice availability check and update procedure is to update (i.e. increase or decrease) the number of PDU Sessions established on S-NSSAI which is subject to NSAC. The SMF is configured with the information indicating which network slice is subject to NSAC.

NOTE 1:	EAC mode is not applicable for Number of PDU Sessions per network slice availability check and update procedure.

Figure 4.2.11.4-1: Number of PDU Sessions per network slice availability check and update procedure

1.	If the SMF is not aware of which NSACF to communicate, the SMF performs NSACF discovery as described in clause 6.3.22 of TS 23.501 [2] and in clause 5.2.7.3.2. The SMF anchoring the PDU session triggers the Number of PDU Sessions per network slice availability check and update procedure for the network slices that are subject to NSAC at the beginning of a PDU Session Establishment procedure (clause 4.3.2.2.1 and clause 4.3.2.2.2) only for new PDU Sessions to be established and as a last step of successful PDU Session Release procedure (clause 4.3.4.2 and clause 4.3.4.3).

NOTE 2:	SMFs handling PDU sessions associated with UE Request Type "Existing PDU Session" for intra access handover purposes do not interact with the NSACF.

2.	The SMF anchoring the PDU session sends Nnsacf_NSAC_NumOfPDUsUpdate_Request message to the NSACF. The SMF includes in the message the UE-ID, the PDU session ID, S-NSSAI for which the number of PDU Sessions per network slice update is required, Access Type and the update flag. The update flag may include one of the following values:

-	'increase' which indicates that the number of PDUs established on the S-NSSAI is to be increased when the procedure is triggered at the beginning of PDU Session Establishment procedure or when a new user plane leg is to be established for an MA PDU Session;

-	'decrease' which indicates that the number of PDU Sessions on the S-NSSAI is to be decreased when the procedure is triggered at the end of PDU Sessions Release procedure or when an existing user plane leg is to be released for an MA PDU Session; or

-	'update' which indicates that for existing PDU Session the Access Type is to be replaced with a new Access Type during inter access mobility.

NOTE 3:	For SSC mode 3 PDU session, the SMF of the new PDU Session invokes the NSACF to increase the number of PDU Session and adds the new PDU session ID in the NSACF. When the old PDU session is released the SMF of the old PDU session invokes the NSACF to decrease the number of PDU Session and remove the old PDU session ID in the NSACF.

NOTE 4:	An SMF anchoring an IPv6 Multi-homed PDU session does not invoke NSACF for an S-NSSAI subject to NSAC when the PDU session replaces an existing anchor according to clause 4.3.5.3.

3.	The NSACF updates the current number of PDU Sessions established on the S-NSSAI, i.e. increase or decrease the number of PDU Sessions per network slice based on the information provided by the anchor SMF in the update flag parameter.

If the update flag parameter from the SMF anchoring the PDU session indicates increase value and the maximum number of PDU Sessions established on the S-NSSAI has already been reached, then the NSACF returns a result parameter indicating that the maximum number of PDU Sessions per network slice has been reached. If the maximum number of PDU Sessions established on the S-NSSAI has not been reached, the NSACF checks the UE ID. If the UE ID is located, the NSACF, stores the PDU Session ID and the Access Type and increases the number of PDU Sessions for that S-NSSAI. If the NSACF did not locate the UE ID, it creates an entry for the UE ID, stores the PDU Session ID and Access Type and increases the number of PDU Sessions for that S-NSSAI.

If the update flag parameter from the SMF anchoring the PDU session indicates decrease value, the current number of PDU Sessions per S-NSSAI, the NSACF locates the UE ID and decreases the number of PDU Sessions for that S-NSSAI and its related PDU Session ID. If the UE ID has no more PDU sessions, after the decrease, the NSACF removes the UE ID entry.

If the update flag parameter from the SMF anchoring the PDU session indicates update value, the NSACF locates the existing entry with UE ID and PDU Session ID and replaces the Access Type in the existing entry.

The NSACF takes the Access Type parameter into account for increasing and decreasing the number of PDU Sessions per S-NSSAI as described in clause 5.15.11.2 of TS 23.501 [2]. For MA PDU Session, if the SMF received information that the UE is registered over both accesses, the SMF provides multiple Access Types to the NSACF. If the NSACF receives a request containing multiple Access Types, the NSACF provides a Result indication for each Access Type.

4.	The NSACF acknowledges the update to the anchor SMF with Nnsacf_NSAC_NumOfPDUsUpdate_Response message including a Result indication. If the NSACF returns a Result indication including 'maximum number of PDU Sessions per S-NSSAI reached', the SMF rejects the PDU Session establishment request with reject cause set to 'maximum number of PDU Sessions per S-NSSAI reached' and optionally a back-off timer and the Access Type.

For MA PDU Session, the NSACF may accept the MA PDU Session and may provide to the SMF a Result indicating 'maximum number of PDU Sessions per S-NSSAI reached' or 'maximum number of PDU Sessions per S-NSSAI not reached' associated with an Access Type. If the NSACF indicates a failure that is associated with the Access Type over which the UE sent the MA PDU Session Establishment Request, the SMF sends to the UE a PDU Session Establishment Reject with a Result indication including 'maximum number of PDU Sessions per S-NSSAI reached' ,optionally a back-off timer and the Access Type. When the SMF rejects the MA PDU Session, the SMF sets the Access Type parameter as follows:

-	If the UE is registered via both accesses and:

-	If the NSACF indicates failure for both accesses, the Access Type indicates both accesses;

-	If the NSACF indicates failure for the access over which the MA PDU Session Establishment Request is received, the Access Type indicates the access over which the MA PDU Session Request is received.

NOTE 5:	If the UE is registered in both accesses and the NSACF indicates failure for the access different from the access over which the MA PDU Session Establishment Request is received, the SMF accepts the MA PDU Session Request and does not provide back-off timer to the UE.

-	If the UE is registered via a single access, the Access Type indicates the access over which the MA PDU Session Request is received.

If the UE receives back-off timer, the UE shall not request the establishment of user-plane resources on the restricted Access Type until the back-off timer expires. The UE may request a PDU Session via the Access Type which is not restricted.

In the case of a PDU Session Establishment failure, the anchor SMF triggers another request to the NSACF with the update flag parameter equal to decrease in order to re-adjust back the PDU Session counter in the NSACF.

#### 4.2.11.4a	Hierarchical NSACF-based Number of PDU Sessions per network slice availability check and update procedure

Figure 4.2.11.4a-1: Hierarchical NSAC-based number of PDU Sessions per network slice availability check and update procedure

For an S-NSSAI subject to counting of the number of PDU sessions, if hierarchical NSACF architecture is deployed in the network the enforcement of maximum number of PDU Session established for an S-NSSAI is performed as follow:

1-2. Same as the steps 1-2 defined in clause 4.2.11.4.

3.	The NSACF performs NSAC for the indicated S-NSSAI.

If the PDU session ID entry update at the NSACF is possible, e.g. create a new entry associated with the received NSAC request for increase case the same action as step 3 defined in clause 4.2.11.4 is executed. Steps 4-8 are skipped.

If the PDU session ID entry at the NSACF is impossible, i.e. by admitting the PDU session the local maximum PDU session number is exceeded, the NSACF applies an updated local maximum PDU session number from the Primary NSACF.

4.	If the Primary NSACF has not been discovered before, the NSACF discovers and selects the Primary NSACF, which manages the global service area as per clause 6.3.22 of TS23.501 [2].

5.	The NSACF invokes  Nnsacf_NSAC_NumOfPDUsUpdate request to the Primary NSACF. The message includes the S-NSSAI and the current local maximum number of PDU Sessions.

6.	The Primary NSACF checks the global maximum PDU session number and determines whether accept or reject the requested the updated local maximum PDU session number from NSACF, i.e. whether the update of the local maximum PDU session number of NSACF is accepted or not.

7.	The Primary NSACF returns the  Nnsacf_NSAC_NumOfPDUsUpdate response. The response includes a new allocated local maximum PDU sessions number or the indication to reject the request of updated local maximum PDU session number.

8.	If the primary NSACF provides an updated local maximum number, the NSACF replaces the local maximum PDU session number with the received local maximum PDU sessions number value. The same action is executed per step 3 in clause 4.2.11.4 based on the updated configured value.

9.	 Same as the step 4 of clause 4.2.11.4.

# * * * * Next change * * * *

##### 4.15.3.2.10	Number of UEs and PDU Sessions per network slice notification procedure

This clause describes the case when the non-Hierarchal tier NSACF architecture is deployed in the network.

This procedure depicts the case of an AF subscribing to receive the registered number of UEs, or the number of PDU sessions in a specific S-NSSAI. The procedure handles the case when there is a single NSACF in the PLMN responsible for the S-NSSAI (single Service Area) or when there are multiple NSACFs responsible for the S-NSSAI in the PLMN (multiple Service Areas).

Figure 4.15.3.2.10-1: Number of UEs and PDU Sessions per network slice notification procedure

1.	To subscribe or unsubscribe for the number of UEs or the number of PDU Sessions per network slice notification with the NEF, the AF sends Nnef_EventExposure Subscribe/Unsubscribe Request (Event ID, Event Filter, Event Reporting information, S-NSSAI) message to the NEF. The Event ID parameter defines the subscribed event ID, i.e. Number of Registered UEs or Number of Established PDU Sessions. The Event Filter parameter defines the S-NSSAI, in case of a trusted AF or AF-Service-Identifier as defined in TS 29.522 for an untrusted AF, for which reporting is required. The Event Reporting information parameter defines the mode of reporting, which includes threshold reporting with a threshold value or periodic reporting with included periodicity time interval. The S-NSSAI is the slice for which the subscription is requested

The AF may request one-time reporting or immediate reporting.

NOTE 1:	When immediate reporting but not for one-time reporting is requested, the subscription is maintained after returning the report to the AF. When one-time reporting is requested, the subscription is terminated right after returning the report to the AF.

Notifications related to the threshold based subscriptions behave as follows:

-	A single notification is sent only when the number of registered UEs or the number of established PDU Sessions reaches the threshold. A single notification is sent every time there is a change from being below the threshold to reach the threshold.

-	A single notification is sent only once when the number of registered UEs or the number of established PDU Sessions go below the threshold after reaching it. A single notification is sent every time there is a change from reaching the threshold to coming down below the threshold.

2.	The NEF confirms with Nnef_EventExposure_Subscribe/Unsubscribe Response message to the AF. This message may include the event reporting, if available in the NEF and immediate reporting or one-time reporting was requested by the AF. In the case of Untrusted AF, the NEF includes the AF-Service-Identifier corresponding to the S-NSSAI in the returned notification.

If immediate reporting or one-time reporting is requested, step 2 occurs after step 5 and the subscription response contains the immediate or one-time report. For the case of one-time reporting, no subscription is created at the NEF/NSACF.

3.	The NEF may query the NRF to find the NSACF(s) responsible for the requested S-NSSAI. If needed, the NEF translates the AF-Service-Identifier to the corresponding S-NSSAI prior to performing the query.

4.	If the NEF has not already subscribed to the event from the NSACF for the requested S-NSSAI, the NEF initiates the request Nnsacf_SliceEventExposure_Subscribe/Unsubscribe Request (Event ID, Event Filter, Event Reporting information, immediate reporting, S-NSSAI) to all the NSACFs supporting the requested S-NSSAI. The NEF stores the AF requested Event Reporting Information. If multiple NSACFs are selected for the requested S-NSSAI, the NEF may set the Event Reporting Information to periodic in its request to the NSACFs. If single NSACF is selected, the NEF sets the Event Reporting Information identical to the received request from the AF. The NEF also sets the Event ID and Event Filter identical to the received request from the AF

NOTE 2:	The period chosen is selected by the NEF based on its internal logic.

5.	The NSACF(s) confirms with Nnsacf_SliceEventExposure_Subscribe/Unsubscribe Response message to the NEF. This message may include the event reporting if available at NSACF and immediate reporting or one-time reporting was requested by the NEF.

6.	When the reporting condition for a subscribed event is fulfilled, the NSACF triggers a notification towards the NEF.

7.	The NSACF sends the Nnsacf_SliceEvent Exposure_Notify (Event ID, Event Reporting information) message to the NEF. If the subscription is for event based notification (e.g. based on the monitored event reaching a threshold value), the Event Reporting information parameter contains confirmation for the event fulfilment. If the subscription is for periodic notification or for immediate reporting, the Event Reporting information parameter provides information for the current number of UEs registered with a network slice (e.g. represented in percentage of the maximum number of the UEs registered with the network slice) or information for the current number of PDU Sessions on a network slice (e.g. represented in percentage of the maximum number of the UEs established on the network slice).

8.	When a single NSACF is returned from the discovery procedure, the NEF sends the Nnef_EventExposure_Notify (Event ID, Event Reporting information) message since the reporting condition is fulfilled. In the case of Untrusted AF, the NEF includes the AF-Service-Identifier corresponding to the S-NSSAI in the returned notification.

9.	When multiple NSACFs are selected for the requested NSSAI the NEF performs the aggregation from reporting NSACF(s) and maintain the overall usage of the S-NSSAI for the selected NSACFs as long as the subscription is active.

NOTE 3:	If multiple NSACFs are selected for the requested S-NSSAI, the NEF continuously updates the aggregated information to be able to fulfil the incoming subscription request from the AF.

10.	When multiple NSACFs are selected for the requested S-NSSAI and when the reporting condition for a subscribed event by the AF is fulfilled, the NEF sends Nnef_EventExposure_Notify (Event ID, Event Reporting information) message towards the AF. In case of untrusted AF; the NEF includes the AF-Service-Identifier corresponding to the S-NSSAI in the returned notification.

##### 4.15.3.2.10a	Number of UEs and PDU Sessions per network slice notification procedure in Hierarchical NSACF architecture

This procedure depicts the case of an AF subscribing to receive the registered number of UEs, or the number of PDU sessions in a specific S-NSSAI. The procedure handles the case when hierarchical NSACF architecture is deployed.

Figure 4.15.3.2.10a-1: Number of UEs and PDU Sessions per network slice notification procedure in Hierarchical NSACF architecture

1-2. Same as the steps 1-2 defined in clause 4.15.3.2.10

3.	The NEF may query the NRF to find the Primary NSACF as per the clause in.

4.	If the NEF has not already subscribed to the event from the NSACF for the requested S-NSSAI, the NEF initiates the request Nnsacf_SliceEventExposure_Subscribe/Unsubscribe Request (Event ID, Event Filter, Event Reporting information, immediate reporting, S-NSSAI) to the Primary NSACFs supporting the requested S-NSSAI. The NEF stores the AF requested Event Reporting Information.

5.	The Primary NSACF confirms with Nnsacf_SliceEventExposure_Subscribe/Unsubscribe Response message to the NEF. This message may include the event reporting if available and immediate reporting or one-time reporting was requested by the NEF.

6.	If the Primary NSACF has not already subscribed to the event from the NSACF for the requested S-NSSAI, the Primary NSACF initiates the request Nnsacf_SliceEventExposure_Subscribe/Unsubscribe Request (Event ID, Event Filter, Event Reporting information, immediate reporting, S-NSSAI) to the NSACFs supporting the requested S-NSSAI. The Primary NSACF may set the Event Reporting Information to periodic in its request to the NSACFs. The Primary NSACF also sets the Event ID and Event Filter identical to the received request from the AF.

NOTE 1:	The period chosen is selected by the Primary NSACF based on its internal logic.

7.	The NSACF(s) confirms with Nnsacf_SliceEventExposure_Subscribe/Unsubscribe Response message to the Primary NSACF.

8.	When the reporting condition for a subscribed event is fulfilled, the NSACFs trigger notification towards the Primary NSACF.

9.	The NSACF sends the Nnsacf_SliceEvent Exposure_Notify (Event ID, Event Reporting information) message to the Primary NSACF. If the subscription is for event based notification (e.g. based on the monitored event reaching a threshold value), the Event Reporting information parameter contains confirmation for the event fulfilment. If the subscription is for periodic notification or for immediate reporting, the Event Reporting information parameter provides information for the current number of UEs registered with a network slice (e.g. represented in percentage of the maximum number of the UEs registered with the network slice) or information for the current number of PDU Sessions on a network slice (e.g. represented in percentage of the maximum number of the UEs established on the network slice).

10.	The Primary NSACF performs the aggregation from reporting NSACF(s) and maintain the overall usage of the S-NSSAI for the selected NSACFs as long as the subscription is active. The Primary NSACF includes the count for the current number of UEs registered or for the number of PDU session while aggregating the reports.

NOTE 2:	During UE’s mobility when the quota for the local maximum number is exceeded at NSACF then Primary NSACF may admit the request and increase the count for the current number of UEs registered or for the number of PDU session. Primary NSACF considers this usage while aggregating the report.

NOTE 3:	The Primary NSACF continuously updates the aggregated information to be able to fulfil the incoming subscription request from the NEF.

11.	When the reporting condition for a subscribed event by the NEF is fulfilled, the Primary NSACF sends Nnef_EventExposure_Notify (Event ID, Event Reporting information) message towards the NEF.

12. The NEF sends Nnef_EventExposure_Notify (Event ID, Event Reporting information) message towards the AF. In the case of untrusted AF; the NEF includes the AF-Service-Identifier corresponding to the S-NSSAI in the returned notification.

# * * * * Next change * * * *

### 5.2.21	NSACF services

#### 5.2.21.1	General

The following table illustrates the NSACF services.

Table 5.2.21.1-1: List of NSACF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnsacf_NSAC | NumOfUEsUpdate | Request/Response | AMF, SMF, (NOTE 1) |
|  | NumOfPDUsUpdate |  | SMF |
|  | EACNotify |  | AMF |
|  | LocalNumberUpdate |  | Primary NSACF |
| Nnsacf_SliceEventExposure | Subscribe | Subscribe/Notify | NEF, NWDAF, AF (NOTE 2) |
|  | Unsubscribe |  | NEF, NWDAF, AF |
|  | Notify |  | NEF, NWDAF, AF |
| NOTE 1: If EPS counting is required for the S-NSSAI, the SMF+PGW-C uses the Nnsacf_NumberOfUEs Update services operation and Nnsacf_NumberOfPDUsUpdate at PDN connection establishment procedure. NOTE 2: The AF can access NSACF services either via NEF to NSACF in case of untrusted AF or directly in case of trusted AF. |  |  |  |

# * * * * Next change * * * *

##### 5.2.21.2.2	Nnsacf_NSAC_NumOfUEsUpdate service operation

Service Operation name: Nnsacf_NSAC_NumOfUEsUpdate

Description: Updates the number of UEs registered with a network slice (e.g. increase or decrease) when the UE registration status for a network slice subject to NSAC has changed. Also, if the number of the UEs registered with the network slice is to be increased and the Early Availability Check (EAC) mode in the NSACF is activated for that network slice (see Nnsacf_NSAC_EACNotify service operation), the NSACF first checks whether the number of UEs registered with the network slice has reached the maximum number of UEs per network slice threshold. If the maximum number of UEs registered with the network slice has already been reached, the UE registration for that network slice via the same Access Type configured in the NSACF is rejected. If the EAC is not activated, the NSACF increases or decreases the number of UEs per network slice as per the input parameters below.

Inputs, Required: S-NSSAI(s), UE ID (SUPI), NF ID, Access Type, update flag.

Inputs, Conditional: Notification endpoint for EAC Notification for the S-NSSAI, UE already registered, current local maximum number, current UE admission threshold.

The S-NSSAI(s) parameter is a list of one or more network slices for which the number of UEs registered with a network slice is to be updated and checked if the maximum number of UEs per network slice threshold has already been reached.

The UE ID parameter is used by the NSACF to maintain a list of UE IDs registered with the network slice. The NSACF also takes Access Type into account for increasing and decreasing the number of UEs per network slice as described in clause 5.15.11.1 of TS 23.501 [2].

The NF ID parameter is the NF instance ID of the NF (e.g. AMF or SMF + PGW-C) sending the request to the NSACF.

The update flag input parameter indicates whether the number of UEs registered with a network slice is to be:

-	increased when the UE registers to a new network slice subject to NSAC. If the UE ID is already in the list of UEs registered with the network slice, the number of UEs registered with the network slice is not increased as the UE has already been counted as registered with the network slice. If the UE ID is not in the list of UE IDs registered with the network slice and the maximum number of UEs registered with the network slice has not been reached yet, the NSACF adds the UE ID in the list of UEs registered with the network slice and increases the number of the UEs registered with the network slice. If the UE_ID is not in the list of UEs registered with that S-NSSAI and the maximum number of UEs per network slice for that S-NSSAI has already been reached, then the NSACF returns maximum number of UEs per network slice reached result;

-	decreased when the UE deregisters for a network slice that is subject to NSAC. The NSACF decreases the number of the UEs registered with the network slice and removes the UE ID from the list of UEs registered with the network slice.

The UE already registered parameter indicates that the UE has already been registered. If this parameter is included, the local NSACF triggers a request to the Primary NSACF for increase of the current local maximum number of registered  UEs or the current local registered UEs admission threshold respectively so that the service continuity is maintained.

The NSACF may optionally return the current status of the network slice availability (e.g. a percentage out of the maximum number of UEs registered with a network slice) in the availability status parameter. This information may be used for NSACF signalling and load balancing in case multiple NSACFs are serving the same network slice.

Outputs, Required: Result indication.

The Result indication parameter contains the outcome of the update and check operation in the NSACF and may indicate one of the values 'maximum number of UEs for the S-NSSAI not reached' or 'maximum number of UEs for the S-NSSAI reached'.

Outputs, Optional: Updated local maximum number of UEs, Updated UE admission threshold.

# * * * * Next change * * * *

##### 5.2.21.2.4	Nnsacf_NSAC_NumOfPDUsUpdate service operation

Service Operation name: Nnsacf_NSAC_NumOfPDUsUpdate

Description: Updates the number of PDU Sessions established on a network slice (e.g. increase or decrease). Also, if the number of PDU Sessions on the network slice is to be increased, the NSACF first checks whether the number of the PDU Sessions on that network slice has reached the maximum number of PDU Sessions per network slice. If the maximum number of PDU Sessions on the network slice has already been reached, the PDU Session Establishment procedure is rejected.

Inputs, Required: S-NSSAI, UE ID, PDU Session ID, Access Type, update flag, current local maximum number.

The S-NSSAI parameter is the network slice for which the number of PDU Sessions established on a network slice is to be updated.

The UE ID parameter is used by the NSACF to maintain a list of UE IDs that has established PDU sessions with the network slice.

PDU Session ID parameter is used by the NSACF to maintain for each UE ID, the PDU Session ID(s) for established PDU Sessions.

The Access Type parameter indicates over which access network type the PDU Session is established. In the case of MA PDU Session, one or multiple Access Types may be included for a PDU Session ID.

The update flag input parameter indicates 'increase', 'decrease' or 'update' as specified in clause 4.2.11.4.

Inputs, Optional: None.

Outputs, Required: Result indication, Access Type.

The Result indication parameter contains the outcome of the update and check operation in the NSACF and may indicate one of the values 'maximum number of PDU Sessions for the S-NSSAI not reached' or 'maximum number of PDU Sessions for the S-NSSAI reached'.

The Access Type parameter is associated with the Result indication parameter.

Outputs, Optional:  Updated local maximum number of PDUs.

# * * * * Next change * * * *

##### 5.2.21.2.w	Nnsacf_NSAC_LocalNumberUpdate service operation

Service Operation name: Nnsacf_NSAC_LocalNumberUpdate

Description: NF service consumer, e.g. Primary NSACF, use this service operation to update local maximum number of UEs or PDU sessions of the network slice at NSACFs. The Primary NSACF may use to update the admission threshold when threshold based approach is used.

Inputs, Required: local maximum number of UEs, local maximum number of PDU sessions.

Inputs, Optional: Admission Threshold

Output, Required: Updated local number. .

Outputs, Optional: Updated admission threshold

# * * * * End of changes * * * *