3GPP TSG-SA2 Meeting #156E	S2-2305114

# E-meeting, April 17 – 21, 2023									                                     (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0364 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve the EN on the session break out for local LMF service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To resolve the EN: Editor's note: It is FSS whether and how to support session break out for local LMF service, i.e. whether SMF needs to perform ULCL/L-PSA insertion for the PDU session used for user plane positioning, e.g. based on UE location received from AMF. The AF influence traffic routing mechanism defined in 23.502 can be reused. After the AMF selecting the proper LMF based on the UE location and LMF serving area, the LMF may decide to use user plane posisioning method and send an IP address of the LMF to the UE. Then the UE can establish userplane connection with the LMF. Then the LMF can use the UE’s IP address of the userplane connection as the target UE IP address and send an AF request to the PCF as described in clause 4.3.6.4 to enable ULCL/L-PSA insertion and local traffic routing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifications are added to resolve the EN. AF influence traffic routing defined in clause 4.3.6.4 of 23.502 is reused for local UP LMFservice. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear about how to support the session break out for local LMF service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.0, 6.18.1, 6.18.2, 6.18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** 1st Change **********

6.18	Procedures of User Plane Connection between UE and LMF

6.18.0	General

Clause 6.18 describes the management of the user plane connection between UE and LMF. LMF or UE may trigger the establishment of the user plane connection.

UE and LMF may maintain the established user plane connection. LMF may modify or terminate the established user plane connection between UE and LMF.

Precondition:

UE uses URSP which includes user plane positioning related PDU session parameters (e.g. a dedicated DNN and S-NSSAI) to establish a PDU session used for user plane positioning. SMF should select a PSA UPF (located in central site or local site) connecting with the LMF for this PDU session, based on S-NSSAI, DNN and UE location information, etc.

After selected by the AMF, the LMF can send an AF request directly to the PCF to support the session break out for local LMF service reusing the procedure defined in clasue 4.3.6.4 of 23.502[19].

********** 2nd Change **********

6.18.1	LMF initiated User Plane Connection

LMF may trigger the user plane connection establishment after receiving a location request from AMF if target UE does not have user plane connection with LMF. AMF also subscribes from LMF the status of LCS user plane connection for the target UE, using a Nlmf_Location_UPNotify Subscribe message if the UE supports user plane positioning. Figure 6.18.1-1 shows a procedure triggered by LMF to support positioning over the user plane connection between UE and LMF.

Figure 6.18.1-1: Positioning via a User Plane Connection between UE and LMF initiated by LMF

1.	Based on UE user plane positioning capability and other implementation factors, LMF decides whether to continue the position procedure via a user plane connection between UE and LMF.

Steps 2-7 are skipped if there is already a user plane connection context of the target UE in LMF and LMF determines to utilize the user plane connection for positioning.

NOTE:	LMF can select user plane positioning for specific positioning methods (e.g. motion sensor-based method) and it is based on implementation and local configuration to determine which positioning method requires user plane transport.

2.	[Conditional] If LMF decides to utilize user plane for positioning and there is no established secure user plane connection between the UE and LMF, LMF sends a user plane information to AMF to indicate UE to utilize user plane for positioning. The user plane information includes the user plane positioning address of the LMF, security related information.

Editor's note:	The security mechanism and security information provided by LMF to UE is subject to SA WG3's work. It is FFS how the UE user plane positioning capability and LMF user plane information is transferred, e.g. based on enhanced LPP, supplementary service or NAS message.

3.	[Conditional] When AMF receives the user plane information from LMF in step 2, AMF sends it to UE via a DL NAS TRANSPORT message.

4.	[Conditional] If there is no established applicable PDU session for the user plane positioning, the UE uses the URSP as defined in TS 23.503 [41] which includes user plane positioning related PDU session parameters, e.g. a dedicated DNN and S-NSSAI, to establish the PDU session for user plane positioning. UE may send an acknowledgement to LMF through AMF to indicate a success of utilization of a user plane connection for positioning service or a failure to utilize the user plane connection, e.g. no suitable PDU session established.

5.	[Conditional] AMF sends the acknowledgement received in step 4 to the LMF via Namf_N1messageNotify service.

6.	[Conditional] If LMF knows the UE IP address information, LMF may notify UE to setup the secure user plane connection through the known UE IP address.

Editor's note:	It is stage 3 to check and decide the protocol used by LMF to notify UE through the known UE IP address.

7.	[Conditional] UE establishes a secured user plane connection with LMF. To support the session break out for local LMF service, the LMF may send an AF request to the PCF using the Npcf_PolicyAuthorization service as described in the clasue 4.3.6.4 of 23.502[19] with the known UE IP address as target UE IP, including the IP address of the LMF, the dedicated DNN and S-NSSAI, the DNAI of the LMF and the LMF’s serving area as Spatial Validity Condition for ULCL/L-PSA insertion and local traffic steering. If LMF send its FQDN to the UE, a DNS server/resolver is used to resolve the IP address of LMF (e.g. EASDF or local DNS for local LMF address resolution).

Editor's note:	IP address and/or FQDN as LMF address information needs to be checked/verified based on CT1/CT3 feedback.

8.	[Conditional] LMF indicates AMF in the Nlmf_Location_UPNotify message that user plane connection between the UE and LMF has been established.

9.	[Conditional] The AMF stores the LCS-UP connection context as part of UE context.

10.	If LMF or UE determines to utilize the user plane connection for positioning and the secure user plane connection is established, LPP messages are transferred between UE and LMF for UE based positioning, UE assisted positioning and delivery of assistance data. Supplementary services event report messages from UE may also be transferred to LMF via the established user plane connection.

********** 3rd Change **********

6.18.2	UE initiated User Plane Connection

UE may trigger the user plane connection establishment if the UE does not have user plane connection with LMF. Figure 6.18.2-1 shows a procedure triggered by UE to support positioning over the user plane connection between UE and LMF.

Figure 6.18.2-1: Positioning via a User Plane Connection between UE and LMF initiated by UE

1.	UE may send a user plane establishment request to AMF via NAS Message, if UE decides to prepare a user plane connection for upcoming positioning requests.

2.	[Conditional] If the UE is authorized based on UE Subscription to use the user plane positioning, AMF selects an LMF which capable to establish a user plane session for positioning with the UE.

3.	[Conditional] The AMF sends a Nlmf_Location_UPConfig Request towards LMF to request set up of an LCS-UP connection.

4.	[Conditional] If LMF accept to utilize user plane for positioning and there is no established secure user plane connection between the UE and LMF, LMF sends a user plane information to AMF to indicate UE to accept and utilize user plane for positioning. The user plane information includes the user plane positioning address of the LMF, security related information.

5.	[Conditional] When AMF receives the user plane information from LMF in step 4, AMF forwards it to UE via a DL NAS TRANSPORT message.

6.	[Conditional] UE establishes a secured user plane connection with LMF. To support the session break out for local LMF service, the LMF may send an AF request to the PCF for ULCL/L-PSA insertion and local traffic steering.

7.	[Conditional] LMF responds to AMF that user plane connection between the UE and LMF has been established.

8.	[Conditional] The AMF stores the LCS-UP connection context as part of UE context

9.	[Conditional] After the secure user plane connection is established, if LMF determines to utilize the user plane connection for positioning after receiving a positioning request from AMF, or UE determines to utilize the user plane connection for positioning, LPP messages are transferred between UE and LMF for UE based positioning, UE assisted positioning and delivery of assistance data. Supplementary services event report messages from UE may also be transferred to LMF via the established user plane connection.

********** 4th Change **********

6.18.3	Modification of User Plane Connection between UE and LMF

The figure 6.18.3-1 shows how a secure user plane connection between UE and LMF is modified. The flow describes change of LMF but applies also when source and target LMF is the same. The procedure can also be used to terminate the user plane connection to Source LMF not selecting any Target LMF.

Figure 6.18.3-1: Connection modification between UE and LMFs

1a.	[Conditional] The LMF discovers a need to change LMF or re-establish the user plane connection between UE and LMF. The LMF sends an Nlmf_Location_UPNotify message that includes user plane information and indicates the reason for modification. The address of the AMF was provided to LMF as a "Notification Target Address" in latest Nlmf_Location_UPConfig message or Nlmf_Location_UPNotify Subscribe message.

NOTE:	The LMF change procedure is independent from the SSC mode of the PDU Session with dedicated DNN used for positioning. For SSC mode 2/3, PSA UPF connection with the LMF can be relocated with UE movement and LMF can discover the need to change LMF to reduce the user plane path latency.

1b.	[Conditional] AMF based on LMF indications need to reselect an LMF used for user plane positioing. LMF needs to be capable to establish a user plane session for positioning with the UE. Further at AMF relocation the target AMF needs to inform the LMF of the AMF change.

2.	[Conditional] If AMF reallocation has occurred, this step is skipped. Otherwise, steps 3-8 of figure 6.18.2-1 are performed between AMF, UE, and Target LMF with addition that UE also terminate connection to Source LMF.

3.	The AMF sends an Nlmf_Location_UPConfig Request towards the source LMF. The message may include a request for the Source LMF to terminate a specific user plane connection to the UE and the Target LMF. Alternatively, it may include information about AMF reallocation.

4-5.[Conditional] The source LMF may invoke an Nlmf_Location_LocationContextTransfer Request service operation towards the target LMF to provide the current location context of the UE, if there is periodic and triggered UE location events report context. The target LMF informs source LMF of the location context transfer operation results. To support the session break out for local LMF service, the target LMF may send an AF request to the PCF for ULCL/L-PSA insertion and local traffic steering.

6.	[Conditional] If the user plane connection to source LMF is still active, the source LMF terminates the connection to the UE.

7.	The LMF sends Nlmf_Location_UPConfig Response message to AMF to confirm connection termination or acknowledge change of AMF.

********** End of Change ************