3GPP TSG-WG SA2 Meeting #156-e	S2-2305124

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0100 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of support of A2X capability of UAV UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is agreed one statement in last meeting, in this version of the specification, it is assumed all UAV UEs requiring DAA and BRID support A2X capability. However, since rel-18 also defined MBS-based BRID solution, which does not require A2X UE to support A2X capability. The agreed texts need update to reflect the conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to reword the sentence: In this version of the specification, UAV UEs support A2X capability for PC5-based DAA and BRID. MBS-based BRID does not require UAV UE to support A2X capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of A2X capability for UAV UE is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.2.1.2.1	General

This clause describes the support of an Aircraft-to-everything (A2X) mechanism based on PC5 reference point. A2X leverages V2X mechanisms as defined in TS 23.287 [X] to support Broadcast Remote ID (BRID) and Direct Detect And Avoid (DDAA). A2X leverages both LTE PC5 as defined in TS 23.285 [Y] and NR PC5. For LTE PC5 in EPS, the network scheduled operation mode defined in TS 23.285 [Y] is not supported and the A2X uses only the UE autonomous resources selection mode.

A2X supports the following communication modes:

-	Broadcast communication mode is used for BRID.

-	Broadcast communication mode is used for DDAA to advertise UAV information.

-	Broadcast over PC5 or unicast over PC5 may be used between two or more UAVs for DDAA deconfliction triggered at the application layer by the information received by UAV via DAA messages received in broadcast.

Groupcast mode for NR based PC5 is not supported. PC5 Relay communications are not supported in this release.

Subscription to A2X services is based on user's profile stored in the UDM containing the subscription information to give the user permission to use A2X services, as described in clause 5.5 of TS 23.287 [X] with the following differences:

- The distinction between Vehicle UE and Pedestrian UE is not applicable to A2X.

Both UAV UEs that utilize Uu connectivity and that do not utilize Uu connectivity (i.e. either UAV UEs that are Uu capable and do not use Uu) are supported. A UAV without utilizing Uu capabilities may use A2X for BRID and DDAA and be configured via A2X1 over a transport outside the scope of 3GPP.NOTE 1: UAV UEs without utilizing Uu capabilities are part of the 3GPP ecosystem since they use A2X1 for configuration by a A2X Application Server and implement PC5 connectivity specified by 3GPP.

Both UAVs with UICC and UAVs without UICC (i.e. with no subscription to an MNO) are supported. UAVs with no UICC can only perform A2X communications when authorized for "not served by E-UTRA" and "not served by NR".

In this version of the specification, UAV UEs support A2X capability for PC5-based DAA and BRID. MBS-based BRID does not require UAV UE to support A2X capability.

Editor's note: 	Whether A2X communication over PC5 between the UAV UEs served by different PLMNs and with subscriptions to different PLMNs is supported depends on the RAN2 feedback.

NOTE 2:	It is assumed that any security of A2X solution is addressed by SA WG3.

NOTE 3:	the A2X application layer schemes developed in other SDOs are outside the scope of this specification.

# * * * * End of changes * * * *