3GPP TSG-WG SA2 Meeting #156e	S2-2305126

# Elbonia, April 17 – 21, 2023		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4490 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the description of reporting suggestion information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2#154AH meeting, TS 23.502 has agreed the description of reporting suggestion information. However, in TS 23.501 the description is different. Therefore, it is proposed to update the TS 23.501 to align with TS 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description of reporting sugegstion inforamtion that replacing “When the related event is detected, the Reporting window defines the last reporting valid time.” With If the Reporting urgency information indicates “delay tolerant”, the Reporting time is also provided, which defines the last valid reporting time, and UPF shall report the detected event before the last valid time. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mis-alignment of the description of reporting suggestion information, between TS 23.501 and TS 23.502. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.8.2.17	Data exposure via Service Based interface

The UPF may expose information described in TS 23.502 [3] clause 5.2.26.2, via a service-based interface directly. The NF consumer, which may receive UPF event notifications, are AF/NEF, TSNAF/TSCTSF and NWDAF.

When the UPF supports the data exposure via the SBI interface, it may register its NF profile to the NRF including the UPF Event Exposure service. For the UPF Event Exposure service, the NF profile includes the related event ID(s).

To get exposure data from UPF, NF consumer do the subscription to the UPF directly or indirectly via SMF. UPF selection is further described in clause 6.3.3.1.

Only NWDAF can subscribe to the UPF event exposure service directly and only following cases:

- Data collected for UPF load analytics.

- Data collected for analytics targeting "any UE", but not related with an AoI or with a specific data flow or BSSID/SSID or application.

To minimize the impact of event notification to UPF data processing, the event subscription may include Reporting suggestion information. The Reporting suggestion information includes Report urgency and Reporting window information. Reporting urgency information represents whether this event report can be delay tolerant, i.e. the event report can be delayed. If the Reporting urgency information indicates “delay tolerant”, the Reporting time is also provided, which defines the last valid reporting time, and UPF shall report the detected event before the last valid time. Per Reporting suggestion information UPF can concatenate several notification messages to the same notification endpoint in one notification message.

# * * * * End of changes * * * *