3GPP TSG-WG SA2 Meeting #156e	S2-2305155

# April 17 – 21, 2023, Elbonia		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4115 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Satellite coverage availability information exposure to AMF from the AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18 FS_5GSAT_Ph2 conclusion, the AMF can obtain the Satellite coverage availability information from the an AF via the NEF/SCEF, from the UE or from OAM. However, no procedure can be reused by the AMF to obtain the Satellite coverage availability information from the AF in TS23.502. It is recommended that a new procedure to support the Satellite coverage availability information exposure to AMF from the AF. The corresponding service operations have also been updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New procedure to support the Satellite coverage availability information exposure to AMF from the AF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AMF cannot obtain the Satellite coverage availability information from the AF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.X, 5.2.6.2.2, 5.2.6.2.4, 5.2.19.2.2, 5.2.19.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.15.3.2.X	Satellite coverage availability information exposure to AMF from the AF

For NR satellite access that provides discontinuous network coverage, the AMF may use Satellite coverage availability information to support satellite access by UEs. This procedure is used by the AMF to obtain Satellite coverage availability information may be via the NEF from the AF.

Figure 4.15.3.2.3-1: Satellite coverage availability information exposure to AMF from the AF

1a. An AMF sends a request to subscribe the Satellite coverage availability information in Nnef_EventExposure_Subscribe from the NEF.

1b. The NEF sends a request to subscribe the Satellite coverage availability information in Naf_EventExposure_Subscribe from the AF.

1c. If the AF is considered to be trusted by the operator, the AMF sends the Naf_EventExposure_Subscribe message directly to AF to obtain the Satellite coverage availability information.

2a. The AF sends Naf_EventExposure_Notify including Satellite coverage availability information to the NEF.

2b. The NEF sends Nnef_EventExposure_Notify including Satellite coverage availability information to the AMF.

2c. If the AF is considered to be trusted by the operator, the AMF sends the Naf_EventExposure_ Notify message including Satellite coverage availability information directly to AMF.

# * * * * Second change * * * *

5.2.6.2.2	Nnef_EventExposure_Subscribe operation

Service operation name: Nnef_EventExposure_Subscribe

Description: the consumer subscribes to receive an event, or if the event is already defined in NEF, then the subscription is updated.

Inputs, Required: (Set of) Event ID(s) as specified in clause 4.15.3.1 or Npcf_PolicyAuthorization_Notify and Naf_EventExposure_Subscribe service operation, Target of Event Reporting (GPSI, SUPI, UE IPv4 address(es), UE IPv6 prefix(es), External Group Identifier, S-NSSAI, Internal Group Identifier, UE addressing information (IP or MAC address), or indication that any UE is targeted), Event Reporting Information defined in Table 4.15.1-1, Notification Target Address (+ Notification Correlation ID), MTC Provider Information.

Inputs, Optional: Event Filter, (set of) External Application Identifier(s), Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, list of group member UE(s) whose subscription to event notification(s) are removed or added for a group-based event notification subscription, operation indication (cancellation or addition), DNN, S-NSSAI, Idle Status Indication request (if UE reachability or Availability after DDN failure reporting is requested), Satellite coverage availability information.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding event report is included, if available (see clause 4.15.1). External Identifier (representing an AF specific UE Identifier).

# * * * * Third change * * * *

5.2.6.2.4	Nnef_EventExposure_Notify service operation

Service operation name: Nnef_EventExposure_Notify

Description: NEF reports the event to the consumer that has previously subscribed.

Inputs, Required: Event ID, Notification Correlation Information, time stamp.

Inputs, Optional: Event information (defined on a per Event ID basis), Event Removal Indication, list of group member UE(s) whose subscription to event notification(s) are removed from a group-based event notification subscription, UE(s) added/removed to/from the group, Satellite coverage availability information.

Outputs, Required: Operation execution result indication.

# * * * * Fourth change * * * *

5.2.19.2.2	Naf_EventExposure_Subscribe service operation

Service operation name: Naf_EventExposure_Subscribe

Description: The consumer NF subscribes the event to collect AF data for UE(s), group of UEs, or any UE, or updates the subscription which is already defined in AF.

Input, Required: Target of Event Reporting (either UE ID(s), or UE IPv4 address(es), or UE IPv6 prefix(es), or Internal/External Group Identifier, or indication that any UE is targeted), (set of) Event ID(s), Notification Target Address (+ Notification Correlation ID) and Event Reporting Information as defined in Table 4.15.1-1.

NOTE 1:	UE ID includes GPSI or SUPI.

Input, Optional: NF ID, Event Filter(s) associated with each Event ID, (set of) External Application Identifier(s), Subscription Correlation ID (in the case of modification of the existing subscription), Expiry time, Satellite coverage availability information.

NOTE 2:	In the case of untrusted AF, NEF ID is used as NF ID.

Output, Required: Operation execution result indication. When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the local policy).

Output, Optional: First corresponding event report is included, if corresponding information is available (see clause 4.15.1).

# * * * * Fifth change * * * *

5.2.19.2.4	Naf_EventExposure_Notify service operation

Service operation name: Naf_EventExposure_Notify

Description: The AF provides the previously subscribed event information to the consumer NF which has subscribed to that event before.

Input, Required: Notification Correlation Information, Event ID, corresponding UE ID(s) (either external UE ID(s), or Internal/External Group Identifier, or UE IP v4 address(es) or UE IP v6 prefix(es), time stamp.

NOTE:	UE ID includes GPSI or SUPI.

Input, Optional: Event specific parameter list, Satellite coverage availability information.

Output, Required: None.

Output, Optional: None.

# * * * * End of changes * * * *