SA WG2 Meeting #S2-156E	S2-2305162

17 - 21 April 2023, Electronic, Elbonia	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4510 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving the ENs on the Provisioning and monitoring for a group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2023-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are two ENs in the clause Provisioning of traffic characteristics and monitoring of performance characteristics for a group. For the 1st EN, it proposes the NEF and TSCTSF description are captured in the 23.501. PCF part should be captured in the 23.503 For the 2nd EN, it proposes the NEF The reference to 23.502 clause 4.15.6.13 hsall be clause 4.15.6.14 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the ENs. Correct the reference clasue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ENs are not resolved and the 23.502 reference clause is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.20c |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 5.20c	Provisioning of traffic characteristics and monitoring of performance characteristics for a group

NEF provisioning capability as defined in clause 5.20 allows an AF to perform provisioning of traffic characteristics and monitoring of performance characteristics for a group of UEs as specified in clause 4.15.6.14 of TS 23.502 [3]. The AF request information may include:

-	DNN and S-NSSAI.

-	Flow Descriptions as described in clause 6.1.3.6 of TS 23.503 [45],

NOTE 1:	At the time of providing Flow Description from AF, the AF can only provide the server side information as Flow Description, e.g. 3-tuple(s) including protocol, server side IP address and port number. At the time of applying the Flow Description for generation of PCC rules, the PCF can supplement the UE side information, e.g. UE IP address and port.

-	Traffic characteristics as described in clause 6.1.3.23 or 6.1.3.23a of TS 23.503 [45],

-	QoS parameters (e.g. QoS Reference or individual QoS parameters or Alternative QoS Parameter Set) as defined in clause 6.1.3.22 of TS 23.503 [45],

-	QoS parameters for monitoring as defined in clause 6.1.3.21 of TS 23.503 [45],

-	Temporal invalidity condition (start-time, end-time), indicate the time period when an established connection will not carry any user payload (e.g. at night or on weekends),

-	External Group Identifier or GPSI.

-	Subscription to events as defined in clause 6.1.3.18 of TS 23.503 [45].

NOTE 2:	The AF can invoke SEAL service as described in clause 14.3.5 of TS 23.434 [177] to handle requirements for UE-to-UE traffic, the SEAL service can perform coordinated QoS/resource management for network assisted UE-to-UE communications to guarantee end-to-end QoS fulfilment (primarily for meeting end-to-end latency requirements), e.g. decompose the requirements for UE-to-UE traffic to initial individual QoS parameters for each individual session, and apply QoS adaption for the individual session that receive QoS notification control notifications based on fulfilment/non-fulfilment evaluation on the requirements for UE-to-UE traffic.

The NEF determines whether to invoke the TSCTSF.

NOTE 3:	The NEF can make the decision according to characteristics parameters, AF identifier and operator configuration.

In the case that the TSCTSF is used, the TSCTSF retrieves the AF request QoS information from the NEF. The TSCTSF applies the AF request QoS information for each UE within the group as specified in clause 4.15.6.14 of TS 23.502 [3]. If the AF request QoS information contains the temporal invalidity condition,

-	When the start-time is reached, the TSCTSF revokes AF request QoS information to the PCF for each UE within the group, so to initiate SM Policy Association Modification to remove the QoS Flow or de-activate the UP connection of a PDU Session as defined in clause 4.16.5.2 of TS 23.502 [3]

-	When the end-time is reached, the TSCTSF provides AF request QoS information to PCF so to initiate SM Policy Association Modification to add a new QoS Flow or activate the UP connection of a PDU Session as defined in clause 4.16.5.2 of TS 23.502 [3]

In the case that TSCTSF is not used, the AF request is handled as described in clause 4.15.6.14 of TS 23.502 [3]. If the AF request QoS information contains the temporal invalidity condition,

-	When the start-time is reached, the PCF initiates SM Policy Association Modification to remove the QoS Flow or de-activate the UP connection of a PDU Session as defined in clause 4.16.5.2 of TS 23.502 [3]

-	When the end-time is reached, the PCF initiates SM Policy Association Modification to add a new QoS Flow or activate the UP connection of a PDU Session as defined in clause 4.16.5.2 of TS 23.502 [3]

END OF CHANGES