SA WG2 Meeting #S2-156E	S2-2305177

17 - 21 April, 2023, Electronic

Source:		Interdigital

Title:	TS 23.586: Procedures for Service exposure to SL Positioning Client UE

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper proposes the Procedures for Service exposure to SL Positioning Client UE.

# 1. Introduction/Discussion

The procedures for service exposure to SL Positioning Client UE shall be specified.

# 2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change (All text new)* * * *

## 6.6	Procedure for Ranging/SL Positioning service exposure

### 6.6.1	Procedures for Service exposure to SL Positioning Client UE

#### 6.6.1.1	Procedures for Ranging/SL Positioning service exposure through PC5

Figure 6.6.1-1.	Service exposure to SL Positioning Client UE via PC5

1.	When a Ranging/SL Positioning service is triggered, SL Positioning Client UE may perform discovery procedure for the Ranging/SL Positioning service defined in clause 6.X.

When SL positioning service is triggered, the client UE tries to discover the target UE and/or a SL Reference UE.

When Ranging service is triggered, the client UE tries to discover UE1 and/or UE2.

During discovery for SL positioning service, the information of SL reference UE is acquired.

During discovery for Ranging service, it is informed to the client UE if UE1 and UE2 are not in proximity.

2.	If discovery procedure succeeds, the SL Positioning Client UE establishes a PC5 connection with the discovered UE.

3.	The SL Positioning Client UE sends a SL Positioning Service Request which includes info of SL Positioning Client UE, info of Target UE and info of SL Reference UE for SL positioning, and info of SL Positioning Client UE, info of UE1 and info of UE2 for Ranging.

4.  Authorization check of the client UE for the service request may be performed via SL Positioning Server UE or 5GC or internally by UE1 or target UE.

5.   Ranging/SL Positioning operation is performed as described in clause 6.Y among target UE, SL Reference UE and SL Positioning Server UE or LMF.

6.  The Ranging/SL Positioning result is provided to the SL Positioning client UE by UE1.

# * * * * End of changes * * * *