SA WG2 Meeting #156e	S2-2305188

E-Meeting April 17 - 21, 2023

Title:	[DRAFT] LS on PDU Set Discard Usage for XR and Media Services

Response to:	None

Release:	Release 18

Work Item:	XRM

Source:	TSG SA WG2

To:	TSG RAN WG2, TSG RAN WG3

Cc:

Contact Person:

Name:		Jinhua WU

E-mail Address:	wujinhua@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-230xxxx

1. Overall Description:

TR 38.835 concludes the discard and the discard operation of PDU Sets. It understood that eligible discard by the NG-RAN is applied.

SA2 would like to indicate that SA2 has made the following conclusions in TR 23.700-60, that if the DL PDU Set eligible dropping by the NG-RAN applied, the charging offset issue can be addressed during the normative work.

SA2 has discussed the issue and concluded that: DL PDUs of eligible discard in the NG-RAN will be send back and discounting from the DL usage measurement for charging in UPF. The attached CR has been agreed by SA2, as the proposal for the usage offset.

SA2 wants to request RAN2, RAN3 whether the DL discarding usage in the NG-RAN is large enough for the charging offset. If it is, which alternative proposal has more feasibility and prefer by the RAN:

- Alternative 1:	 The eligible discard PDUs of the DL PDU set can be marked and sent back to the UPF ensuring that these discard PDUs are not included in charging and usage monitoring records.

- Alternative 2:	 RAN enhanced to support the discard PDUs of the DL PDU set counting and send the usage measurement result to UPF for the charging offset.

[Texts to be provided based on the meeting discussion result]

2. Actions:

To RAN WG2, RAN WG3:

ACTION: 	SA2 respectfully asks RAN2, RAN3 to take the above information into account and provide feedback on the topic above.

3. Date of Next TSG-RAN WG2 Meeting:

Next SA2 Meetings dates