3GPP TSG-SA2 Meeting #156-e	S2-2305224

# Online, , 17th Apr 2023 - 21st Apr 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4530 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Experience filering criteria in assistace to UE member selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital, Europe, Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 23.501, clause 5.46.2, “The 5G system may support UE member selection assistance functionality to assist the AF in selecting member UE(s) that can be used in application operations such as AI/ML based applications”. The functionality entails an AF request that includes a list of target UE members, an optional time window(s) and one or more filtering criteria. Furthemore, as per current Editor’s note in clause 4.15.13.2, “The list of UE Member filtering criteria and their corresponding actions are FFS”. In addtion, as part of the conclusions from 23.700-80, clause 8.7.2, whethere Service Experience analytics can be used as filtering criteria to assistt in UE member selection can be discussed during normative phase. Therefore this CR discusses how Service Experience analytics can be used as filtering criteria to assistt in UE member selection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new Filtering Criteria considering Service Experience, enbling AF to request UE member selection assistance based on Service Experience for a particular Service Experience Type and with the possibility to define thresholds consdiering contribution weights |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE member selection functionality won’t be able to use Service Experince analytics, useful to detect e.g., unperforming UEs, as filtering criteria |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.46.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.502, CR 4031 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 5.46.2	UE member selection assistance functionality for application operation

5G system may support UE member selection assistance functionality to assist the AF in selecting member UE(s) that can be used in application operations such as AI/ML based applications (e.g. Federated Learning). The UE member selection assistance functionality shall be hosted by NEF, and the features of the UE member selection assistance functionality hosted by NEF include (see clause 4.15.13 of TS 23.502 [3] for details of UE member selection procedures):

-	Receiving a request from the AF that shall include a list of target member UE(s), optionally (a) time window(s), and one or more filtering criteria as specified in Table 4.15.13.2-1 of TS 23.502 [3] (e.g. number of UEs, UE location, trajectory and direction, QoS requirements, Area of Interest, DNN, S-NSSAI, expected application operation time duration, preferred access/RAT type, Desired UE transmission latency performance, or Service Experience).

-	Referring to the filtering criteria provided by the AF, NEF interacts with 5GC NFs using existing services to collect the corresponding data from relevant 5GC NFs (e.g. PCF, NWDAF and AMF) to derive the list(s) of candidate UE(s) (i.e. UE(s) among the list of target member UE(s) provided by the AF).

-	Providing the AF with the UE member selection assistance information, including one or more lists of candidate UE(s), and optionally other additional information (e.g. one or more recommended time window(s) for performing the application operation, QoS of each target UE, UE(s) location, Access/RAT type, or Service Experience.).

The UE member selection assistance information provided by the NEF can be used by the AF to select member UEs. (See clause 5.2.6.31 of TS 23.502 [3] for details of parameters)

NOTE:	AF can choose whether to use the UE member selection assistance functionality provided by NEF.

AF in either trusted or untrusted domain can select the UE members e.g. participating in federating learning operation, by collecting the corresponding data using network exposure information as described in clause 4.15.13 of TS 23.502 [3], e.g. UE location reporting from the AMF, user plane information from the UPF and data analytics from NWDAF.

Editor's note:	The term "UE member selection" is FFS.

* * * End of Changes * * *