3GPP TSG-SA WG2 Meeting #156 (e-meeting)	 S2-2305242

# Elbonia, April 17 – 21, 2023																	(rev of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4536 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove the EN for Non-3GPP delay budget |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the clause 5.44.3.4 it clearly states that “The 5GC takes the non-3GPP delay budget received from the PEGC into consideration when allocate the delay budget for 5GS”, so that means SMF will take the non-3GPP delay budget into account when allocates the delay budgets in 5GS, which already resolve this EN. Editor's note: Whether SMF or PCF reduce the PDB by the non-3GPP delay budget is FFS. As in non-3GPP network, e.g., wifi, the delay budget between PINE and PEGC is not stable or even with huge jitter, from time to time, then it will cost lots of signallings to frequently request to update the delay budget in 5GS. The benefit is less than the costs it brings. Only in the specific PIN service, e.g., delay sensitive service with stable delay budget, It may bring thet benefit. So propose to remove the following NOTE, with new NOTE to indicate that When the non-3GPP delay budget between PINE and the PEGC is not stable, the PEGC will not provide it to 5GC. NOTE: It is up to CT WG1 to discuss to potentially introduce a timer to limit how often a PEGC is allowed to request a delay budget. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the EN above as it is resolved by existing normative text - Add clarification for scenarios or conditions when applying the non-3GPP delay budget - Remove the NOTE above, by adding new NOTE to state when the non-3GPP delay budget between PINE and the PEGC is not stable, the PEGC will not provide it to 5GC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The non-3GPP delay budget feature is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.44.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

5.44.3.4	Non-3GPP delay budget between PINE and PEGC

Non-3GPP delay budge is the delay occurred between the PEGC and the PINE in the non-3GPP network, which is not covered in the PDB. 5GC may need to be aware of the non-3GPP delay budget when providing connection for the PINE behind the PEGC, for the specific delay sensitive service with stable delay budget.

If the PEGC supports requesting of the non-3GPP delay budget for a specific flow, the PEGC may use the UE requested PDU Session Establishment/Modification procedure to provide a non-3GPP delay budget. The 5GC takes the non-3GPP delay budget received from the PEGC into consideration when allocate the delay budget for 5GS.

NOTE: When the non-3GPP delay budget between PINE and the PEGC is not stable, the PEGC will not provide it to 5GC.

NOTE:	When the non-3GPP delay budget between PINE and the PEGC is not stable, the PEGC will not provide it to 5GC.

* * * End of Changes * * *