3GPP TSG-WG SA2 Meeting #156E 	S2-2305339

Elbonia, April 17 - 21, 2023

Source:	Intel

Title:	TS 23.586: Procedures for Service Authorization to NG-RAN

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper proposes procedures for Service Authorization to NG-RAN for Ranging based services and sidelink positioning.

# 1. Introduction/Discussion

This paper proposes procedures for Service Authorization to NG-RAN for Ranging based services and sidelink positioning.

# 2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

## 6.3	Procedures for Service Authorization to NG-RAN

### 6.3.1	General

In order to support PC5 radio resource control in NG-RAN, UE authorization status about Ranging/SL Positioning over PC5 and PC5 QoS parameters related to Ranging/SL positioning need to be made available in NG-RAN. This clause describes the corresponding procedures and aspects.

### 6.6.2	Registration procedure

The Registration procedure for UE is performed as defined in TS 23.502 [5] clause 4.2.2.2 with the following additions:

-	The UE includes the Ranging/SL positioning Capability in the Registration Request message. The AMF stores the Ranging/SL positioning Capability.

-	The AMF obtains the Ranging/SL positioning subscription data as part of the user subscription data from UDM during UE Registration procedure using Nudm_SDM service as defined in clause 4.2.2.2.2 of TS 23.502 [5].

-	The AMF determines whether the UE is authorised to use Ranging/SL positioning services based on UE's Ranging/SL positioning Capability and the Ranging/SL positioning Service Authorisation included in the subscription data received from UDM. The AMF stores the authorized Ranging/SL positioning services Capability.

-	The AMF sends the authorized Ranging/SL positioning Capability for Ranging/SL positioning operation to PCF. Based on the received Ranging/SL positioning Capability from the AMF, the PCF provides the PC5 QoS parameters related to Ranging/SL positioning to AMF. The AMF stores such information as part of the UE context.

-	If the UE is authorised to use Ranging/SL positioning services, then the AMF shall include in a NGAP message sent to NG-RAN:

-	" Ranging/SL positioning services authorised" information

-	the PC5 QoS parameters related to Ranging/SL positioning used by the NG-RAN for the resource management.

-	If the UE is authorised to use Ranging/SL positioning services, then the AMF should not initiate the release of the signalling connection after the completion of the Registration procedure. The release of the signalling connection relies on the decision of NG-RAN, as specified in TS 23.502 [5].

### 6.6.3	Service Request procedure

The Service Request procedures for UE in CM-IDLE state are performed as defined in TS 23.502 [5] clause 4.2.3.2 and clause 4.2.3.3 with the following additions:

-	If the UE is authorised to use Ranging/SL positioning services, then the AMF shall include " Ranging/SL positioning services authorised" information in the NGAP message, indicating the UE is authorised to use Ranging/SL positioning services as described in clause 6.6.2.

-	The AMF sends the PC5 QoS parameters related to Ranging/SL positioning to NG-RAN via N2 signalling. The PC5 QoS parameters related to Ranging/SL positioning may be stored in the UE context after the registration procedure. If the UE is authorised to use Ranging/SL positioning services but AMF does not have PC5 QoS parameters related to Ranging/SL positioning available, the AMF fetches the PC5 QoS parameters related to Ranging/SL positioning from the PCF.

### 6.6.4	N2 Handover procedure

The N2 based handover procedures for UE are performed as defined in TS 23.502 [5] clause 4.9.1.3 with the following additions:

-	If the UE is authorised to use Ranging/SL positioning services, then the target AMF shall send the " Ranging/SL positioning service authorised" information and PC5 QoS parameters related to Ranging/SL positioning to the target NG-RAN in the NGAP Handover Request message.

### 6.6.5	Xn Handover procedure

The Xn based handover procedures for UE are performed as defined in TS 23.502 [5] clause 4.9.1.2 with the following additions:

-	If the "Ranging/SL positioning service authorised" information is included in the UE context, then the source NG-RAN shall include a "Ranging/SL positioning service authorised" information and PC5 QoS parameters related to Ranging/SL positioning in the XnAP Handover Request message to the target NG-RAN.

-	If the "Ranging/SL positioning service authorised" information is included in the UE context, then the AMF shall send the "Ranging/SL positioning service authorised" information and PC5 QoS parameters related to Ranging/SL positioning to the target NG-RAN in the Path Switch Request Acknowledge message.

### 6.6.6	Subscriber Data Update Notification to AMF

The procedure of Subscriber Data Update Notification to AMF is performed as defined in TS 23.502 [5] clause 4.5.1 with the following additions:

-	The Nudm_SDM_Notification service operation may contain the Ranging/SL positioning service Authorisation.

-	The AMF updates the UE Context with the above new Ranging/SL positioning service subscription data.

-	When the AMF updates UE context stored at NG-RAN, the UE context contains the Ranging/SL positioning service subscription data.

### 6.6.7	Delivery of PC5 QoS parameters related to Ranging/SL positioning to NG-RAN

The UE Policy Association Establishment procedure and UE Policy Association Modification procedure, as defined in TS 23.502 [5], is used to provide the AMF with PC5 QoS parameters related to Ranging/SL positioning used by NG-RAN. When receiving the Ranging/SL positioning service Capability in Npcf_UEPolicyControl_Create Request from the AMF or when receiving the updated subscription data from UDR, the PCF generates the PC5 QoS parameters related to Ranging/SL positioning used by NG-RAN corresponding to a UE.

The (V-)PCF provides the information to the AMF as follows:

-	In the roaming case, the H-PCF includes the PC5 QoS parameters related to Ranging/SL positioning used by NG-RAN in the Npcf_UEPolicyControl_Create Response message or Npcf_UEPolicyControl UpdateNotify Request message sent to V-PCF in an N2 PC5 policy container, and V-PCF relays this N2 PC5 policy container as the N2 container in the Namf_Communication_N1N2MessageTransfer message sent to AMF.

-	In the non-roaming case, the PCF includes the PC5 QoS parameters related to Ranging/SL positioning used by NG-RAN in an N2 container in Namf_Communication_N1N2MessageTransfer message sent to AMF.

When the AMF receives the N2 PC5 policy container from (V-)PCF, the AMF stores such information as part of the UE context.

In the UE Configuration Update procedure triggered by UE Policy Association Establishment or UE Policy Association Modification:

-	The AMF forwards the PC5 QoS parameters related to Ranging/SL positioning in the NGAP message to the NG-RAN if a N2 PC5 policy container is received in the Namf_Communication_N1N2MessageTransfer message.

NOTE 1:	If the PC5 QoS parameters related to Ranging/SL positioning are provided to both NG-RAN and UE, both the N2 PC5 Policy Container and the UE Policy Container are included in the Namf_Communication_N1N2MessageTransfer message.

NOTE 2:	Non-UE specific PC5 QoS parameters, e.g. default PC5 QoS parameters, can also be locally configured in NG-RAN. How such configuration is performed is out of scope of this specification.

# * * * * End of changes * * * *