3GPP TSG-WG SA2 Meeting #156E 	S2-2305342

Elbonia, April 17 - 21, 2023

Source:	Intel

Title:	TS 23.586: Procedures for UE Discovery

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper proposes Procedures for UE Discovery.

# 1. Introduction/Discussion

This paper proposes Procedures for UE Discovery.

# 2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

## 6.4	Procedures for Ranging/ Sidelink Positioning UE Discovery

### 6.4.1	Ranging/ Sidelink Positioning UE Discovery for 5G ProSe capable UEs

Both Model A and Model B discovery as defined in TS 23.304 [7] are supported:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

Depicted in figure 6.4.1.1-1 is the procedure for Ranging/ Sidelink Positioning UE Discovery with Model A.

Figure 6.4.1.1-1: Ranging/Sidelink Positioning UE Discovery with Model A

1.	The Announcing UE sends an Announcement message as defined in TS 23.304. The Announcement message may include the Type of Discovery Message, Announcer Info and Ranging/SL Positioning service identifier, Ranging/SL Positioning UE role (e.g. target UE, SL Reference UE).

The Destination Layer-2 ID and Source Layer-2 ID used to send the Announcement message are specified in TS 23.304 [7].

Depicted in figure 6.4.1.1-2 is the procedure for Ranging/Sidelink Positioning UE Discovery with Model B.

Figure 6.3.2.1-2: Ranging/Sidelink Positioning UE Discovery with Model B

1.	The Discoverer UE sends a Solicitation message. The Solicitation message may include Type of Discovery Message, Discoverer Info, Ranging/SL Positioning service identifier, Target Info, Ranging/SL Positioning UE role (e.g. target UE, SL Reference UE).

The Destination Layer-2 ID and Source Layer-2 ID used to send the Solicitation message are specified in TS 23.304 [7].

2.	The Discoveree UE that matches the solicitation message responds to the Discoverer UE with the Response message. The Response message may include Type of Discovery Message, Ranging/SL Positioning service identifier, Discoveree Info

The Source Layer-2 ID used to send the Response message is specified in TS 23.304 [6]. The Destination Layer-2 ID is set to the Source Layer-2 ID of the received Solicitation message.

### 6.4.2	Ranging/ Sidelink Positioning UE Discovery for V2X capable UEs

When unicast mode V2X communication procedure is used, the Layer-2 link establishment procedure as specified in clause 6.3.3.1 of TS 23.287 [6]. In this procedure, Service Type indicates "Ranging/Sidelink Positioning";

When broadcast mode or groupcast mode V2X communication procedure is used as specified in clause 6.3.1 and clause6.3.2 of TS 23.287 [6], Service Type indicates "Ranging/Sidelink Positioning";.

### 6.4.3	Procedures for Located UE Discovery

For 5G ProSe capable Located UEs discovery, both Model A and Model B specified in 6.4.1 are supported. The Located UE may send Announcement message as an Announcing UE in Model A or monitor Solicitation message and send Response message as a Discoveree UE in Model B.

For V2X capable Located UEs discovery, communication procedure as specified in 6.4.2 are used.

Editor's note:	How to include Located UE ID and which Located UE ID is used in discovery message are FFS.

### 6.4.4	Procedures for Ranging/SL Positioning Group Member Discovery

Editor's note:	Procedure for Ranging/SL Positioning Group Member Discovery will be added later.

# * * * * End of changes * * * *