3GPP TSG-SA2 Meeting #156E	S2-2305380

# Elbonia, April 17 – 21, 2023												 (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4157 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.502 – Member UE terminology update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There were discussions on deciding the proper terminology to the UEs which are involved in the AI/ML operation. After some offline discussions among the AIMLsys colleagues (see S2-2305345), a tentative converged view seems to prefer the “Member UE” as the most supported term. The intent of this CR is to update the descriptions of the member selection functionality to use the term “Member UE”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating the descriptions of the member selection functionality to use the term “Member UE”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent reference of UEs which are referred in the member selection functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13, 4.15.13.0, 4.15.13.1, 4.15.13.2, 4.15.13.3, 4.15.13.3.1, 4.15.13.3.2, 4.15.13.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y | X | Other core specifications |  |  |  | TS/TR. |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 4.15.13	5GS assistance for Member UE selection

#### 4.15.13.0	General

AF provides a UE list and at least one Member UE filtering criterion as part of the service operation parameters to assist the candidate UEs selection. Upon receiving the AF request, NEF triggers corresponding 5GC procedures to retrieve the information from 5GC NFs. Before sending the list(s) of candidate UE to the AF, NEF consolidates all the information collected from other 5GC NFs and derives one or more list(s) of candidate UEs and possibly additional information according to the Member UE filtering criteria requested by the AF.

The member selection assistance capability can be used to assist the AF to select the group of UEs to support application service (e.g. FL operation) and it is further defined in clause 5.46.2 of TS 23.501 [2].

AF leverages the 5GC network exposure for UE member selection without the NEF assistance as described in clause 5.46.2 of TS 23.501 [2]. An example of how AF to leverage the 5GC network exposure for UE member selection can be referred to informative Annex I.

#### 4.15.13.1	Member UE selection general information flow

This clause describes the procedures that are generally applicable independently of the Member UE filtering criteria sent by the AF.

Figure 4.15.13.1-1: 5GC assistane to Member UE selection

1.	AF subscribes the member selection assistance functionality by sending Nnef_MemberUESelectionAssistance_subscribe request including a list of target UE(s), one or more UE member filtering criteria listed in the Table 4.15.13.2-1, and optionally, time window(s).

2.	NEF verifies the authorization of the AF Request and identifies which information needs to be collected and executes the corresponding service operation based on the Member UE filtering criteria provided by the AF, e.g. events, analytics and/or notifications.

3.	NEF interacts with different 5GC network functions to collect the required information. The set of interactions between NEF and among 5GC NFs are dependent on the Member UE filtering criteria provided by the AF. See Table 4.15.13.2-1 for details.

4.	Based on the collected information from other 5GC NFs, NEF consolidates all the information collected from other 5GC NFs to derive the list(s) of candidate UE(s) which fulfill the Member UE filtering criteria in the AF request.

5.	NEF sends a Nnef_MemberUESelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and possibly additional information.

#### 4.15.13.2	Member UE Filtering Criteria for 5GS assistance to Member UE selection

Table 4.15.13.2-1 provides a summary of the Member UE filtering criteria that the AF may request.

Table 4.15.13.2-1: Description of Member UE filtering criteria

| Member UE filtering criteria | Description of filtering criterion | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| QoS | The Quality of Service of the selected UEs match or exceed the QoS of the filtering criteria |  | 4.15.13.3 |
| Access Type and/or RAT Type of the PDU Session | Indicate the Access Type and/or RAT Type of the selected UE for the PDU Session used by the application (e.g. 3GPP/NR, Non-3GPP/WLAN, additional Access Type and RAT Type for MA PDU session) | Service operation: Nsmf_EventExposure Filter: a list of GPSI(s) or SUPI(s), DNN/S-NSSAI Event ID: Change of Access Type and/or Change of RAT Type | This can be provided by existing service operations, events, and procedures. |
| End-to-end data volume transfer time | Indicate the end-to-end data volume transfer time that refers to a time for completing the transmission of a specific data volume between UE to AF | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Analytics ID = End-to-end data volume transfer time | Clause 6.18 of TS 23.288 [50]. |
| UE current location | Indicate the certain area that the selected UE currently located in. | Service operation: Namf_EventExposure Filter: a list of GPSI(s) or SUPI(s) | 4.15.13.3 |
| UE historical location | Indicate the certain area that the selected UE appeared in a historical period of time | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period | 4.15.13.3 |
| UE direction | Indicate the selected UEs should include different moving direction | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID= UE Mobility, Direction | 4.15.13.3 |

Editor's note:	The list of Member UE filtering criteria and their corresponding actions are FFS.

Editor's note:	Each Filter Criteria may add a new clause where the specific procedures for such filter criteria are described. For example:"4.15.13.3	Specific procedure for criterial foo".

#### 4.15.13.3	Specific procedure for QoS Member UE filtering criteria

##### 4.15.13.3.1	General

An AF may invoke Nnef_MemberUESelectionAssistance_Subscribe service operation with a QoS filtering criteria for receiving a list of UEs that match or exceed such criteria.

In addition to the mandatory parameters, the AF also includes in the request:

-	QoS filtering criteria.

-	Optionally, an Area of Interest: location area of the candidate UEs.

-	Optional time windows for selecting the candidate UEs: start time and stop time.

If one or more time windows are included in the request, the AF is expressing its intention to obtain the list of UEs that have been (statistics) or will be (predictions) matching or exceeding the QoS filtering criteria and have been or will be optionally located in the AoI, for the duration of the subscription. This is further described in clause 4.15.13.3.2.

If no time window is included in the request, the AF is expressing its intention to obtain the current list of UEs that match or exceed the QoS filtering criteria and are optionally located in the AoI in real time, for the duration of the subscription. This is further described in clause 4.15.13.3.3.

##### 4.15.13.3.2	Specific procedure for the 5GC assistance to Member UE selection based on the UE's current location, historical location and direction.

Figure 4.15.13.3.2-1: 5GC assistane to Member UE selection based on the UE's travel history, direction and separation distance

1.	AF requests 5GS assistance to support the Member UE selection by considering the UE's historical location, UE's current location and direction. AF includes the UE lists and the following criteria as part of the FL UE selection request:

-	UE historical location: The Target AoI where the UEs have been roving over the historical nomadic period before moving into the FL coverage area.

-	UE current location: The current AoI which is the coverage area of the FL training server where the selected UEs located in to participate in the FL operation.

-	Direction: Select the UE with the different direction in the FL coverage Area.

When providing a target area for FL, the AF may provide sub-areas, and provide a maximum number of UEs that should take part in FL from each sub-area.

2.	NEF translates the GPSIs to SUPIs and maps the filtering criteria into the corresponding UE filtering information.

3-4. The NEF invokes Namf_EventExposure_Subscribe service operation with the UE list, the current AoI and event ID = Location report. AMF will provide a list of UEs that are within the current AoI to the NEF using the Namf_EventExposure_Notify service operation. The NEF obtains the list of possible targets UEs from AMF within the current AoI.

5.	In order to identify the appropriate NWDAF which can provide analytic output to derive the visited AOI info and to get the UE direction for the possible target UEs above, the NEF needs to select the suitable NWDAF by initiating the NWDAF discovery request (Analytics ID = UE mobility, AOI = Target AOI) with UE list received from step 4.

6-7. The NEF invokes NWDAF Analytics Info request (UE list received from step 4, Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period"). NWDAF will then provide a list of UEs that were ever roving within the target AOI, at the minimum, over the historical nomadic period NWDAF and also provide a list of UEs location in the order of which the UE passes through to the NEF. Then the NEF gets the corresponding statistics of UE mobility and the UE's direction.

8-9. Based on the information provided by the AMF and NWDAF, the NEF can determine the FL candidate UEs which are now within the FL coverage area but were roving within the target AOI over the historical nomadic period and UE with the different direction as requested by AF. The NEF notifies AF for such UE candidate list.

10.	The NEF also needs to consider the list of UEs which are now within the FL coverage area, but may move out of the FL coverage area. Therefore, for each UE in the candidate list in steps 8-9, the NEF invokes the Namf_EventExposure_Notify service with UE ID = SUPI, Event ID=UE moving in/out of AOI to keep track the movement of the UE(s) in/out of the AOI.

11-12.	If step 10 identifies any UE which is moving out of the FL coverage area, the NEF may further notify AF of the given UE which is moving out the coverage.

##### 4.15.13.3.3	Member UE Assistance with QoS filtering criteria for real-time QoS Monitoring

If no time window is present in the Nnef_MemberUESelectionAsistance_Subscribe request of step 1, NEF determines to do real-time QoS monitoring for determining the list of UEs that match or exceeds the QoS filtering criteria in real time. QoS Monitoring must have been activated in SMF for that QoS Flow before (e.g. by AF Session with required QoS). NsmF_Event_Exposure consumer becomes an additional received of the QoS Monitoring information.

Editor's note:	For QoS Flows other than QoS Flow associated to the default QoS rule, whether and how QoS monitoring is performed is FFS.

Figure 4.15.13.3.3-1: 5GC assistance to Member UE selection for real-time QoS monitoring

1.	AF subscribes the Member UE selection assistance functionality by sending Nnef_MemberUESelectionAssistance_subscribe request including the Application Identity, Flow Description(s) and the QoS filtering criteria that includes one or more of UL packet delay, UL packet delay and round-trip packet delay.

2.	NEF verifies the authorization of the AF Request and identifies which information needs to be collected and executed based on the QoS filtering criteria provided by the AF.

3.	If an Application ID is present in the request from the AF, the NEF derives the S-NSSAI and DNN to which this Application ID have access. Due to the absence of a time window in the request from the AF, NEF discovers querying UDM and NRF the SMFs that are deployed in the Area of Interest. The NEF may also restrict the discovery to those SMFs that serve some S-NSSAI and DNN combination (this S-NSSAI and DNN combination has been derived from the Application Id or AF Id).

4.	For each target SMF, NEF sends an Nsmf_EventExposure_Subscribe request (Event: 'QoS monitoring', target UEs, Application ID or Flow Description(s), S-NSSAI, DNN, etc.).

5.	The SMF many need to interact with UPF for requesting the QoS monitoring for certain flows.

6.	SMF sends an Nsmf_EventExposure_Notify request including an Event Exposure notification.

7.	Based on the Analytics report received from the SMFs, NEF consolidates the results received from the SMFs and derives the list(s) of candidate UE(s) and additional information which fulfil the QoS filtering criteria in the AF request.

8.	NEF sends a Nnef_ MemberUESelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and additional information.

* * * End of Changes * * *