3GPP TSG-SA2 Meeting #156-e	S2-2305388

# Online, , 17th Apr 2023 - 21st Apr 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0818 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving Open issues for Location Accuracy Analytics of key issue 9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  |  |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following issues need to be resolved: 1. Editor´s note: Other target of analytics reporting is FFS. So far only “Any UE” is supported, but the location accuracy is expected to be not dependent on the UE, so the EN can be removed. 2. Editor's note: Additional procedures for location accuracy analytics are FFS. No need for additional procedures was identified in the conclusions of TR 23.700.81 or TR 23.700-71, so the EN can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editors Notes are removed and some editorial improvments. Wrong refrernce to TS 22.271 is replaced by reference to TS 22.071. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Open ENs remain |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.17.1, 6.17.2, 6.17.3, 6.17.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 6.17.1	General

This clause specifies how an NWDAF can provide Location Accuracy Analytics which provides analytics for LCS QoS accuracy to an analytics consumer such as LMF.

The Location Accuracy Analytics provides one or a combination of the following information:

-	location accuracy (i.e. Horizontal Accuracy (see clause 4.3.1 of TS 22.071 [35]) and optinally Vertical Accuracy (see clause 4.3.2 of TS 22.071 [35]);

-	Indication on whether a UE is indoor or outdoor when a location measurement is made (optional);

-	Indication on the ratio of LOS/NLOS measurements;

-	Applicable area for location accuracy prediction (optional);

-	requested Confidence of location accuracy prediction (optional).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "Location Accuracy";

-	Target of Analytics Reporting: "any UE";

-	Location (horizontal and optional vertical) or area of interest;

-	Measurement method;

-	Analytics Filter Information as defined in table 6.17.1-1;

-	optionally, a preferred level of accuracy of the analytics;

-	optionally, preferred level of accuracy per analytics subset (see clause 6.17.3);

-	optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for respective analytics subsets (see clause 6.17.3) in order to be notified by the NWDAF;

-	optionally, maximum number of objects; and

-	an Analytics target period indicates the time period over which the statistics or predictions are requested.

Table 6.17.1-1: Analytics Filter Information related to Location Accuracy Analytics

| Information | Description |
| --- | --- |
| Area of Interest | Identifies the Area (i.e. set of TAIs), as defined in TS 23.501 [2] for which the analytics information is subscribed or requested. |
| List of analytics subsets | List of analytics subsets that are requested among those specified in clause 6.17.3. |
| NOTE: All parameters are optional. |  |

2nd change

### 6.17.2	Input Data

A training should be first performed with correlated positioning estimates obtained by two positioning methods, one of them (e.g. GPS information) used as ground truth to evaluate the accuracy of the other, see Table 6.17.2-1.

Table 6.17.2-1: Data Collected by NWDAF for Location Accuracy Analytics

| Information | Source | Description |
| --- | --- | --- |
| (horizontal and optional vertical) UE location determined by two positioning methods | LMF, OAM | UE location as estimated by the LMF with the selected localization method. |
| Positioning Method | LMF/GMLC | The positioning method used by the LMF when the location measurement was made. |
| UE ID | LMF, OAM | UE identifier. |
| Location methods/Assistance Data | LMF, OAM | Selected location method and corresponding parameters. |
| Area | LMF | The region of interest where the UE is located (e.g. could be list of cell IDs). |
| LOS or NLOS measurement | LMF | Indication whether the UE measurements were based on LOS/NLOS measurements. |
| Indoor/outdoor indication | LMF | Indication of indoor/outdoor when LMF can distinguish indoor/outdoor environment. |

3rd change

### 6.17.3	Output Analytics

The Location Accuracy analytics are shown in table 6.17.3-1.

Table 6.17.3-1: Location Accuracy statistics

| Information | Description |
| --- | --- |
| List of Accuracy sustainability Analytics (1..max) |  |
| > Applicable Area | A list of TAIs or Cell IDs or below cell level location information that the analytics applies to. |
| > Applicable Time Period | The time period within the Analytics target period that the prediction applies to. |
| > Positioning Method | The positioning method used to measure location |
| > analytics accuracy (horizontal and optionaly vertical) | predictions about the horizontal and optionally vertical location accuracy of applicable positioning method at at indicated location or location area. |
| > Percentage of UE in indoor/outdoor in target area (optional) | Provides a predicted ratio of UEs that are indoor/outdoor in the target area |
| > Percentage of NLOS/LOS measurements in target area | Provides predicted ratio of LOS to NLOS measurements of UEs in target area |
| > Location | TA, or cell, or Other location information as defined in TS 22.071 [18] |
| > Applicable Time Period | The time period within the Analytics target period that the prediction applies to. |
| > Positioning Method | The positioning method used to measure location |
| > analytics accuracy (horizontal and optionaly vertical) | predictions about the horizontal and optionally vertical location accuracy of applicable positioning method at at indicated location or location area. |
| > Indication whether location is indoor/outdoor (optional) | If a single location for a UE is provided, indicates whether this location is predicted to be indoor or outdoor |
| > Indication whether the location is measured NLOS/LOS | If a single location for a UE is provided, indicates whether this location is measured with NLOS or LOS |
| >Confidence | Confidence of the prediction. |

4th change

### 6.17.4	Procedures to request Location Accuracy Analytics

Figure 6.17.4-1: Location accuracy analytics retrieval

0.	Pre-condition: NWDAF has a trained supervised ML model for predicting location accuracy. In the training phase, the NWDAF consumes input data of LMF and/or OAM as listed in listed in clause 6.17.2

1.	The Analytics consumer, e.g. LMF, requests from the NWDAF  location accuracy statistics giving as inputs the corresponding Analytics ID and UE location estimate (in the form of horizontal and/or vertical location) or corresponding area (cell ID/TAI). The analytics request to the NWDAF may be a one time or a subscription request. Other inputs such as UE positioning method, applicable period can be assistance data.

NOTE 1:	It is assumed that the analytics consumer determines a location. For instance, the LMF can be triggered by LCS client request to determine a location and then use location accuracy analytics to inquire the accuracy of the determined location.

NOTE 2:	Both Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo services can be used for step 1. above. The Nnwdaf_AnalyticsSubscription service can be used by a service consumer to receive notifications about location accuracy, e.g. when a change is detected.

2.	The NWDAF uses the trained ML model and provides the location accuracy as output to the analytics consumer.

NOTE 3:	The NWDAF can derive the accuracy based on the model without further input data.

End of changes