3GPP TSG-WG SA2 Meeting #156e	S2-2305389

# Elbonia, April 17 – 41, 2023		(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0144 | rev |  | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common EAS/DNAI determination for a set of UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Edge_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - There are several FFS in 6.2.3.2.7 related to Coordination among SMFs and creating/updating/deleting set of UEs data determined by 5GC i.e. Common EAS and common DNAI determination. This CR resolves those FFS - The procedure in 6.2.3.2.7 related to Synchronization of Common EAS/DNAI for SMFs has concerns, for example it requires that SMFs updates to UDR via respective PCFs, however, this may not work for HR-SBO cases where SMF does not interface directly with PCF and to receive AF traffic influence data subscribes directly to NEF using TrafficInfluenceData _Subscribe. - Current procedure in 6.2.3.2.7 does not work well e.g. due to UDR locking the data, unclear which SMF allowed to updating data, etc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates 6.2.3.2.7 to provide details on determination of common EAS and/or DNAI, coordination among SMFs serving the set of UEs and storing information for a set of UEs in UDR together with AF traffic influence data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No clear solution to common EAS/DNAI determination and storing information for a set of UEs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 23.502, CR 4160 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.2.3.2.7	Coordination among SMFs for Common EAS/DNAI determination

Figure 6.2.3.2.7-1: Common EAS/DNAI for set of UEs

1.  AF requests to influence traffic routing to a set of UEs as in clause 4.3.6.2, TS 23.502, with below changes

-  In step 3a, NEF updates the AF influence data in the UDR with the NEF ID to indicate it as responsible of the set of UEs associated with the Traffic correlation ID. NEF also includes a subscription to be notified with start/stop of SMF involvement in a PDU session for a UE member of the set of UEs.

- 	In step 5a, SMF receives PCC rules with AF provided information, and also includes NEF ID and the NEF subsciption

2.  SMF determines the UE belongs to the set of UEs based on Traffic correlation ID received in the PCC rule. If there is already EAS IP and DNAI then this step is skipped, and this EAS IP and DNAI is used as common EAS IP and common DNAI respectively.  However, if there is no EAS IP and/or no DNAI available, then this SMF assumes the role of initial SMF and determines common EAS IP and/or common DNAI.  SMF determines EAS IP using EASDF procedure. Also, SMF determines DNAI using logic for DNAI selection, and additionally collectively considering all the UE members of set of UEs that this SMF serves.

3. SMF sends request to NEF identified by NEF ID as received in the step 1, to update traffic influence data with 5GC determined, in step 2, EAS IP and DNAI to be used as common EAS IP and common DNAI for the set of UEs.

SMF also sends notification related with start of SMF involvement in a PDU session for one or multiple of the UE(s) member of the set of UEs; this notification contains SMF ID and Traffic correlation ID.

4. NEF determines that currently no SMF is registered or associated as the SMF responsible for the set of UEs, thus it assigns this as master SMF i.e. the SMF responsible for the set of UEs and is allowed to update 5GC determined EAS ID or common DNAI.  Accordingly, NEF updates UDR with data as received in step 3. The information is stored/updated to the Traffic Influence data. NEF updates EAS and DNAI information only if it is provided by the SMF assigned as responsible for the set of UEs.

This triggers notifications to PCF(s) that in turn trigger associated PCC rule sending to those SMF(s) involved, if any, with PDU Session associated with the traffic correlation ID.

The NEF updates its list of SMFs serving the set of UEs based on the received notifications related with start/Stop of SMF involvement in a PDU session for a UE member of the set of UEs. The list of SMFs serving the set of UEs also contains the number of PDU sessions served by each of these SMF(s).

5. NEF responds to SMF. If this SMF was assigned as the SMF responsible for the set of UEs, for example if this is the initial or first SMF to serve the set of UEs, then NEF in its response also indicates this to be master SMF i.e. owner of the Common EAS and/or common DNAI information in the UDR, thus enabling this SMF to delete or update these information if required.

In case of master SMF unavailability e.g. due to failure or shut-down, or due to no the master SMF longer serving UEs belonging to the set of UEs, etc. or based on local policies, the NEF may select another master SMF e.g. from among the list of SMFs serving the set of UEs considering e.g. number of UEs being served by each SMFs, or the number of PDU sessions served by each of these SMF(s) in the list of SMFs serving a set of UEs.

# * * * * End of changes * * * *