3GPP TSG-SA WG2 Meeting #156E	S2-2305427

# Elbonia, April 17 – 21, 2023,													   (revision of S2-2304617r00)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0776 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIMLsys: KI#7 – Clarification for validity conditions of E2E data volume transfer time analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are two validity period and spatial validity in the Output analytics of End-to-end data volume transfer time. One is for E2E data volume transfer time for an individual UE, the other is for Classified E2E data volume transfer times for a list of UEs. However, the description for each is the same, which is confusing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the description of the validity conditions for Classified E2E data volume transfer times for a list of UEs. - Editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear what duplicate validy period and spatial validity mean. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

6.18.3	Output Analytics

The NWDAF supporting E2E data volume transfer time analytics provides the analytics results to consumer NFs, e.g. AF, or NEF. The analytics results provided by the NWDAF could be E2E data volume transfer time statistics as defined in Table 6.18.3-1 or predictions as defined in table 6.18.3-2.

Table 6.18.3-1: E2E data volume transfer time statistics

| Information | Description |
| --- | --- |
| UE ID or list of UE IDs (1..SUPImax) | Identifies a UE or a group of UEs, e.g. a list of UEs for which the statistic applies. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > E2E data volume transfer times (1…max) | List of E2E data volume transfer time per UE. Max. is the number of UEs, if applicable. |
| >> Application ID | Identifies the application in use during the time slot |
| >> DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| >> UE location | Indicating the UE location information when the UE service is delivered. |
| >> DNN | DNN for the PDU Session which contains the QoS flow. |
| >> S-NSSAI | Identifies the Network Slice used to access the Application. |
| >> Validity period | The validity period for the E2E data volume transfer time statistics as defined in clause 6.1.3. |
| >> Spatial validity | Area where the E2E data volume transfer time statistics applies. |
| >> E2E data volume transfer time UL | E2E data volume transfer time UL indicators. Statistics of E2E data volume transfer time for uplink over the Analytics target period (e.g. average, variance). |
| >>> Data volume UL | Indicates uplink data volume used to derive E2E data volume transfer time UL. |
| >> E2E data volume transfer time DL | E2E data volume transfer time DL indicators. Statistics of E2E data volume transfer time for downlink over the Analytics target period (e.g. average, variance). |
| >>> Data volume DL | Indicates downlink data volume used to derive E2E data volume transfer time DL. |
| > Classified E2E data volume transfer times for a list of UEs (NOTE 1) | Classified E2E data volume transfer time statistics for multiple UEs with respect to one or more reporting thresholds (e.g. NWDAF may provide the ratio of UEs that have reached certain Reporting Threshold(s)). The list of UEs is indicated in the request of service consumer. |
| >> E2E data volume transfer time classes (1…max) (NOTE 2) | List with group of UEs classified by ranges of E2E data volume transfer time. |
| >>> UE ID(s) | Identifies the UE(s) in the transfer time class with respect to the threshold of the corresponding transfer time class. |
| >>> Ratio of UEs per E2E data volume transfer time class | Ratio of UEs. |
| >> Validity period | The validity period for the Classified E2E data volume transfer times for a list of UEs statistics as defined in clause 6.1.3. |
| >> Spatial Validity | Area where the Classified E2E data volume transfer times for a list of UEs analytics applies. |
| > Geographical distribution of the UE(s) | If requested, a list of UEs per location information. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 2: The number of transfer time classes may be pre-configured by the operator or provided by the service consumer via reporting thresholds. |  |

Table 6.18.3-2: E2E data volume transfer time predictions

| Information | Description |
| --- | --- |
| UE ID or list of UE IDs (1..SUPImax) | Identifies a UE or a group of UEs, e.g. a list of UEs for which the predictions applies. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > E2E data volume transfer times (1…max) | List of E2E data volume transfer time per UE. Max. is the number of UEs, if applicable. |
| >> Application ID | Identifies the application in use during the time slot. |
| >> DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| >> UE location | Indicating the UE location information when the UE service is delivered. |
| >> DNN | DNN for the PDU Session which contains the QoS flow. |
| >> S-NSSAI | Identifies the Network Slice used to access the Application. |
| >> Validity period | The validity period for the E2E data volume transfer time prediction as defined in clause 6.1.3. |
| >> Spatial validity | Area where the E2E data volume transfer time prediction applies. |
| >> E2E data volume transfer time UL | E2E data volume transfer time indicators. Predictions of E2E data volume transfer time for uplink over the Analytics target period (e.g. average, variance). |
| >>> Data volume UL | Indicates uplink data volume used to derive E2E data volume transfer time UL. |
| >> E2E data volume transfer time DL | E2E data volume transfer time DL indicators. Predictions of E2E data volume transfer time for downlink over the Analytics target period (e.g. average, variance). |
| >>> Data volume DL | Indicates downlink data volume used to derive E2E data volume transfer time DL |
| > Classified E2E data volume transfer times for a list of UEs (NOTE 1) | Classified E2E data volume transfer time prediction for multiple UEs with respect to one or more reporting thresholds (e.g. NWDAF may provide the ratio of UEs that have reached certain Reporting Threshold(s)). The list of UEs is indicated in the request of service consumer. |
| >> E2E data volume transfer time classes (1…max) (NOTE 2) | List with group of UEs classified by ranges of E2E data volume transfer time. |
| >>> UE ID(s) | Identifies the UE(s) in the transfer time class with respect to the threshold of the corresponding transfer time class. |
| >>> Ratio of UEs per E2E data volume transfer time class | Ratio of UEs. |
| >> Validity period | The validity period for the Classified E2E data volume transfer times for a list of UEs prediction as defined in clause 6.1.3. |
| >> Spatial Validity | Area where the Classified E2E data volume transfer times for a list of UEs analytics applies. |
| > Geographical distribution of the UE(s) | If requested, a list of UEs per location information. |
| Confidence | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 2: The number of transfer time classes may be pre-configured by the operator or provided by the service consumer via reporting thresholds. |  |

NOTE:	When Target of Analytics Reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics transfer time result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE.

* * * * End of Changes * * * *