3GPP TSG-WG SA2 Meeting #156-e	S2-2305453

# Elbonia, April 17 – 21, 2023		(revision of S2-2304886)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0789 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to remove the EN for DN performance predictions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The DN performance statistics has been extended with the following: - Aggregated Traffic rate; - Variance Traffic rate; - UE ID or list of UE IDs for traffic rate performance; - Variance Packet Delay; - UE ID or list of UE IDs for packet delay performance; - Variance Packet Loss Rate; - UE ID or list of UE IDs for packet loss performance. These statistics outputs are intended to support aggregated UE performance monitoring and exposure for a group of UEs, and no requirements for deriving corresponding predictions for these parameters. Therefore, there is no need to apply all these parameters for prediction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the following EN. Editor’s note: It is FFS if the prediction is applicable to all the performance defined in table 6.14.3-1. A NOTE is added to clarify the intention to add these parameters only in statistics outputs. NOTE X: Analytics subset that is intended to support aggregated UE performance monitoring and exposure for a group of UEs, and only statistics output is needed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EN for DN performance predictions still exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.14.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.14.3	Output Analytics

The DN performance analytics is shown in table 6.14.3-1 and table 6.14.3-2.

Table 6.14.3-1: DN service performance statistics

| Information | Description |
| --- | --- |
| Application ID | Identifies the application for which analytics information is provided. |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. See note 1. |
| DNN | Identifies the data network name (e.g. "internet") for which analytics information is provided. See NOTE 1. |
| DN performance (0-x) | List of DN performances for the application. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address/FQDN of the Application Server). |
| > Serving anchor UPF info | The UPF ID/address/FQDN information for the involved anchor UPF. See NOTE 1. |
| > DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| > Performance (NOTE 4) | Performance indicators. |
| >> Average Traffic rate (NOTE 2) | Average traffic rate observed for UEs communicating with the application. |
| >> Maximum Traffic rate (NOTE 2) | Maximum traffic rate observed for UEs communicating with the application. |
| >> Minimum Traffic rate (NOTE 3) | Minimum traffic rate observed for UEs communicating with the application. |
| >> Aggregated Traffic rate (NOTE 2, NOTE X) | Aggregated traffic rate observed for UEs communicating with the application. |
| >> Variance Traffic rate (NOTE 2, NOTE X) | Variance of the traffic rate observed for the UEs communicating with the application. |
| >> UE ID or list of UE IDs for traffic rate performance (1..SUPImax) (NOTE 2, NOTE X) | Identifies a UE or a list of UEs whose observed traffic rate is higher or lower than the Reporting Threshold. |
| >> Average Packet Delay (NOTE 2) | Average packet delay observed for UEs communicating with the application. |
| >> Maximum Packet Delay (NOTE 2) | Maximum packet delay for observed for UEs communicating with the application. |
| >> Variance Packet Delay (NOTE 2, NOTE X) | Variance of packet delay observed for UEs communicating with the application. |
| >> UE ID or list of UE IDs for packet delay performance (1..SUPImax) (NOTE 2, NOTE X) | Identifies a UE or a list of UEs whose observed packet delay is higher or lower than the Reporting Threshold. |
| >> Average Packet Loss Rate (NOTE 2) | Average packet loss observed for UEs communicating with the application. |
| >> Maximum Packet Loss Rate (NOTE 2) | Maximum packet loss observed for UEs communicating with the application. |
| >> Variance Packet Loss Rate (NOTE 2, NOTE X) | Variance of packet loss rate observed for UEs communicating with the application. |
| >> UE ID or list of UE IDs for packet loss performance (1..SUPImax) (NOTE 2, NOTE X) | Identifies a UE or a list of UEs whose observed packet loss rate is higher than the Reporting Threshold. |
| > Spatial Validity Condition | Area (i.e. list of TAIs) where the DN performance analytics applies. If a Spatial granularity size was provided in the request or subscription, the number of elements of the list is smaller than or equal to the Spatial granularity size. |
| > Temporal Validity Condition | Validity period for the DN performance analytics. If a Temporal granularity size was provided in the request or subscription, the duration of this period is greater than or equal to the Temporal granularity size. |
| NOTE 1: The item "Serving anchor UPF info" shall not be included if the consumer NF is an AF. NOTE 2: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 3: Minimum traffic rate measurements are only derived from active traffic. NOTE 4: Performance statistics may not be applicable to short group operation cycle for the given application. NOTE X: Analytics subset that can be used to support aggregated UE performance monitoring and exposure for a group of UEs. |  |

Table 6.14.3-2: DN service performance predictions

| Information | Description |
| --- | --- |
| Application ID | Identifies the application for which analytics information is provided. |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. See NOTE 1. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. See NOTE 1. |
| DN performance (0-x) | List of DN performance for the application. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address/FQDN of the Application Server). |
| > Serving anchor UPF info | The UPF ID/address/FQDN information for the involved anchor UPF. See NOTE 1. |
| > DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| > Performance | Performance indicators |
| >> Average Traffic rate (NOTE 2) | Average traffic rate predicted for UEs communicating with the application. |
| >> Maximum Traffic rate (NOTE 2) | Maximum traffic rate predicted for UEs communicating with the application. |
| >> Minimum Traffic rate (NOTE 3) | Minimum traffic rate predicted for UEs communicating with the application. |
| >> Average Packet Delay (NOTE 2) | Average packet delay predicted for UEs communicating with the application. |
| >> Maximum Packet Delay (NOTE 2) | Maximum packet delay for predicted for UEs communicating with the application. |
| >> Average Packet Loss Rate (NOTE 2) | Average packet loss predicted for UEs communicating with the application. |
| >> Maximum Packet Loss Rate (NOTE 2) | Maximum packet loss predicted for UEs communicating with the application. |
| > Spatial Validity Condition | Area (i.e. list of TAIs) where the DN performance analytics applies. If a Spatial granularity size was provided in the request or subscription, the number of elements of the list is smaller than or equal to the Spatial granularity size. |
| > Temporal Validity Condition | Validity period for the DN performance analytics. If a Temporal granularity size was provided in the request or subscription, the duration of this period is greater than or equal to the Temporal granularity size. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: The item "Serving anchor UPF info" shall not be included if the consumer is an AF. NOTE 2: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 3: Minimum traffic rate measurements are only derived from active traffic. |  |

# * * * * End of changes * * * *