3GPP TSG-WG SA2 Meeting #156E	S2-2305468

# April 17-21,2023, e-meeting		(revision of S2-2304909r04)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4087 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME |  | Radio Access Network |  |  |  |  |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Clarifications on Network initiated service request procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph3 |  |  |  |  |  |  |  |  |  | Date: |  |  | 2023-04-04 |  |  |  |
| Category: | B |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-18 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |  |  |
| Reason for change: |  | In Network Triggered Service Request procedure, if the UPF decide to transfer the downlink data over the access different from the access type over which the traffic is currently transferd and there is no AN tunnel information of this access type in the UPF, the UPF needs to indicate to the SMF the access type (3GPP access or non-3GPP) over which the user plane resources are to be activated for the MA PDU Session. Without this indication, e.g., the redundant mode cannot be supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that during Network Triggered Service Request procedure, the UPF may indicate the target access type when the UPF receive the downlink traffic data and there is no the AN tunnel information in UPF. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Steering modes (e.g., Redundant Steering mode) cannot be activated in Network Triggered Service Request procedure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.9.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.22.9.4	Network Triggered Service Request

The signalling flow for a Network Triggered Service Request is based on the signalling flow in Figure 4.2.3.3-1 with the following differences and clarifications:

-	In step 3a, the SMF indicates to the AMF the access type (3GPP access or non-3GPP access) over which the user plane resources are to be activated for the MA PDU Session.

Editor’s note:	Whether access type needs to be reported from UPF to SMF for activating access leg(s) is FFS.

In the case of DN-AAA or SMF initiated Secondary re-authentication procedure, when the SMF invokes the Namf_Communication_N1N2MessageTransfer service operation, SMF may indicate the target access type of sending N1 NAS message to the UE.

-	In step 4, the AMF considers the MA PDU Session is associated with the access type the SMF has indicated in step 3a.

The AMF determines the access type of which to send the N1 NAS message to the UE based on the target access type value if received from the SMF in step 3a. If the AMF does not receive a target access type value and the UE is CM-CONNECTED in both accesses, the AMF determines the target access type.

-	In step 5, if the SMF requested to re-activate user-plane resources over 3GPP access and the AMF has determined the UE is unreachable over 3GPP access (e.g. the AMF receives no response from the UE to the Paging), the AMF shall notify that the UE is unreachable. The (H-) SMF shall indicate the Anchor UPF that the user-plane resources on 3GPP access is unavailable by triggering N4 Session Modification procedure. Further action by the UPF is implementation dependent.

If the SMF requested to re-activate the user-plane resources over non-3GPP access and the AMF has determined the UE is unreachable over non-3GPP access (e.g. the UE is in CM-IDLE on non-3GPP access), the AMF shall reject the request from the SMF. The (H-) SMF shall indicate the Anchor UPF that the user-plane resources on non-3GPP access is unavailable by triggering N4 Session Modification procedure. Further action by the UPF is implementation dependent.

NOTE:	The provision of access availability/unavailability reports via user plane specified in clause 5.32.5.3 is UE implementation dependent. Such reporting by UE to UPF, can assist Anchor UPF to decide on handling DL traffic for the UE.

# * * * * End of changes * * * *