3GPP TSG-WG SA2 Meeting #156-e	S2-2305590

# Elbonia, April 17 – 21, 2023		(revision of S2-2305122r01)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0368 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of PRU UE using NR satellite access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2#155 meeting, there is a proposal in S2-2302651 that PRU UE supports location correction of target UE with positioning reference data. It is justified that PRU UE, if registered to network via NR satellite access, can behave as a GNSS reference receiver and generate GNSS assistance data. However, there is no consensus that PRU can support such capability, so that a LS was sent to RAN2 asking: whether or not a UE registered to network by NR satellite access can work as a PRU. According to RAN2 reply in R2-2302270, RAN2 does not intend to make spec changes to support NTN operation as a PRU in Rel-18. RAN2 has also made the below agreement on the LS from SA2: Agreement: RAN2 does not intend to make spec changes to support NTN operation as a PRU in Rel-18. Therefore, similar to RAN2 consideration, it is proposed that SA2 DOES NOT support PRU UE can perform location correction of target UE with positioning reference data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a text “A UE accesses 5G network via NR satellite shall not operate as a PRU” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An open issue “whether or not a UE registered to network by NR satellite access can work as a PRU” does not conclude. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.3.11	Positioning Reference Unit, PRU

A UE may support the functions of a PRU. The PRU supports the following functions including functions defined in TS 38.305 [9]:

-	Support service level association, association update and disassociation with a serving LMF.

-	The PRU sends service level association, association update or disassociation to LMF via LCS supplementary service message.

-	Support association with multiple LMFs. e.g. for the case a PRU is in multiple LMF overlapped serving areas.

-	The PRU information included in a PRU association or PRU association update contains one or more than one of the following aspects:

-	PRU Positioning Capabilities.

-	Location information if known.

Editor's note:	It is FFS whether PRU static/mobile type and ON/OFF state is needed in PRU information.

A UE accesses 5G network via NR satellite shall not operate as a PRU.

# * * * * End of changes * * * *