3GPP TSG-WG SA2 Meeting #156-e	S2-2305608

# Elbonia, April 17 – 21, 2023		(revision of S2-2304845r02)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1006 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for monitoring the data rate per group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2023-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The term Group MBR is misleading as the corresponding enforcement mechanisms is neither direct nor very accurate. As the enforcement mechanism is similar to the monitoring of the data rate per network slice, an equivalent terminology of Maximum Group Data Rate would be more appropriate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The term Group MBR is replaced by the term Maximum Group Data Rate. A note is added which describes when the feature can be used, which is when the PDU Sessions of the group are exclusively used for group traffic and no other traffic is transferred. Furthermore, a new clause is added to properly reflect the new feature in the structure of the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading terminology. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.5, 6.2.1.1.x (new), 6.2.1.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.1.5	Group related policy control

Group related policy control supports limitation of the data rate per group and is only applicable to 5G VN groups, as described in TS 23.501 [2], clause 5.29.2.

The Maximum Group Data Rate defines the maximum allowed aggregate data rate across all GBR and Non-GBR QoS Flows within the group.

A Maximum Group Data Rate can be configured for a group by the operator. The Maximum Group Data Rate has an UL and a DL value.

The PCF monitors the data rate of the group and ensures that it does not exceed the Maximum Group Data Rate for that group. The method of PCF to realize the above functions is same as limitation of the data rate per network slice as defined in clauses 6.1.4.1 and 6.1.4.3 with the following differences:

-	Instead of handling Maximum Slice Data Rate per S-NSSAI, the UDR and PCF handles the Maximum Group Data Rate per group identified by DNN and S-NSSAI.

-	Instead of deducting the value of the authorized Session-AMBR and the MBR of every GBR SDF for every PDU Session of a slice, the PCF deducts such value for every PDU Session accessing to the group.

# * * * * Next change * * * *

##### 6.2.1.1.x	Group related functionality

The PCF provides the following group related functionality:

-	Limitation of the data rate per 5G VN group (as described in clause 6.1.5).

# * * * * Next change * * * *

#### 6.2.1.11	Monitoring the data rate per Group

##### 6.2.1.11.1	General

The PCF supports monitoring and limitation of the data rate per 5G VN group.

If the group is subject to data rate limitation the Maximum Group Data Rate for the group is stored in the UDR as defined in clause 5.29.2 of TS 23.501 [2].

The PCF interacts with the UDR to update the Maximum Group Data Rate specific policy control information per group in the UDR, i.e. the Remaining Maximum Group Data Rate per group, so that the usage of multiple PCFs for the same group is enabled.

##### 6.2.1.11.2	Monitoring the data rate per Group by using QoS parameters

For the purpose of monitoring the data rate per group, the PCF shall perform the same operations as for the monitoring of the data rate per network slice described in clause 6.2.1.10.2 with the following differences:

-	Instead of handling Maximum Slice Data Rate per S-NSSAI, the UDR and PCF handles the Maximum Group Data Rate per group identified by DNN and S-NSSAI.

-	Instead of deducting the value of the authorized Session-AMBR and the MBR of every GBR SDF for every PDU Session of a slice, the PCF deducts such value for every PDU Session accessing to the group.

# * * * * End of changes * * * *