3GPP TSG-WG SA2 Meeting #156e	S2-2305749

# E-Meeting April 17 – 21, 2023		(revision of S2-2304635r05 of S2-2303028)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0322 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ranging and Sidelink Positioning MO-LR procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-04-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is concluded in FS_Ranging_SL study (TR 23.700-86) that the LMF may be used to support network assisted SL Positioning when one of the UEs involved in Ranging and Sidelink Positioning is in coverage. This CR provides the MO-LR based procedure for network assisted Ranging and Sidelink Positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add MO-LR based procedure for network assisted Ranging and Sidelink Positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete network assisted Ranging and Sidelink Positioning support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2; (new) 6.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

APN	Access Point Name

APN-NI	APN Network Identifier

EDT	Early Data Transmission

E-SMLC	Evolved Serving Mobile Location Centre

GMLC	Gateway Mobile Location Centre

HGMLC	Home GMLC

LCS	LoCation Services

LDR	Location Deferred Request

LIR	Location Immediate Request

LMF	Location Management Function

LPI	LCS Privacy Indicator

LRF	Location Retrieval Function

MBSR	Mobile Base Station Relay

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

NI-LR	Network Induced Location Request

PMD	Pseudonym mediation device functionality

POI	Privacy Override Indicator

SDT	Small Data Transmission

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TWAP	Trusted WLAN Access Point

VGMLC	Visited GMLC

SL-MO-LR	Sidelink Mobile Originating Location Request

# * * * * next change * * * *

## 6.x	Network assisted Ranging/SL Positioning procedures

### 6.x.z	SL-MO-LR Procedure

Figure 6.x.z-1 illustrates a procedure to enable a UE, designated UE1 (i.e. Target UE in TS 23.586 [40]), to obtain sidelink positioning/ranging location results for the UE1 using one or more other UEs designated UE2, UE3, … UEn (i.e. Reference/Located UEs in TS 23.586 [40]) with the assistance of an LMF in a serving PLMN for UE1. The Ranging/SL Positioning location results may include absolute locations, relative locations or distances and directions, depending on the service request.

Editor's Note: The solution needs to be updated to cover the following case: when the Target UE (UE1) has NAS connection and can communicate with the LMF and absolute positioning of UE1 is requested and location estimation is performed the LMF. The LMF can either retrieve the location of the Located UEs (UE2-n) from locally stored location, from the Located UE, or by requesting a new location estimation using the service offered by the GMLC. The solution could also include the options where the UE1 location estimation is performed by the UE1 with assistance by the LMF.

Figure 6.x.z-1: SL-MO-LR Procedure

Precondition:

UE1 is in coverage and registered with a serving PLMN. UEs 2 to n may or may not be in coverage and, if in coverage, may or may not be registered with the same serving PLMN as UE1.

1.	The procedures and signalling specified in clause 6.2 of TS 23.586 [40] may be used to provision the Ranging/SL positioning service authorization and policy/parameter provisioning to UE1 and one or more of UEs 2 to n, when in coverage.

2.	Based on a trigger (e.g. received from the application layer), UE1 discovers UEs 2 to n for Ranging/SL positioning. The discovery procedure is specified in clause 6.4 of TS 23.586 [40].

3.	Secure groupcast and/or unicast links are established between UEs 1 to n as defined in clause 5.3 of TS 23.586 [40] to enable UE1 to exchange Ranging and Sidelink Positioning Protocol (RSPP) messages over PC5-U reference point with each of UEs 2 to n and possibly enabling UEs 2 to n to exchange RSPP over PC5-U between each other.

Editor's note:	Security aspects for RSPP signalling need to be defined or agreed by SA3.

4.	UE1 and  UEs 2 to n may communicate over PC5 for authorization of Ranging/SL positioning and receiving QoS parameters if needed. Each of UEs 2 to n verifies that Ranging/SL positioning is permitted, including whether Ranging/SL positioning results may be transferred to an LCS Client or AF if this is used, according to any service authorization and policy/parameter provisioning received at step 1. QoS requirements for the Ranging/SL positioning may be also provided based on QoS requirements in the service request.

5.	UE1 may obtain the sidelink positioning capabilities of UEs 2 to n using the groupcast and/or unicast links established in step 3.

Step 4 and 5 may be performed to transfer the information of UEs which are not served by the LMF.

Editor's note:	For UEs is in coverage, it is FFS whether step 4 and step 5 are needed.

Editor's note:	It needs to be verified that RAN2 will provide support for steps 4 and 5.

6.	Based on the sidelink positioning capabilities of UE1 and received information in step 4 and step 5, UE1 determines SL-MO-LR is to be performed.

7.	If UE1 is in CM-IDLE state, UE1 instigates a UE triggered Service Request in order to establish a signalling connection with the serving AMF of UE1.

8.	UE1 sends a supplementary services SL-MO-LR request to the serving AMF in an UL NAS TRANSPORT message. The SL-MO-LR request indicates the other UEs 2 to n (using application layer ID, e.g. GPSI), indicates any assistance data needed, indicates whether location calculation assistance is needed, and indicates whether location results should be transferred to an LCS client or AF.  For transfer to an LCS Client or AF, details of the LCS Client or AF (and possibly of a GMLC) and application layer IDs for all UEs are also included. For location calculation assistance from the LMF, the preferred type of sidelink positioning/ranging location results (e.g. absolute locations, relative locations or distances and directions between pairs of UEs) and the required QoS are included.

Editor’s note: It is FFS whether indication about the need of assistance data from UE.

9.	The serving AMF selects an LMF serving UE1(e.g. an LMF that supports sidelink positioning/ranging) and sends an Nlmf_Location_DetermineLocation service operation towards the LMF with the information from the SL-MO-LR Request.

10.	The LMF sends a request to UE1 for the capabilities of all UEs 1 to n.

11.	UEs 1 to n returns its capabilities to the LMF. UE1 may return the capabilities for the UEs obtained at step 5 to the LMF.

Editor's note: More details on how the LMF retrieve information and communicate with UE2-n is needed.

12. UE1 may send a request for specific assistance data to the LMF.

13.	LMF sends the requested assistance data to UEs 1 to n. The assistance data may assist UEs 1 to n to obtain sidelink location measurements at step 15 and/or may assist UE1 to calculate sidelink positioning/ranging location results at step 16.

Editor's note: More details on how the LMF retrieve information and communicate with UE2-n is needed.

NOTE 1:	Steps 10 and 11 can be omitted if UE1 includes a message containing the capabilities of UEs 1 to n in the SL-MO-LR request at step 8. Step 12 can be omitted if UE1 includes a message containing the request for specific assistance data in the SL-MO-LR request at step 8,

14.	If the SL-MO-LR request at step 8 indicated location calculation assistance is needed and/or indicated transfer of sidelink positioning/ranging location results to an LCS Client or AF, the LMF sends a request for location information to UE1 and may also send a request for location information to UEs 2 to n.

Editor's note: More details on how the LMF retrieve information and communicate with UE2-n is needed.

15.	UE1 instigates a sidelink positioning/ranging procedure among UEs 1 to n in which UEs 1 to n obtain sidelink location measurements and UEs 2 to n transfer their sidelink location measurements to UE 1 and/or to the LMF (depending on the assistance requested).

Editor's note:	The sidelink positioning/ranging procedure for step 15 needs to be defined by RAN2.

16. If UE1’s absolute location information is required at step 8 and if absolute locations of UE2…UEn are not available, UE2…UEn may perform 5GC-MO-LR procedure to acquire their absolute location.

17.	If UE1 did not determine to request assistance from LMF at step 6 or LMF dermintes to use UE based calculation, UE1 calculates sidelink positioning/ranging location results for itself and for each of UEs 2 to n based on the sidelink location measurements obtained at step 15 and possibly using assistance data received at step 13. The sidelink positioning/ranging location results can include absolute locations, relative locations or ranges and directions related to the UEs 1 to n.

18.	If UE1 received a request for location information at step 14, UE1 sends a response to the LMF and includes either the sidelink location measurements obtained at step 15 or the sidelink positioning/ranging location results obtained at step 17 if step 17 was performed.

Editor's note:	 How to ensure LMF serving UE1 also serves UE2 to UEn is FFS.

19.	If step 18 occurred and if location calculation assistance from the LMF was requested at step 8, the LMF calculates sidelink positioning/ranging location results for UEs 1 to n from the sidelink location measurements received at step 17. The sidelink positioning/ranging location results can include absolute locations, relative locations or ranges and directions related to the UEs 1 to n.

20.	The LMF returns an Nlmf_Location_DetermineLocation service operation response to the AMF and includes any sidelink positioning/ranging location results received at step 18 or calculated at step 19.

21.	If sidelink positioning/ranging location results were received at step 20, the AMF performs steps 7-12 of clause 6.2 to send the sidelink positioning/ranging location results to the GMLC and to an AF or LCS Client if this was requested at step 8. The sidelink positioning/ranging location results include the global identities for UEs 1 to n received at step 8.

NOTE 2:	Sending location results and global identities for UEs 1 to n to an AF or LCS Client may require privacy verification from UEs 1 to n and/or from the HPLMNs of UEs 1 to n.

22.	The LMF returns a supplementary services SL-MO-LR response to UE1 in a DL NAS TRANSPORT message and includes any sidelink positioning/ranging location results calculated at step 19 if step 19 was performed. UE1 may then transfer any sidelink positioning/ranging location results to UEs 2 to n.

# * * * * End of changes * * * *