SA WG2 Meeting #156-e	S2-2305757

# 17 – 21 April 2023, Electronic meeting	(revision of S2-2304967r01)

Title:	Reply LS on NF ID in Nupf_EventExposure_Subscribe Request

Response to:	LS S2-2303942 / C4-230727 on NF ID in Nupf_EventExposure_Subscribe Request from CT4

Release:	3GPP Rel-18

Work Item:	UPEAS

Source:	SA2

To:	CT4

Cc:

Contact Person:

#### Name:	 Yan Han

E-mail Address:	 hanyan@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

SA2 thanks CT4 for their LS response (S2-2303942 / C4-230727) on NF ID in Nupf_EventExposure_Subscribe Request.

The questions in the incoming LS from CT4 in C4-230727 are as following

-	Q.1 In Rel-15 CT4 defined "NF instance ID" with the format of UUID, each 3GPP defined NF shall be allocated with an NF instance ID. Is the "NF ID" parameter which is mandated for Nupf_EventExposure_Subscribe service operation also intended to take the same format as NF instance ID?

-	Q.2 Entities which are not allocated with an "NF ID" (e.g. an external AF which is not defined by 3GPP) will not be able to consume the Nupf_EventExposure, is this the intention of stage2?

-	Q.3 Is there any specific handling of Nupf_EventExposure_Subscribe service operation which requires the "NF ID" to be always present?

SA2 has some discussion the answers are below:

- For Q1, Yes. SA2 has agreed that the "NF ID" parameter also intended to take the same format as NF instance ID. This is like every service request in 5GC where the source NF Instance ID is recommended to be provided (per 3gpp-Sbi-NF-Peer-Info header in TS 29.500 which is recommended to be present and which shall include the source NF instance ID when present).

- For Q2, the consumers of Nupf_EventExposure_Subscribe service are allocated with an "NF ID".

- For Q3, there is no specific handling of Nupf_EventExposure_subscribe service operation which requires the “NF ID” to be always present. The NF ID is mandatory for 5GC NF to request subscription service.

2. Actions:

To CT4:

ACTION: 	SA2 kindly requests CT4 to take the above information into consideration.

3. Date of Next TSG-SA WG2 Meetings:

TSG-SA2 Meeting #157	            22-26 May 2023	Berlin, DE

TSG-SA2 Meeting #158  	 21-25 August 2023            Goteborg , SE