3GPP TSG-WG SA2 Meeting #156E 	S2-2305766

Electronic meeting, 17-21 April 2023

# Title:	Reply LS on LI requirements applicable to non-3GPP access to SNPNs

# Response to:	Reply LS on LI requirements applicable to non-3GPP access to SNPNs (S3i230106)

# Release:	Release 18

# Work Item:	eNPN_Ph2

Source:	SA2

To:	SA3-LI

Cc:	CT4, CT1

Contact Person:

Name:	Haris Zisimopoulos

Tel. Number:

E-mail Address:	harisz AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR 4238 to TS 23.501

1. Overall Description:

SA2 would like to thank SA3-LI for the LS on LI requirements applicable to non-3GPP access to SNPNs.

SA3-LI indicated the following LI requirements for UE accessing to SNPN for onboarding using untrusted non-3GPP access network:

"SA3-LI have concluded that the LI requirements for UE access to SNPN for UE Onboarding are identical to the requirements for UE access to SNPN for service, a.k.a. “general access”.

SA3-LI expect the procedure described in TS 23.501, clause 6.3.6.2a, for “general access” would apply to the UE Onboarding case, as well."

The issue with the existing procedure in TS 23.501, clause 6.3.6.2a is that in case the UE is in visited country it always uses the UEs "subscribed SNPN". See related extract:

If the UE determines that it is located in a country different from the country where the configured N3IWF is located (called the visited country), then:

- The UE shall construct an FQDN consisting of the SNPN ID of the SNPN subscription and the Visited Country FQDN and indicating the query is for SNPN, as specified in TS 23.003 [19] and perform a DNS query for the resulting FQDN.

[..]

The UE though in case of onboarding cannot be expected to know the ON-SNPN when it would need to construct the FQDN since the UE is only configured with default UE credentials that are not pointing to any SNPN ID. SA2 believes that the most appropriate way forward in this case is to define a Visited Country Onboarding N3IWF FQDN. A UE that wants to select an N3IWF for UE onboarding would perform a DNS query for this FQDN and determine based on the returned DNS records, which SNPNs in the visited country support Untrusted non-3GPP access for UE Onboarding via N3IWF.

SA2 approved the attached CR inline with what is explained above.

2. Actions:

To SA3-LI:

ACTION: 	SA2 kindly ask SA3-LI to take the above information into account and provide any feedback if necessary.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #157	22-26 May 2023		Berlin, Germany

TSG-SA2 Meeting #158	21-25 August 2023		Gothenburg, Sweden