3GPP TSG-SA WG2 Meeting #156E	S2-2305776

# April 17 – 21, 2023, E-Meeting												 				(was S2-2304374)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4288 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configuration of Credentials Holder for determining SNPN selection information (Annex N.x) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following conclusion of KI#4 was in TR 23.700-08 for “from where and how UE obtains the information for accessing localized service” serves as the baseline to introduce this concepts a new clause in TS 23.501: The information for localized service can be obtained by UE at the application layer from the home network or the localized service provider via means that are outside of 3GPP scope. The hosting network selection information is provisioned as below: 1. The information for hosting network, selection and access can be preconfigured in the UE or dynamically provisioned by home network (via the VPLMN when roaming) using existing mechanisms. 2. In the case of SNPN as hosting network, the dynamic provisioning of prioritized list of hosting network information can be done via SoR: NOTE: How SOR-AF and/or UDM acquires hosting network information is outside the scope for 3GPP. i. The home network UDM may determine to update UE with prioritized list of hosting network information using SoR procedure. Following triggers may apply: - UE location as part of Registration procedure. - UE subscription data change, e.g. via external parameter provisioning. 3. In the case of PNI-NPN as hosting network, the dynamic provisioning of allowed CAG ID list reuses existing procedure in clause 5.30.3.3 of TS 23.501 [3]. SNPN providing access to localized services (or localized service operator) and HPLMN or subscribed SNPN can exchange information based on service agreements between the SNPN and the HPLMN or subscribed SNPN to enable HPLMN or subscribed SNPN acting as Credentials Holder to generate and provision UEs with SNPN selection information. Existing OAM based mechanisms can be used by HPLMN or subscribed SNPN operator to configure the operator backend systems with this information that can be used by the HPLMN or subscribed SNPN acting as Credentials Holder to determine SNPN selection information that can be provisioned to UEs using Steering of Roaming procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New sub-clause is added in Informative Annex N, clarifying that OAM based mechanism is used to exchange information between the SNPN providing localised services and the HPLMN or the subscribed SNPN, to enable the HPLMN or the subscribed SNPN to provision (SoR update) the UE with appropriate information for SNPN selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex N.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

Annex N (informative):
Support for access to localized services

# N.x	Configuration of Credentials Holder for determining SNPN selection information

To enable the HPLMN or the subscribed SNPN acting as Credentials Holder to generate and provision UEs with SNPN selection information for discovery and selection of SNPNs providing localised services, based on service agreement between the Localised Service Provider or the SNPN providing localized services and the HPLMN or the subscribed SNPN acting as Credentials Holder, the Localised Service Provider or the SNPN providing access to localized services can provide configuration information for SNPN selection to the HPLMN or the subscribed SNPN acting as Credentials Holder. The configuration information for SNPN selection may contain at least one of the following parameters:

a.	Identifier of the SNPN providing access to one or more localized services;

b.	Identification of each localized service;

c.	validity information for each localized service, e.g. the validity time information or/and location assistance information; and/or

d.	List of UE IDs (e.g. GPSIs or External Group ID) identifying the UEs subscribed with a localized service.

Editor's Note:	The exchange between the Localised Service Provider or the SNPN providing access to localized service and the HPLMN or the subscribed SNPN acting as Credentials Holder is FFS.

The operator of the HPLMN or the subscribed SNPN acting as Credentials Holder then may use the information received from the SNPN providing localised services and/or Localized Service Provider to create or update the Credentials Holder controlled prioritized lists of preferred SNPNs/GINs for accessing Localized Services and provision the UEs using the Steering of Roaming procedure as defined in TS 23.122 [17].

* * * * End of Changes * * * *