3GPP TSG-WG SA2 Meeting #156-e	S2-2305918

# Elbonia, April 17 – 21, 2023		(revision of S2-2304416r00)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0253 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of policy for U2U and Direct Discovery and Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to assist the UEs which wish to communicate with each other over PC5 there is direct discovery/communication and U2U relay discovery/communication. The discovery and communication’s each have their own policy, which includes the radio parameters that are used. When performing discovery, it is possible to discover both direct and relayed UEs. It is expected that this discovery can be performed so that both relayed and direct UEs are found. In order to enable this, it is better if the radio resources are aligned to save repeated discoveries in different radio resources. The radio parameters for communication can also be aligned. However, as not all scenarios may want to use the same radio resources for U2U and direct communication, it should be noted that this can and should be done. The principles for applying policy/parameters to 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay discovery, 5G ProSe UE-to-Network Relay communication, 5G ProSe UE-to-UE Relay discovery and 5G ProSe UE-to-UE Relay communication are the same and can refactored into a common clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a note to Direct Communication discovery parameters and U2U discovery parameters, and Direct Communication parameters and U2U relayed communication parameters that they should be aligned for the radio resources they use. Refactor into a common clause the principles for applying policy/parameters and reference it from the relevant clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Sub-optimal provisioning of radio resources for discovery for UEs which communicate with each other, adding latency and power consumption. More complex implementation as the principles for applying policy/parameters is applied individually to each operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.1a (new), 5.1.2.1, 5.1.2.2, 5.1.3.1, 5.1.3.2, 5.1.4.2.1, 5.1.4.2.2, 5.1.5.1, 5.1.5.2.1, 5.1.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.1.1	General

In 5GS, the parameters for 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay and 5G ProSe UE-to-UE Relay services may be made available to the UE in following ways:

-	provisioned in the ME; or

-	configured in the UICC; or

-	provisioned in the ME and configured in the UICC; or

-	provided or updated by the ProSe Application Server via PCF and/or PC1 reference point; or

-	provided or updated by the PCF to the UE.

If the same parameters described in clauses 5.1.2.1, 5.1.3.1, 5.1.4.1 and 5.1.5.1 are provided by different sources, the UE shall consider them in the following priority order:

-	provided or updated by the PCF (including parameters determined by the PCF itself and parameters provided by the ProSe Application Server to the PCF as specified in clause 6.2.5);

-	provided or updated by the ProSe Application Server via PC1 reference point;

-	configured in the UICC;

-	provisioned in the ME.

The parameters provided or updated by the ProSe Application Server via PC1 reference point may need to be complemented with configuration data from other sources listed above.

NOTE:	The ProSe Application Server can provision the same ProSe parameters via 5GC as specified in clause 6.2.5 or directly to the UE via PC1 reference point, and can revoke (e.g. delete) the ProSe parameters via 5GC as specified in clause 6.2.5 in order for the provisioning via PC1 reference point to take effect.

The basic principles of service authorization and provisioning for 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay and 5G ProSe UE-to-UE Relay services are as follows:

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorized to use 5G ProSe Direct Discovery.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to use 5G ProSe Direct Communication.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe UE-to-Network Relay. Authorisation for 5G ProSe Layer-2 UE-to-Network Relay and 5G ProSe Layer-3 UE-to-Network Relay are independent of each other.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to access 5GC via 5G ProSe UE-to-Network Relay (i.e. to act as 5G ProSe Remote UE). Authorisation to access via 5G ProSe Layer-2 UE-to-Network Relay and via 5G ProSe Layer-3 UE-to-Network Relay are independent of each other.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe UE-to-UE Relay. Authorisation for 5G ProSe Layer-2 UE-to-UE Relay and 5G ProSe Layer-3 UE-to-UE Relay are independent of each other.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to access 5G ProSe UE-to-UE Relay (i.e. to act as 5G ProSe End UE). Authorisation to access via 5G ProSe Layer-2 UE-to-UE Relay and via 5G ProSe Layer-3 UE-to-UE Relay are independent of each other.

-	The PCF in the HPLMN merges authorization information from home and other PLMNs and provides the UE with the final authorization information.

-	The PCF in the VPLMN or HPLMN may revoke the authorization (via H-PCF when roaming) at any time by using the UE Configuration Update procedure for transparent UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [5].

-	The ProSe Policy/parameters provisioning to UE is controlled by the PCF and may be triggered by UE. The PCF provisions one or more of the following ProSe Policy/parameters:

-	ProSe Policy/parameters for 5G ProSe Direct Discovery as specified in clause 5.1.2.1;

-	ProSe Policy/parameters for 5G ProSe Direct Communications as specified in clause 5.1.3.1;

-	ProSe Policy/parameters for 5G ProSe Layer-2 and/or Layer-3 UE-to-Network Relay as specified in clause 5.1.4.1;

-	ProSe Policy/parameters for 5G ProSe Layer-2 and/or Layer-3 Remote UE as specified in clause 5.1.4.1.

-	ProSe Policy/parameters for 5G ProSe Layer-2 and/or Layer-3 UE-to-UE Relay as specified in clause 5.1.5.1.

-	ProSe Policy/parameters for 5G ProSe Layer-2 and/or Layer-3 End UE as specified in clause 5.1.5.1.

-	The PCF includes the 5G ProSe Policy/parameters in a Policy Section identified by a Policy Section Identifier (PSI) as specified in clause 6.1.2.2.2 of TS 23.503 [9].

In addition to the above, ProSe usage reporting configuration and rules for charging can be (pre)configured in the UE or provided by the PCF.

In addition to the above, the path selection policy can be (pre)configured in the UE or provided by the PCF as defined in clause 5.11. A path preference for ProSe Services can be provided by ProSe Application Server to UDR, and may be used by PCF for path selection policy generation and update.

When a 5G ProSe Layer-3 Remote UE is using a 5G ProSe Layer-3 UE-to-Network Relay without involving N3IWF, the PCF based provisioning and update of 5G ProSe Policy/parameters to the 5G ProSe Layer-3 Remote UE are not supported.

# * * * * Second change (all new) * * * *

### 5.1.1a	General principles for applying policy/parameters

For services (i.e. 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay discovery, 5G ProSe UE-to-Network Relay communication, 5G ProSe UE-to-UE Relay discovery, and 5G ProSe UE-to-UE Relay communication) over PC5 reference point, the operator may pre-configure the UEs with the required provisioning parameters for the service, without the need for the UEs to connect to the 5GC to get this initial configuration. The following apply:

-	The provisioning parameters for the service could be from different sources and their priorities are described in clause 5.1.1.

-	The ME provisioning parameters shall not be erased when a USIM is deselected or replaced.

-	The UE shall use radio resources for the service as follows:

-	While a UE has a serving cell and is camped on a cell and the UE intends to use for 5G ProSe the radio resources (i.e. carrier frequency) operated by this cell, then the UE shall use the radio resource description indicated by this cell the UE is camped on and ignore any radio resource description of the same radio resource provisioned in the ME or the UICC. If the cell does not provide radio resources for ProSe, the UE shall not perform ProSe message transmission and reception on radio resources operated by this cell. The UE is allowed to perform the service with another UE not served by the same PLMN;

-	If the UE intends to use "operator-managed" radio resources (i.e. carrier frequency) for 5G ProSe that are not operated by the UE's serving cell, as specified in the related Policy/Parameter provisioning, or if the UE is out of coverage, the UE shall search for a cell in any PLMN that is operating the provisioned radio resources (i.e. carrier frequency) as defined in TS 38.300 [12] and TS 38.304 [13]; and:

-	If the UE finds such a cell in the registered PLMN or a PLMN equivalent to the registered PLMN, and authorization for the service to this PLMN is confirmed, the UE shall use the radio resource description indicated by that cell. If that cell does not provide radio resources for ProSe, the UE shall not perform ProSe message transmission and reception on those radio resources;

-	If the UE finds such a cell but not in the registered PLMN or a PLMN equivalent to the registered PLMN, and that cell belongs to a PLMN authorized for the service and provides radio resources for ProSe then the UE shall perform PLMN selection triggered by the service as defined in TS 23.122 [14];

-	If the UE finds such cell but not in a PLMN authorized for the service the UE shall not use the service;

-	If the UE does not find any such cell in any PLMN, then the UE shall consider itself "not served by NG-RAN" and use radio resources provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize the service, then the UE is not authorized to transmit;

-	The UE is allowed to use "operator-managed" radio resources (i.e. carrier frequency) provisioned in the ME or the UICC for the service if the UICC indicates it is authorized;

-	If the UE intends to use "non-operator-managed" radio resources (i.e. carrier frequency) for 5G ProSe, according to TS 36.331 [15] or TS 38.331 [16] and as specified in clause 5.1.2.1, then the UE shall perform the service using resource provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize the service, then the UE is not authorized to transmit;

NOTE 1:	It is possible for operators to configure UEs (e.g. Public Safety UEs) to use only "operator-managed" radio resources (i.e. carrier frequency) for the service when the UE is "not served by NG-RAN".

-	The UE provisioning shall support setting Geographical Areas;

NOTE 2:	It is possible for a UE to use other radio resources for 5G ProSe based on the Geographical Area instead of those operated by the serving NG-RAN cell, when provisioned in the UE, even if the UE's serving cell offers normal service and the SIBs for NR sidelink communication defined in TS 38.331 [16] indicates that the service (the service) is available. This is to cover the scenario when e.g. the radio resources used for the service are not owned by the serving network of the UE.

NOTE 3:	When cross-carrier operation is supported, according to TS 36.331 [15] or TS 38.331 [16], a UE can be instructed by its serving cell to perform the service over a different carrier frequency. The UE is still considered as "served by NG-RAN" in this case.

NOTE 4:	The scenario that a cell is detected and the cell does not provide support for the service when the UE attempts to use a carrier frequency configured for the service, is considered a configuration error. Therefore, the UE does not transmit on that frequency to avoid interference to the network.

-	The service is only specified for NR.

NOTE 5:	It is out of scope of the present specification to define how the UE can locate itself in a specific Geographical Area. When the UE is in coverage of a 3GPP RAT, it can for example, use information derived from the serving PLMN. When the UE is not in coverage of a 3GPP RAT, it can use other techniques, e.g. Global Navigation Satellite System (GNSS). User provided location is not a valid input.

# * * * * Third change * * * *

### 5.1.2	Authorization and Provisioning for 5G ProSe Direct Discovery

#### 5.1.2.1	Policy/Parameter provisioning for 5G ProSe Direct Discovery

The following sets of information for 5G ProSe Direct Discovery over PC5 reference point is provisioned to the UE:

1)	Authorization policy for 5G ProSe Direct Discovery:

-	When the UE is "served by NG-RAN":

-	For open 5G ProSe Direct Discovery:

a)	open 5G ProSe Direct Discovery Model A monitoring authorization policy:

-	PLMNs in which the UE is authorised to perform 5G ProSe Direct Discovery monitoring.

b)	open 5G ProSe Direct Discovery Model A announcing authorization policy:

-	PLMNs in which the UE is authorized to perform announcing.

-	For restricted 5G ProSe Direct Discovery:

a)	restricted 5G ProSe Direct Discovery Model A monitoring authorization policy:

-	PLMNs in which the UE is authorised to perform restricted 5G ProSe Direct Discovery Model A monitoring.

b)	restricted 5G ProSe Direct Discovery Model A announcing authorization policy:

-	PLMNs in which the UE is authorized to perform restricted 5G ProSe Direct Discovery Model A announcing.

c)	restricted 5G ProSe Direct Discovery Model B Discoverer operation authorization policy:

-	PLMNs in which the UE is authorized to perform Model B Discoverer operation.

d)	restricted 5G ProSe Direct Discovery Model B Discoveree operation authorization policy:

-	PLMNs in which the UE is authorized to perform Model B Discoveree operation.

NOTE 1:	In this specification, [When the UE is "served by NG-RAN"] and [When the UE is "not served by NG-RAN"] have similar meaning as in "served by E-UTRAN" and "not served by E-UTRAN" in TS 23.303 [3], but applied to 5G ProSe Direct discovery/communications over NR PC5 reference point.

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorized to perform 5G ProSe Direct Discovery for Model A and Model B when "not served by NG-RAN".

NOTE 2:	If both Model A and Model B are authorized for 5G ProSe Direct Discovery, it is up to UE and application implementation to select a discovery model or perform both models simultaneously.

2)	Parameters used for 5G ProSe Direct Discovery:

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for sending/receiving initial signalling of discovery messages.

NOTE 3:	The same Destination Layer-2 ID for 5G ProSe Direct Discovery can be mapped to more than one ProSe services.

NOTE 4:	The values provisioned for the Destination Layer-2 ID(s) for 5G ProSe Direct Discovery, for Destination Layer-2 ID(s) for 5G ProSe Direct Communication, defined in clause 5.1.3.1 and for Destination Layer-2 ID(s) for 5G ProSe UE-to-Network Relay Discovery defined in clause 5.1.4.1, are different from each other.

-	Application identifiers to be used for 5G ProSe Direct Discovery over PC5 interface.

NOTE 5:	The Security parameters for 5G ProSe Direct Discovery can be provisioned by 5G DDNMF as defined in TS 33.503 [29].

3)	Radio parameters when the UE is "not served by NG-RAN":

-	Includes the radio parameters per NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 6:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

-	Default PC5 DRX configuration (see TS 38.331 [16]).

NOTE x:	Radio parameters for 5G ProSe UE-to-UE Relay Discovery when the UE is not "served by NG-RAN" in clause 5.1.5.2.1 and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe Direct Discovery are expected to be aligned for direct and relayed discovery for UE to UE communication.

4)	Restricted ProSe Discovery UE ID for Restricted Direct Discovery:

-	ProSe Discovery UE ID.

5)	Group Member Discovery parameters:

-	For each discovery group that the UE belongs to include the following parameters that enable the UE to perform Group Member Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	Application Layer Group ID: Identifies an application layer group or a discovery group that the UE belongs to;

-	Layer-2 Group ID: layer-2 ID for Application Layer Group ID;

-	User Info ID: For Model A, this corresponds to the Announcer Info parameter when the UE is acting as an announcing UE. For Model B, this corresponds to the Discoverer Info in Solicitation messages and the Discoveree Info in Response messages, when the UE is acting as a discoverer or discoveree UE respectively.

NOTE 7:	User Info ID is expected to be assigned uniquely to a user within the discovery group.

6)	Optionally, the 5G DDNMF address (either FQDN or IP address) in the Home PLMN.

7)	Validity timer indicating the expiration time of the Policy/Parameter for 5G ProSe Direct Discovery.

The above parameter sets bullet 2), bullet 3), bullet 5) and bullet 7) may be provided or updated to the UE by the ProSe Application Server, except for the Security parameters in bullet 2).

# * * * * Forth change * * * *

#### 5.1.2.2	Principles for applying parameters for 5G ProSe Direct Discovery

The general principles for applying policy/parameters defined in clause 5.1.1a apply to the 5G ProSe Direct Discovery service.

# * * * * Fifth change * * * *

#### 5.1.3.1	Policy/Parameter provisioning for 5G ProSe Direct Communication

The following sets of information for 5G ProSe Direct Communications over PC5 reference point is provisioned to the UE:

1)	Authorization policy:

-	When the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to perform 5G ProSe Direct Communications over PC5 reference point when "served by NG-RAN".

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorized to perform 5G ProSe Direct Communications over PC5 reference point when "not served by NG-RAN".

NOTE 1:	In this specification, [When the UE is "served by NG-RAN"] and [When the UE is "not served by NG-RAN"] are relevant to 5G ProSe Direct Communications over NR PC5 reference point.

2)	Groupcast mode 5G ProSe Direct Communication policy/parameters:

-	For each application layer group supported include the parameters that enable the UE to perform Groupcast mode 5G ProSe Direct Communication when provided by PCF or provisioned in the ME or configured in the UICC:

-	Application Layer Group ID: Identifies an application layer group that the UE belongs to.

-	ProSe Layer-2 Group ID: Destination Layer-2 ID applicable only when the Application Layer Group ID is provided by the application layer

-	ProSe Group IP multicast address

-	Indication whether the UE should use IPv4 or IPv6 for that group

-	For a specific Group configured to operate using IPv4, optionally an IPv4 address to be used by the UE as a source address. If none is provisioned, then the UE shall use Dynamic Configuration of IPv4 Link-Local Addresses RFC 3927 [18] to obtain a link local address for the Group.

3)	Radio parameters when the UE is "not served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform ProSe Direct Communications over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 2:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

NOTE x:	Radio parameters for 5G ProSe Direct Communication when the UE is not "served by NG-RAN" in clause 5.1.5.2.1 and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe UE-to-UE Relay communication are expected to be aligned for direct and relayed UE to UE communication.

4)	Policy/parameters related to privacy:

-	The list of ProSe services (i.e. ProSe identifiers) with Geographical Area(s) that require privacy support.

-	A privacy timer value indicating the duration after which the UE shall change each source Layer-2 ID self-assigned by the UE when privacy is required.

5)	Policy/parameters when NR PC5 is selected:

-	The mapping of ProSe services (i.e. ProSe identifiers) to radio frequencies with Geographical Area(s).

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for broadcast.

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for groupcast.

-	The mapping of ProSe services (i.e. ProSe identifiers) to default Destination Layer-2 ID(s) for initial signalling to establish unicast connection.

NOTE 3:	The same default Destination Layer-2 ID for unicast initial signalling can be mapped to more than one ProSe service. In the case where different ProSe services are mapped to distinct default Destination Layer-2 IDs, when the UE intends to establish a single unicast link that can be used for more than one ProSe service, the UE can select any of the default Destination Layer-2 IDs to use for the initial signalling.

NOTE 4:	Security policies for Unicast mode 5G ProSe Direct Communication can be provisioned by PCF as defined in TS 33.503 [29].

-	The mapping of ProSe services (i.e. ProSe identifiers) to PC5 QoS parameters defined in clause 5.6.1 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.).

-	The mapping of ProSe services (i.e. ProSe identifiers) to the corresponding NR Tx Profiles for broadcast and groupcast (see TS 38.300 [12] and TS 38.331 [16] for further information).

NOTE 5:	A "catch all" entry with the lowest priority in each above mapping can be used for the ProSe services that do not have explicit policy/parameters mapping above.

-	AS layer configurations (see TS 38.331 [16]), i.e. the mapping of PC5 QoS profile(s) to radio bearer(s), when the UE is "not served by NG-RAN".

-	The PC5 QoS profile contains PC5 QoS parameters described in clause 5.4.2 of TS 23.287 [2], and value for the QoS characteristics regarding Priority Level, Averaging Window, Maximum Data Burst Volume if default value is not used as defined in Table 5.4.4-1 of TS 23.287 [2] and in Table 5.6.1-1.

-	For broadcast, groupcast and initial signalling to establish unicast connection, PC5 DRX configuration (see TS 38.331 [16]), e.g. the mapping of PC5 QoS profile(s) to PC5 DRX cycle(s), default PC5 DRX configuration, when the UE is "not served by NG-RAN".

6)	Path selection policy:

-	The mapping of ProSe services (i.e. ProSe identifiers) to path preference (i.e. PC5 preferred, Uu preferred, or no preference) as defined in clause 5.11.

NOTE 6:	The path selection policy can be a one mapping for all ProSe services, i.e. same path preference for all ProSe services.

Editor's note:	It is FFS whether policy for path switching needs to be defined.

7)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Direct Communication.

The above parameter sets from bullet 2) to 7) may be provided or updated to the UE by the ProSe Application Server.

# * * * * Sixth change * * * *

#### 5.1.3.2	Principles for applying parameters for 5G ProSe Direct Communication

The general principles for applying policy/parameters defined in clause 5.1.1a apply to the 5G ProSe Direct Communication service.

# * * * * Seventh change * * * *

#### 5.1.4.2	Principles for applying parameters for 5G ProSe UE-to-Network Relay

##### 5.1.4.2.1	Principles for applying parameters for ProSe UE-to-Network Relay discovery

The general principles for applying policy/parameters defined in clause 5.1.1a apply to the 5G ProSe UE-to-Network Relay discovery service.

# * * * * Eighth change * * * *

##### 5.1.4.2.2	Principles for applying parameters for 5G ProSe UE-to-Network Relay communication

The general principles for applying policy/parameters defined in clause 5.1.1a apply to the 5G ProSe UE-to-Network Relay communication service.

# * * * * Nineth change * * * *

#### 5.1.5.1	Policy/Parameter provisioning for 5G ProSe UE-to-UE Relay

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe UE-to-UE Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 and/or Layer-2 UE-to-UE Relay when the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for 5G ProSe Layer-3 and/or Layer-2 End UEs accessing UE-to-UE Relays over PC5 reference point.

-	The authorisation for a UE to act as a 5G ProSe UE-to-UE Relay also authorizes the use of 5G ProSe UE-to-UE Relay Discovery with Model A and Model B.

NOTE 1:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

Editor's note:	The policy/parameter provisioning corresponding to 5G ProSe UE-to-UE Relay Communication with integrated discovery is FFS.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe UE-to-UE Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe UE-to-UE Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe UE-to-UE Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-UE Relay Layer indicator). The UE-to-UE Relay Layer indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-UE Relay service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement and message and receiving Relay Discovery Solicitation messages;

-	Includes security related content for Relay Discovery for each Relay Service Code.

NOTE 2:	SA WG3 will determine the security related content for 5G ProSe UE-to-UE Relay.

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe End UE and thereby enabling the use of a 5G ProSe UE-to-UE Relay:

1)	Authorisation policy for using a 5G ProSe Layer-3 and/or Layer-2 UE-to-UE Relay:

-	whether the UE is authorised to use a 5G ProSe Layer-3 UE-to-UE Relay.

The authorisation for a UE to act as a 5G ProSe End UE also authorizes the use of 5G ProSe UE-to-UE Relay discovery with Model A and Model B.

NOTE 3:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

2)	Policy/parameters for 5G ProSe UE-to-UE Relay Discovery:

-	Includes the parameters for 5G ProSe UE-to-UE Relay Discovery as a 5G ProSe End UE and for enabling the UE to connect to the 5G ProSe UE-to-UE Relay after discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe UE-to-UE Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-UE Relay Layer indicator). The UE-to-UE Relay Layer indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-UE Relay service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Solicitation messages and receiving Relay Discovery Announcement message;

-	Includes security related content for Relay Discovery for each Relay Service Code.

NOTE 4:	SA WG3 will determine the security related content for 5G ProSe UE-to-UE Relay.

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe UE-to-UE Relay as well as in the UE in support of the UE assuming the role of a 5G ProSe End UE and thereby enabling the use of a 5G ProSe UE-to-UE Relay:

1)	Radio parameters for 5G ProSe UE-to-UE Relay Discovery when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

-	Default PC5 DRX configuration (see TS 38.331 [16]).

NOTE 5:	RAN WG2 will determine the default PC5 DRX configuration for 5G ProSe UE-to-UE relay scenario.

NOTE x:	Radio parameters for 5G ProSe UE-to-UE Relay Discovery when the UE is not "served by NG-RAN" and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe Direct Discovery in clause 5.1.2.1 are expected to be aligned for direct and relayed discovery for UE to UE communication.

2)	Radio parameters for 5G ProSe UE-to-UE Relay communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE x:	Radio parameters for 5G ProSe UE-to-UE Relay communication when the UE is not "served by NG-RAN" and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe Direct Communication in clause 5.1.3.1 are expected to be aligned for direct and relayed UE to UE communication.

# * * * * Tenth change * * * *

#### 5.1.5.2	Principles for applying parameters for 5G ProSe UE-to-UE Relay

##### 5.1.5.2.1	Principles for applying parameters for ProSe UE-to-UE Relay discovery

The general principles for applying policy/parameters defined in clause 5.1.1a apply to the 5G ProSe UE-to-UE Relay discovery service.

# * * * * Eleventh change * * * *

##### 5.1.5.2.2	Principles for applying parameters for 5G ProSe UE-to-UE Relay communication

The general principles for applying policy/parameters defined in clause 5.1.1a apply to the 5G ProSe UE-to-UE Relay communication service.

# * * * * End of changes * * * *